3GPP TSG-RAN WG2 Meeting #105bis	R2-1904274

Xi’an, China, 8th – 12th of April 2019

Title:	[DRAFT] Response LS on QoS characteristics for NR V2X over Uu

Response to:	S2-1813386

Release:	Release 16

Work Item:	NR_V2X-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG SA WG2

Cc:	TSG RAN WG1, TSG RAN WG3

Contact Person:

#### Name:	Jedrzej Stanczak

E-mail Address:	jedrzej [dot] stanczak [at] nokia [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During RAN2#105 and RAN2#105bis meetings, QoS requirements for NR V2X over Uu have been considered, with a particular focus on the following two scenarios highlighted by SA WG2 in S2-1813386:

- Collision Avoidance/Platooning requiring the following QoS characteristics over Uu: PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes

- Emergency Trajectory Alignment/Sensor Information Sharing requiring the following QoS characteristics over Uu: PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes

RAN2 has studied the latency requirements and captured related conclusions in section 11.1 of TR 38.885. With a proper combination of subcarrier spacing and configured grant the end-to-end latency below 2 ms could be achievable over NR Uu. However, the study has not been conducted with a large size of packet in mind (i.e. MDBV of approximately 1300 bytes).  Thus, especially the use case b), where a combination of very low latency, high reliability and large amount of user data, needs to be supported, appears to be infeasible with the existing NR Uu solutions.

2. Actions:

To SA WG2:

ACTION: 	RAN WG2 respectfully asks SA WG2 to take into account the aforementioned RAN WG2 conclusions.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#106		13 - 17 May 2019			Reno, USA

3GPP RAN2#107 		26 - 30 Aug 2019    			Prague, Czechia