3GPP TSG-RAN WG2 Meeting #105bis	R2-1904294

Xian, China, 08 – 12 April 2019

Agenda item:	11.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	CFRA resources update for Conditional HO

WID/SID:	NR_Mob_enh - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the email discussion [1], it was identified that outdated CFRA resources is one of the problems which has to be solved for CHO in NR in comparison to CHO in LTE, when beamforming is used. In this contribution, we describe the problem in more detail, propose a split into subproblems and solution directions.

# 2	Problem of outdated CFRA resources

During the preparation phase of a legacy handover, the target cell may reserve a CFRA resource (i.e. a dedicated preamble) for the UE. If multiple beams are used, the target cell may reserve a single dedicated preamble on a single beam, or it may also reserve multiple dedicated preambles, each on a different beam. The reservation is typically based on the measurements which have previously been reported by the UE and which have been forwarded to the target gNB via Xn (i.e. in HO Request).

Even for the legacy handover, it may happen that, at the moment of executing the HO (i.e. random access to the target), the beam(s) for which dedicated preambles are reserved is (are) no longer valid/relevant. On one hand, this is not very likely to happen for the legacy HO since the time between measurement report and RACH is not too long; HO execution (i.e. RACH) follows HO preparation immediately, and HO preparation follows the measurement report immediately. On the other hand, this also implies that there is no opportunity to update the CFRA resources without introducing the same uncertainty again.

For CHO, the time between measurement report and execution might be much longer. Furthermore, for LTE it has been agreed [2] that the source cell remains RRC responsible for the UE until the execution (i.e. RRC Connection Reconfiguration Complete), so that there would be opportunities to update the CFRA resources.

Observation 1: The beam specific resources (e.g. CFRA preambles) may become obsolete in NR CHO due to a significantly longer (compared to LTE) time between the CHO preparation and the actual CHO execution.

Figure 1 illustrates an example for the described scenario. Black dots represent gNBs, different colors represent different cells, and different shades represent different beams of the same cell (32 beams per cell are shown). A single UE is shown which moves from the magenta cell towards the green cell following the white trajectory. The green target cell might be prepared for CHO when the measurements have indicated that beam #8 is the strongest. As a consequence, the target cell would resource a dedicated preamble on beam #8. Optionally, the target may also reserve  further dedicated preambles e.g. on beams #7 and #9 (or even more) to increase the likelihood of successful CFRA. However, we would like to emphasize that could be expensive, in particular since dedicated preambles may also be reserved for beam recovery.

Figure 1 Example for outdated CFRA resources

The example shows that it may easily happen that the CHO condition expires as late as when the UE has already proceeded to beam #5. So, even if the target cell has decided to prepare beam neighbors as well (e.g. #7 and #9), the UE would still have no CFRA resources for the strongest beam #5 and thereby falls back to CBRA (unless it decides to desperately access a weak beam with CFRA resources).

Hence, we will either see a lot of CBRA fallbacks for CHO (which induces more interruption and more interference), or we have to reserve a lot of CFRA resources for CHO (which might be too expensive).

Observation 2: For CHO there will be a trade-off between high costs for preparing many CFRA resources and drawback of many fallbacks to CBRA.

# 3	Solutions

In order to resolve the conflict described above, we should consider updates of the CFRA resources after a UE has been prepared for CHO to a specific target. We can split this problem into two parts: i) the update procedure itself and ii) providing the required information / measurements to the network.

## 3.1	Update procedure

In principle, the network can repeat the preparation procedure if an update is necessary. Let us assume for the moment, that the source cell realizes an update is necessary. It can send another HO Request, the target reserves new CFRA resources (on new beams), sends HO Request Acknowledge, and the source cell updates the RRC configuration inside the UE.

On one hand, this procedure seems to be quite exhaustive, so RAN2 (and obviously RAN3) should consider optimizations in a later stage. On the other hand, there are some race condition problems. The target cell has to keep the outdated CFRA reservations until it is really sure that the UE has received the updates. Otherwise there will be a conflict between the UE and the target configuration.

Proposal 1: A procedure for updating the CFRA resources for a prepared CHO shall be studied.

## 3.2	Triggers for update procedure

It is still open how the source cell can realize an update is necessary. This information has to be reported by the UE, as only the UE can measure the beams of the target cell. Unfortunately, the measurement reporting events for RRC reports are purely based on conditions using cell-level measurements. One idea could be to introduce new measurement reporting events based on neighbor’s beam measurements, e.g.

“Non-CFRA beam of prepared neighbour is offset better than (strongest/weakest) CFRA beam of the same neighbour”

Another study direction could be to have beam measurements of neighbors prepared for CHO in the periodic MAC reporting.

Proposal 2: RAN2 shall study how the UE can inform the network when CFRA resources configured for a CHO require an update.

# 4	Conclusion

This paper focused on outdated CFRA resources for CHO. The following observations and proposals have been made:

Observation 1: The beam specific resources (e.g. CFRA preambles) may become obsolete in NR CHO due to a significantly longer (compared to LTE) time between the CHO preparation and the actual CHO execution.

Observation 2: For CHO there will be a trade-off between high costs for preparing many CFRA resources and drawback of many fallbacks to CBRA.

Proposal 1: A procedure for updating the CFRA resources for a prepared CHO shall be studied.

Proposal 2: RAN2 shall study how the UE can inform the network when CFRA resources configured for a CHO require an update.

# 5	References

[1]	R2-1903450 Report of the email discussion [105#58][NR/MOB] Comparison of LTE and NR Conditional handover, Intel Corporation, 3GPP TSG-RAN WG2 Meeting #105bis	Xian, China, 8th – 12th of April 2019

[2] R2-1902238 Report from session on Rel-16 LTE Mobility Enhancements WI and NR Mobility Enhancements WI, 3GPP TSG-RAN WG2 Meeting #105	Athens, Greece, 25th February - 1st of March 2019