3GPP TSG-RAN WG2 Meeting 105bis	R2-1904451

# Xi’an, China, 8th Apr –12nd Apr, 2019

Agenda Item:	11.2.2.1

Souce:	Huawei, Hisilicon

Title:	Consideration on Paging for NR-U

Document for:	Discussion and decision

# 1 Introduction

In RAN#82, a WID on NR-based Access to Unlicensed Spectrum has been approved [1]. One of the objective is that

-	Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

In RAN2#105, the following agreements were made:

A UE can be configured for an additional number of monitoring occasions at or after or before (FFS) its calculated PO (when the paging message can be transmitted) FFS dynamic extension FFS dynamic termination

In this contribution, we further discuss the paging issues for NR-U and provide our proposals.

# 2 Discussion

Based on the TS 38.304 [2], the PF and PO calculation is given by the following formulae.

The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns

During SI stage, RAN2 agreed “For Inactive and Idle mode procedures, Rel-15 NR design is considered as the baseline.”  Therefore we believe the calculation of PF and PO in Rel-15 can be used as baseline.

Proposal 1: Calculation of PF and PO in Rel-15 can be used as baseline.

## 2.1 Additional page occasions

RAN2 has discussed the paging related issues, however no agreements on how to achieve additional paging occasions were made. Several potential solutions are available.

- Option 1: introduce multiple PDCCH occasions for several beam sweepings within one PO, these PDCCH occasions can be consecutive or interleaved. Considering LBT procedure, one COT includes several ms which can be used after LBT success. For consecutive PDCCH occasions, one disadvantage is all the consecutive PDCCH occasions may be blocked if these occasions are within a COT that is occupied by other device. The interleaving method has benefit on relieving LBT impact yet it is more complex. The UE may stop to monitor the remaining PDCCH occasions when paging message is received.

- Option 2: introduce multiple POs in one PF, i.e. introduce larger numbers of paging occasion for a PF (Ns). It is a simpler method with only new RRC value is to be introduced.

- Option 3: introduce dynamic termination. Some predefined rule are to be used and when LBT is blocked for one PO, the UE will determine next slot as additional paging occasion automatically. It is a relatively complex method.

Considering complicity aspect, we prefer option 2.

Proposal 2: Introduce multiple POs in one PF.

# 3 Conclusion

Based on the discussions above, we have the following observations and proposals:

Proposal 1: Calculation of PF and PO in Rel-15 can be used as baseline.

Proposal 2: Introduce multiple POs in one PF.

# 4 References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum.

- TS 38.804.

- 3GPP TS 36.300 f30.

- 3GPP TS 38.321 f30.