3GPP TSG-RAN WG2#105bis	R2-1904482

Xi’an, P.R. China, 8th – 12th April 2019

Agenda Item:	11.10.3

Source:	Ericsson

Title:	Handling of idle/inactive measurements during inter-RAT cell reselection

Document for:	Discussion, Decision

# Introduction

The Work Item Description for DC and CA enhancements in [1] was approved at the RAN#81 meeting. One of the objectives concerns how to reduce the delay until the setup of DC and/or CA through early measurements:

Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode

  - The impacts on UE power consumption should be minimized

  - The LTE Rel-15 euCA work should be utilized, when applicable

Early measurement reporting was discussed at the RAN2#105 meeting, with the following agreements:

Agreements:

For IDLE/INACTIVE

- Rel-16 early measurement configuration may contain both NR and LTE configuration, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. FFS on details.  IDLE mode and INACTIVE mode details will be discussed separately

- NR early measurement configuration should include NR specific measurement parameters configurations.

- Available beam and cell level measurement results can be included in early measurement reporting if configured.

It has been agreed that the Rel-16 early measurement configuration, i.e. for idle/inactive measurements, may contain both NR and LTE configurations. This allows support for fast setup of various MR-DC scenarios as well as CA. How these configurations and the corresponding measurements are to be handled in case the UE moves between the two RATs is however not clear. This is discussed in this paper.

# Discussion

## Idle mode measurements in LTE Rel-15

In LTE Rel-15 euCA the UE can be configured with an idle mode measurement configuration, for measurements on different E-UTRA carriers/cells, through measIdleConfig in RRCConnectionRelease and/or measIdleConfigSIB in SIB5. The UE performs the measurements while in RRC_IDLE and reports them to the network, if possible and requested, after entering RRC_CONNECTED.

The idle mode measurement configuration is stored in the UE until expiry or stop of timer T331. Timer T331 is stopped if the UE receives RRCConnectionSetup or RRCConnectionResume or, if a validity area is configured for the idle mode measurements, when the UE reselects to cell that is outside that area.

It can however be noted that it is up to UE implementation whether to continue the measurements, according to the broadcasted configuration in SIB5, also after stop or expiry of T331.

After successful transmission of the idle mode measurement results to the network (in UEInformationResponse), the UE discards the VarMeasIdleReport, i.e. the measurements that were transmitted. The stored measurements are also cleared if the UE receives an RRCConnectionRelease message with a new measIdleConfig included.

- In LTE Rel-15, the idle mode measurement configuration is stored at least until expiry of T331, until the UE receives RRCConnectionSetup or RRCConnectionResume or until the UE leaves the validity area, if configured.

- In LTE Rel-15, the idle mode measurements are stored until the UE has successfully reported them to the network or until the UE receives RRCConnectionRelease with a new measIdleConfig included.

## Validity of idle/inactive measurement at inter-RAT cell reselection

The Rel-16 idle/inactive measurements may include both NR and E-UTRA carriers/cells in order to support both CA and different MR-DC scenarios. The idle/inactive measurement configuration may thus include both NR and E-UTRA related configurations. The measurements that are performed may typically be useful for several different scenarios, such as for different variants of CA, NR-DC and/or MR-DC. For example, measurements performed on a specific NR carrier may be used either for setup of NR-DC (as PSCell) or NR CA (SCell) if the UE connects to an NR cell, or for setup of EN-DC (as PSCell) if the UE connects to an E-UTRA cell.

With the support of idle/inactive measurements in both LTE and NR, the question is then whether the idle/inactive measurements that are configured in one of the RATs should be possible to continue in the other RAT and if stored idle/inactive measurements should be possible to report in the other RAT than where they were configured. The UE may also receive idle/inactive measurement configuration through system information after an inter-RAT cell reselection, thus updating the current idle/inactive measurement configuration.

If E-UTRA and NR cells are deployed in the same area, and the idle/inactive measurement configurations then support both E-UTRA and NR configurations, a configuration received from an NR cell is likely very similar to a configuration received from a neighbouring E-UTRA cell in terms of which frequencies and cells that are included.

If a UE is configured with idle/inactive measurement configurations in one RAT and performs an inter-RAT cell reselection, the idle/inactive configurations and measurements from the source RAT are probably still relevant in the target RAT. For instance, if a UE is configured with idle/inactive measurements in E-UTRA for both E-UTRA and NR cells and later reselects to an NR cell, the NR measurements could be useful to quickly setup NR CA when the UE connects to the network. Thus, the UE should not release the idle/inactive measurement configurations or results when it performs an inter-RAT cell reselection, and it should be possible to report the stored idle/inactive measurements in the target RAT, even if these were configured in a different RAT.

UE can keep or release idle/inactive measurement configurations after an inter-RAT cell reselection based on network configuration. Details are FFS.

UE can keep or release idle/inactive measurement results after inter-RAT cell reselection based on network configuration. Details are FFS.

UE can report idle/inactive measurements to a different RAT than where the idle/inactive measurements was configured.

# Conclusion

In the previous sections we have made the following observations:

Observation 1	In LTE Rel-15, the idle mode measurement configuration is stored at least until expiry of T331, until the UE receives RRCConnectionSetup or RRCConnectionResume or until the UE leaves the validity area, if configured.

Observation 2	In LTE Rel-15, the idle mode measurements are stored until the UE has successfully reported them to the network or until the UE receives RRCConnectionRelease with a new measIdleConfig included.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE can keep or release idle/inactive measurement configurations after an inter-RAT cell reselection based on network configuration. Details are FFS.

Proposal 2	UE can keep or release idle/inactive measurement results after inter-RAT cell reselection based on network configuration. Details are FFS.

Proposal 3	UE can report idle/inactive measurements to a different RAT than where the idle/inactive measurements was configured.

# References

RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core), RAN#81