3GPP TSG-RAN WG2 #105bis	Tdoc R2-1904522

Xi’an, China, 8th April – 12th April 2019

Agenda Item:	11.6.3.3

Source:	Ericsson

Title:	On PDCP interruptions during handover for NTN

Document for:	Discussion, Decision

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The results are summarized in [2]. The new study item has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture.

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In this paper, we discuss impacts on PDCP interruptions on NTN.

# Discussion

Some important features of the PDCP layers are to offer reliable, secure and error free communication by using integrity protection, ciphering, reordering and retransmissions. PDCP makes use of transmit and receive window functionality to ensure these services are provided. In this paper we will mainly discuss PDCP and higher layer procedures during handovers.

## Re-transmissions on the PDCP layer

In addition to the re-transmission functionality of RLC and MAC(through HARQ), PDCP also provides the possibility of re-transmissions in certain cases. PDCP re-transmissions are however expected to be a lot less compared to HARQ or ARQ, since during normal operation RLC is expected to provide reliability to the PDCP layer. PDCP re-transmissions are typically only done during handover when there are no possibilities for the RLC layer to send any packets. Therefore, the PDCP layer needs to have its transmitted PDUs acknowledged similar to in the RLC layer.

PDCP retransmissions are enabled by PDCP re-establishment/PDCP recovery and PDCP status reports, and below we describe the general procedures for Data Radio Bearers utilizing AM RLC.

### PDCP re-establishment and PDCP data recovery

When the PDCP re-establishment is triggered the PDCP transmitting entity shall first reset the header compression state, then apply the ciphering algorithm and integrity protection algorithm as well as the corresponding given keys from the higher layers.

After the first two steps have been taken the PDCP entity shall retransmit all PDCP SDU starting from the first SDU that has not been acknowledged by the lower layers. This step is the same as in the PDCP recovery case.

The receiving PDCP entity shall process the data PDUs received from the lower layers due to the re-establishment, then reset header compression state as well as apply the deciphering and integrity protection algorithm with corresponding keys from the higher layers. If configured, the receiving entity shall also trigger a PDCP status report.

For the PDCP data recovery procedure the transmitting entity perform the same procedures except only retransmissions.

PDCP shall retransmit non-acknowledged PDCP PDUs during re-establishment and data recovery.

### PDCP acknowledging and status reports

Acknowledging of transmitted PDCP PDUs is different from RLC in that it can be done either through the corresponding RLC SDU being acknowledged by RLC or through PDCP status reports. In normal circumstances RLC is expected to acknowledge PDCP PDUs, and PDCP status reports are only used to acknowledge PDCP PDUs and does not trigger retransmissions as for RLC PDUs.

PDCP status reports can be configured to be triggered when the PDCP entity is either re-established or the PDCP data recovery is triggered.

PDCP PDUs can either be acknowledged through acknowledgement by RLC or through status reports.

## Connection control procedures

PDCP re-establishment is used after the handover procedure and may also be configured to be used when radio bearers are reconfigured. When RRC command to perform a handover(Reconfiguration with Sync) is received by the UE, the UE according to 38.331 shall:

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

This means that the UE shall interrupt any transmissions currently on-going, which also stops the UE from transmitting feedback(RLC or HARQ) for received packets and also stops the UE from receiving feedback(RLC or HARQ) for previously transmitted packets.

PDCP re-establishment is an important procedure during handovers.

## Interruptions during handover for NTN

For non-terrestrial networks the propagation delays are a lot bigger compared to the terrestrial case and the interruptions of the lower layers due to the handover is expected to be a lot bigger. In the contribution [3], possible interruptions due to the handover procedure can be seen. The time before receiving feedback is expected to be a lot longer compared to the terrestrial case, so the question remains how the PDCP re-establishment procedure would work in the non-terrestrial case.

Long propagation delays may cause problems for the PDCP re-establishment procedure.

One example of a problem that might occur is shown in Figure 1.

Figure 1. Figure showing the problem of retransmission of outstanding packets.

In this case before the handover is started, the UE has a lot of outstanding PDCP PDUs that have not been acknowledged by the network due to the large propagation delay. After the handover when the PDCP re-establishment procedure is started, as the UE shall retransmit non-acknowledged PDUs, the UE might need to retransmit a large number of PDUs that have already been successfully received by the network.

RAN2 to study PDCP re-establishment during NTN handovers.

# Conclusion

In section 2 we made the following observations:

Observation 1	PDCP shall retransmit non-acknowledged PDCP PDUs during re-establishment and data recovery.

Observation 2	PDCP PDUs can either be acknowledged through acknowledgement by RLC or through status reports.

Observation 3	PDCP re-establishment is an important procedure during handovers.

Observation 4	Long propagation delays may cause problems for the PDCP re-establishment procedure.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to study PDCP re-establishment during NTN handovers.

# References

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla, USA, June 2018

TR 38.811, Study on New Radio (NR) to support non terrestrial networks, Release 15

R2-190xxxx, Mobility for GEO NTN, Ericsson, RAN#105-Bis, Xi’an, China, April 2019