3GPP TSG-RAN WG2#105	R2-1904626

# Xian, China, 8 – 12 April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of handover between NR and EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-03-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR to EN-DC handover procedures are not defined. This change is solely related to handover between NR SA and EN-DC, and does not cover handovers: from ng-eNB to EN-DC; from NGEN-DC to EN-DC; from NR-DC to EN-DC. NOTE: This CR is based on a preliminar draft version of TS 37.340 v15.5.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.8 Master Node to eNB/gNB change - Addition of 10.8.x to include handover from EN-DC to NR. 10.9 eNB/gNB to Master Node change - Addition of 10.9.x to include handover from NR to EN-DC. Impact analysis If the network implements the CR and the UE does not, the UE will not expect to receive configuration for EN-DC setup in a handover command message. If the UE implements the CR and the network does not, there is no inter-operability issue, since even though the UE may support EN-DC setup during handover from NR, the network will not initiate it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC setup during handover from NR and EN-DC release within handover procedures from E-UTRA to NR will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8, 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This draft CR is to be re-written on top of the final version of the specification. |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 10.8.x	EN-DC to NR

The eNB MN to gNB change procedure is used to transfer UE context data from a source eNB MN and en-gNB SN to a target gNB node.

Figure 10.8.x-1: EN-DC to gNB change procedure

Figure 10.8.x-1 shows an example signalling flow for EN-DC to gNB change procedure:

1. Step 1-15 from clause 4.11.1.2.2.2 in TS 23.502 for handover from EPS to 5GS.

2. The source MME sends a Handover Command message to the source eNB. This message conveys a Target to Source transparent container, generated by the target MN, to be sent to the UE as an MN RRC message, to perform the handover, and may also provide forwarding addresses to the source MN.

3. The source MN initiates the release of the source SN resources towards the source SN including a Cause indicating MCG mobility. The source SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SgNB Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

4. The source MN triggers the UE to perform handover with MobilityFromEUTRACommand message and apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration.

5/6. The UE synchronizes to the target gNB node and replies with an RRC message to the gNB.

7. If applicable, data forwarding takes place from source MN and SN to target MN.

8.	Step 4-13 from clause 4.11.1.2.2.3 in TS 23.502 for handover from EPS to 5GS.

9.	Upon reception of the UE Context Release message from the MME (from clause 4.11.1.2.2.3 in TS 23.502, which is part of the handover execution phase), the source MN initiates UE Context Release towards the source SN, which releases radio and C-plane related resource associated to the UE context. Any ongoing data forwarding may continue.

END OF FIRST CHANGE

START OF SECOND CHANGE

### 10.9.x	NR to EN-DC

The NR to MN change procedure is used to transfer UE context data from a source gNB to a target MN connected to EPC, that adds an SgNB during the handover.

Figure 10.9.x-1: NR to eNB change procedure with SgNB addition

Figure 10.9.x-1 shows an example signalling flow for NR to E-UTRA change procedure, with SgNB addition:

1. Step 1-6 from clause 4.11.1.2.1 in TS 23.502 for handover from 5GS to EPS.

2. The target MN, based on the measurement results it has got during the handover preparation part 1, decides to initiate EN-DC, and sends a SgNB Addition Request to the target SN.

3. The target SN replies with SgNB Addition Request Acknowledge. If data forwarding is needed, the target SN provides forwarding addresses to the target MN.

4. Step 7-10 from clause 4.11.1.2.1 in TS 23.502 for handover from 5GS to EPS, including 10a, 10b and 10c.

5. The AMF sends a Handover Command message to the source NR node. This message conveys a Target to Source transparent container, generated by the target MN, to be sent to the UE as an E-UTRA RRC message, including the SCG configuration, to perform the handover, and may also provide forwarding addresses to the source NG-RAN node.

6. The source NR node triggers the UE to perform handover with MobilityFromNRCommand message and apply the new configuration.

7/8. The UE synchronizes to the target MN and replies with MN RRCConnectionReconfigurationComplete message.

9.	The UE synchronizes to the target SN.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the target SN via SgNB Reconfiguration Complete message.

11. If applicable, data forwarding takes place from source MN to target MN and SN.

12.	Step 12b-21 from clause 4.11.1.2.1 in TS 23.502 for handover from 5GS to EPS, including 12b,12c and 12d.

END OF SECOND CHANGE