3GPP TSG-RAN WG2#105bis	R2-1904627

# Xian, China, 8 – 12 April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of handover between NR-DC and E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-03-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR-DC to E-UTRA handover procedures are not defined. This change is solely related to handover between NR-DC and E-UTRA (single connectivity), and does not cover handovers: from ng-eNB to EN-DC; from NGEN-DC to EN-DC; from NR to EN-DC. NOTE: This CR is based on a preliminar draft version of TS 37.340 v15.5.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.8 Master Node to eNB/gNB change - Addition of 10.8.x to include handover from NR-DC to eNB. 10.9 eNB/gNB to Master Node change - Addition of 10.9.x to include handover from eNB to NR-DC change. Impact analysis If the network implements the CR and the UE does not, the UE will not expect to receive configuration for NR-DC or NR setup in a handover command message. If the UE implements the CR and the network does not, there is no inter-operability issue, since even though the UE may support EN-DC setup during handover from NR or NR setup during handover from E-UTRA, the network will not initiate it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-DC setup during handover from E-UTRA and NR-DC release within handover procedures from NR to E-UTRA will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8, 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This draft CR is to be re-written on top of the final version of the specification. |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 10.8.x	NR-DC to eNB change

The NR-DC to eNB change procedure is used to transfer UE context data from a source gNB MN/SN to a target eNB.

Figure 10.8.x-1: NR-DC to eNB change procedure

Figure 10.8.x-1 shows an example signalling flow for NR-DC to eNB change procedure:

1. Step 1-10 from clause 4.11.1.2.1 in TS 23.502 [XX] for handover from 5GS to EPS, including steps 10a, 10b and 10c.

2. The AMF sends a Handover Command message to the source gNB node. This message conveys a Target to Source transparent container, generated by the target MN, to be sent to the UE as an E-UTRA RRC message, including a NR RRC configuration message, to perform the handover, and may also provide forwarding addresses to the source MN.

3. The source MN initiates the release of the source SN resources towards the source SN including a cause indicating MCG mobility. The source SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SN Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

4. The source MN triggers the UE to perform handover with MobilityFromNRCommand message and apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration.

5/6. The UE synchronizes to the target eNB and replies with MN RRCConnectionReconfigurationComplete message.

7. If applicable, data forwarding takes place from source MN and SN to target MN.

8. Step 12b-21 from clause 4.11.1.2.1 in TS 23.502 [XX] for handover from 5GS to EPS, including 12b,12c and 12d.

9. Upon reception of the UE Context Release message from the AMF (from clause 4.11.1.2.1 in TS 23.502 [XX]), the source MN initiates UE Context Release towards the source SN, which releases radio and C-plane related resource associated to the UE context. Any ongoing data forwarding may continue.

END OF FIRST CHANGE

START OF SECOND CHANGE

### 10.9.x	eNB to NR-DC change

The eNB to MN change procedure is used to transfer UE context data from a source eNB to a target gNB that adds an SN during the handover.

Figure 10.9.x-1: eNB to gNB change procedure with SN addition

Figure 10.9.x-1 shows an example signalling flow for eNB to gNB change procedure, with SN addition:

1. Step 1-9 from clause 4.11.1.2.2.2 in TS 23.502 [XX] (“EPS to 5GS handover using N26 interface”) for handover preparation phase.

2. If the target MN, based on the measurement results it has got during the handover preparation part 1, decides to initiate DC, and sends a SN Addition Request to the target SN.

3. The target SN replies with SN Addition Request Acknowledge. If data forwarding is needed, the target SN provides forwarding addresses to the target MN.

4. Step 10-15 from clause 4.11.1.2.2.2 in TS 23.502 [XX] for handover preparation phase.

5. The source MME sends a Handover Command message to the source eNodeB. This message conveys a Target to Source transparent container, generated by the target MN, to be sent to the UE as an MN RRC message including a SN RRC configuration message which also includes the SCG configuration, to perform the handover, and may also provide forwarding addresses to the source eNB.

6. The source eNB triggers the UE to perform handover with MobilityFromEUTRACommand message and apply the new configuration.

7/8. The UE synchronizes to the target MN and replies with MN RRCReconfigurationComplete message.

9. The UE synchronizes to the target SN.

10.	If the RRC reconfiguration procedure was successful, the target MN informs the target SN via SN Reconfiguration Complete message.

11. If applicable, data forwarding takes place from source MN to target MN and SN.

12.	Step 4-13 from clause 4.11.1.2.2.3 in TS 23.502 [XX] for handover execution phase.

END OF SECOND CHANGE