3GPP TSG-RAN2 105-bis                                                 	R2-1904643

Xi’an, China, April 8th – 12th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3956 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TDD UL DL Alignment offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The offset between the UL carrier frequency center with respect to DL carrier frequency center was initially described in the RAN1 specifications, but eventually it was removed to be captured only in the RAN4 specifications (R4-1816194). The corresponding change needs to be reflected in 36.331, and TDD-UL-DL-AlignmentOffset-NB should refer to the RAN4 specifications instead of the RAN1 specifications. Impact analysis NB-TDD Impacted functionality: UL DL Alignment offset Inter-operability: 1. If the network is implemented according to the CR and the UE is not, UE may not be able to find the UL carrier. 2. If the UE is implemented according to the CR and the network is not, UE may not be able to find the UL carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the TDD-UL-DL-AlignmentOffset-NB has been updated according to the information in R4-1816194. Accordingly, the reference to the RAN1 specification has been removed, and a reference to the RAN4 specification has been added (R4-1816191). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The re will be an ambiguity when selecting the offset of category NB1/NB2 Channel Number to EARFCN, with respect to what is indicated by the IE TDD-UL-DL-AlignmentOffset-NB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of change

#### 6.7.3.2	NB-IoT Radio resource control information elements

#### –	CarrierConfigDedicated-NB

The IE CarrierConfigDedicated-NB is used to specify a carrier in NB-IoT.

<Skipped unmodified changes>

#### –	TDD-UL-DL-AlignmentOffset-NB

The IE TDD-UL-DL-AlignmentOffset-NB is used to specify the offset between the UL carrier frequency center with respect to DL carrier frequency center. This information should be used when selecting the offset of category NB1/NB2 Channel Number to EARFCN, as defined in  TS 36.101 [42].

TDD-UL-DL-AlignmentOffset-NB information element

-- ASN1START

TDD-UL-DL-AlignmentOffset-NB-r15 ::=				ENUMERATED {	khz-7dot5, khz0, khz7dot5}

-- ASN1STOP

End of change