3GPP TSG-RAN2 Meeting #105bis	R2-1904655

# Xian, China, 8 – 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on mandatory information in inter node RRC messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NOTE: This CR is based on a preliminar draft version of TS 38.331 v15.5.0. In CR 799r2 (R2-1902787) a new section 12.2.3 was added on Mandatory information in inter-node RRC messages. The purpose of the section is to clarify the presence of fields in inter node messages, e.g. how the source shall set the fields for fileds used to reflect the AS configuration of the UE. There is also a general statement that “For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise”. After this statement there is a list of fields for which the receiver shall maintain the values informed by the fields. The current list includes: - measGapConfig (for which delta signaling applies); - ue-CapabilityInfo - configRestrictInfo During the email discussion where the CR was agreed, come companies indicated that there could be also other fields which could be added to the list of exceptions. - drx-InfoMCG, drx-InfoSCG. These fields contain the IE DRX-Info, in which long DRX is mandatory field and short DRX optional. This means that adding this field to the list means that once a long DRX cycle is once signalled, it is not possible to signal a “release” of the long DRX cycle. However, as the field is only used to inform the receiving node of the DRX cycle of the sending node, in order to allow DRX alignment, we don’t think this is a critical restriction. - measConfigMN, measConfigSN. These fields are used to inform the receiving node of the measured frequencies and gap configuration of the sending node. All subfields of the field are optional, and the gap configuration is already listed as exception, so we propose to add these fields to the list. - selectedBandCombinationNR. This field in CG-Config is used to indicate the band combination selected by the SN. It uses the IE BandCombinationInfoSN, which contains only mandatory fields, so once signalled, it will not be possible to fully release the configuration. However, considering that SCG must always have at least one band assigned, the release should not be needed. - fr-InfoListSCG. This field in CG-Config contains FR information of serving cells configured in SCG, i.e. whether FR1 or FR2. It uses the the IE FR-Info containing only mandatory fields, so once signalled, it will not be possible to fully release the configuration. However, considering that SCG must always have at least one serving cell configured, the release should not be needed. - pSCellFrequency. This field in CG-Config indicates the frequency of the PSCell in SCG. It uses the IE ARFCN-ValueNR, which contains mandatory integer to indicate the frequency, but considering that SCG must always have the PSCell configured, the full release should not be needed. - ph-InfoMCG, ph-InfoSCG. These fields carry power headroom information between MN and SN that is needed in the reception of PHR MAC CE. They use the IE PH-InfoMCG and PH-InfoSCG, containing mandatory fields, so once configured, it will not be possible to fully release the configuration. However, given that both MCG and SCG need to contain at least one serving cell each, this should not be an issue. Some further editing is also needed of the text presenting the list of fields for which absence means the receiver maintains the values. Currently the explanation is given after the list, but it would be clearer to give the explanation before the list. Also, it would be good to clarify that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted. Finally, for fields that convey the UE configuration in CG-Config, the case of SN initiated SN change is currently mentioned, but the case of MN enquiry of SN configuration is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 12.2.3 Mandatory information in inter-node RRC messages - Added the case of MN enquiry of SN configuration to fields that convey the UE configuration in CG-Config - Moved the text explaining the excemption list to before the list - Added the following fields to the list of exceptions: - drx-InfoMCG; - drx-InfoSCG; - measConfigMN; - measConfigSN; - selectedBandCombination; - fr-InfoListSCG; - pSCellFrequency; - ph-InfoMCG; - ph-InfoSCG. Impact analysis: Impacted architectures: EN-DC Impacted functionalities: Inter node messages Interoperability analysis: This CR is provided only for clarification and does not require any new NW/UE implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be not clear about when certain fields in INMs are present. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
|  |  | This draft CR is to be re-written on top of the final version of the specification. |  |  |  |  |  |  |  |  |

START OF CHANGES

### 11.2.3	Mandatory information in inter-node RRC messages

For the AS-Config transferred within the HandoverPreparationInformation:

-	The source node shall include all fields necessary to reflect the AS configuration of the UE;

-	Need codes or conditions specified for subfields according to IEs defined in section 6 do not apply. I.e. some fields shall be included regardless of the "Need" or "Cond" e.g. discardTimer;

-	Based on the received AS configuration, the target node can indicate the delta (difference) to the UE’s AS configuration (as included in HandoverCommand).

For a field that conveys the UE configuration in CG-Config (SN initiated change of SN and MN enquiry of SN configuration) and in CG-ConfigInfo upon change of SN (i.e. mcg-RB-Config, scg-RB-Config and sourceConfigSCG):

-	The source node shall include all fields necessary to reflect the AS configuration of the UE, unless stated otherwise in the field description or in this sub-clause;

-	Need codes or conditions specified for subfields according to IEs defined in section 6 do not apply;

-	Based on the received AS configuration, the target node can indicate the delta (difference) to the UE’s AS configuration (as included in CG-Config).

For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise. As an exception to this general rule, the absence of the below listed fields means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted:

-	measGapConfig (for which delta signaling applies);

-	ue-CapabilityInfo;

-	configRestrictInfo;

-	drx-InfoMCG;

-	drx-InfoSCG;

-	measConfigMN;

-	measConfigSN;

-	selectedBandCombination;

-	fr-InfoListSCG;

-	pSCellFrequency;

-	ph-InfoMCG;

-	ph-InfoSCG.

END OF CHANGES