3GPP TSG-RAN WG2 #105-bis	 TDoc R2-1904707

# Xi’an, China, 8th – 12th April 2019

Agenda Item:	11.4.6a

Source:	Ericsson

Title:	On lower layer IDs

Document for:	Discussion, Decision

# Introduction

In LTE SL, the L2 source ID and destination ID are used to identify the transmitter and the target receiver, and the packet filtering is performed by L2 based on L2 ID conveyed in MAC header. In NR SL, L2 IDs are determined by V2X layer, e.g. converted from application ID, and delivered to lower layers. The use of lower layer IDs need to consider some different aspects, e.g. L1 ID is needed to support HARQ procedure. This paper further discusses the use of L2 ID and L1 ID taking into account the progress in RAN1 and SA2.

During the SI phase, RAN2 has agreed that for unicast and groupcast, source ID and destination ID should be visible at Layer 2, however, FFS if packet filtering will be performed at layer 2.

RAN2#103-bis Agreement: - For groupcast, destination ID for a specific group and for unicast, destination ID for the target UE need to be visible in Layer 2 respectively. Source UE id should be also visible to Layer 2 RAN2#104 Agreement: - RAN2 will capture L2 packet filtering function with the condition (i.e. if full L1 id is not used in L1 control information). It is FFS whether we need additional filtering function for unicast and groupcast.

# Discussion

## Packet filtering

In LTE, the source/destination L2 ID are appended in the MAC subheader of each SL MAC PDU. From the source L2 ID, which is a 24-bit sequence, the receiver can distinguish the different transmitters. From the destination L2 ID, which is a 16-bit or 24-bit sequence, the receiver can distinguish the different V2X services that the MAC PDU conveys. For example, from the destination L2 ID, the receiving MAC entity can determine whether to discard the MAC PDU, if the content is not of interest, or to pass it to higher layers.

In LTE, the source L2 ID (24-bit) and the destination L2 ID (16/24-bit) is conveyed in the MAC subheader.

The introduction of such layer-1 destination ID to be conveyed via PSCCH may significantly improve sidelink decoding performances compared to LTE. For example, a packet which is not for the interested UE and/or service need not be received on PSSCH. This gives a significant gain in terms of latency, UE decoding efforts, and finally packet overhead.

In previous RAN1 meetings (RAN1#94-bis meeting, RAN1#ah1901), RAN1 has agreed that L1 IDs should be also transmitted at L1 in the PSCCH.

RAN1#ah1901 Agreement: - Layer-1 destination ID can be explicitly included in SCI - FFS how to determine Layer-1 destination ID - FFS size of Layer-1 destination ID - The following additional information can be included in SCI - Layer-1 source ID - FFS how to determine Layer-1 source ID - FFS size of Layer-1 source ID - HARQ process ID - NDI - RV - FFS whether some of the above information may not be present etc. in some operations (e.g., depending on whether they are used for unicast, groupcast, broadcast)

In RAN1#94-bis, RAN1 has agreed to introduce layer-1 destination ID to be conveyed via PSCCH to enhance SL decoding performances.

If the PSCCH already contains useful information to unambiguously detect the intended receiver/service, packet filtering can be performed at L1. Then one may wonder if it is still needed to convey the L2 IDs as part of the MAC subheader. If this is the case, the MAC design can be simplified a lot with a significant gain in terms of overhead and system efficiency.

L1 source and destination IDs are conveyed in SCI, as such MAC PDU subheader does not convey L2 IDs.

L1 performs packet filtering for SL unicast/groupcast/broadcast.

Of course, the L2 destination ID can still be useful for the UE to identify the specific V2X service being transmitted and select the transmitting parameters accordingly. However it might not be needed to transmit such L2 destination ID explicitly in the L2 MAC header.

Even if a layer-1 destination ID is introduced, the legacy layer-2 destination ID might still be needed to allow a transmitter UE identifying a V2X service and apply the associated transmitting policies, e.g. TX profile selection, QoS criteria selection, TX parameters selection, etc.

## L1 ID determination

Besides, it seems natural to let L2 determine L1 IDs based on given L2 IDs taking into account other aspects such as the link information for unicast. Particularly, for groupcast and broadcast, L1 source/destination IDs can be the same as the corresponding L2 source/destination IDs. As such, L1 source ID reflects the transmitter UE while L1 destination ID reflects the tended group/service.

L2 determines L1 IDs based on given L2 IDs taking into account other aspects such as the link information for unicast.

For SL groupcast/broadcast L1 source/destination IDs are the same as the corresponding L2 source/destination IDs.

For SL unicast, between the same UE pair, it is allowed to establish multiple links using same or different source IDs.

SA2 TS 23.287: 5.6.1.4 Identifiers for unicast mode V2X communication over PC5 reference point A UE may establish multiple unicast links with a peer UE and use the same or different source Layer-2 IDs for these unicast links. Editor's note: Further updates of the identifier description may be required based on RAN WG feedback.

The purpose of such design is to keep certain flexibility for upper layer link management. However, we foresee some critical impacts to link management at access stratum. For instance, it is not clear that whether the peer UE can understand if different source IDs are referring to the same transmitter UE. Thus, it will be problematic for a UE to know if the UE capability received via one link can be applied to other links as well. Besides, having link management at access stratum, e.g. RLM/RLF, for all links between the same UE pair seems to be unnecessary.

As for L1 ID, between the same UE pair for different links with different source L2 IDs the corresponding L1 IDs can also be different. However, in our view, this is not necessary and may cause issues for other procedures e.g. CSI report. First of all, from packet filtering perspective, the receiver UE will decode all packets from the peer UE even if those packets belong to different links. Secondly, among different links between the same UE pair, the channel condition is always the same. Therefore, it makes no sense to aqcuire CSI report for different links deduced from different source/destination L1 ID pairs between the same UE pair.

For SL unicast, a UE may establish multiple unicast links with a peer UE and use the same or different source Layer-2 IDs for these unicast links. Impacts to access stratum design are foreseen with respect to UE capability exchange, RLM/RLF procedure, and CSI report.

RAN2 investigates the impacts of allowing one UE to use multiple L2 source IDs for communication with the same peer UE. If needed, RAN2 sends LS to SA2 to clarify and feedback RAN2’s view.

Moreover, different from SL groupcast and broadcast, a global unique ID is not always necessary for SL unicast communication. After a unicast link is established between two UEs, they don’t need a full L1 ID, e.g. 24bits, to identify each other. Instead, a local L1 ID of smaller size might be enough, as long as one UE can distinguish different links with the same or different peer UEs. Therefore, after two UEs discovers each other and establish a unicast link, they can exchange their short local L1 IDs for future communication, i.e. short local L1 IDs are conveyed in SCI instead of full L1 ID.

Shortened local L1 IDs can be configured and conveyed in SCI for SL unicast after link establishment.

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE, the source L2 ID (24-bit) and the destination L2 ID (16/24-bit) is conveyed in the MAC subheader.

Observation 2	In RAN1#94-bis, RAN1 has agreed to introduce layer-1 destination ID to be conveyed via PSCCH to enhance SL decoding performances.

Observation 3	Even if a layer-1 destination ID is introduced, the legacy layer-2 destination ID might still be needed to allow a transmitter UE identifying a V2X service and apply the associated transmitting policies, e.g. TX profile selection, QoS criteria selection, TX parameters selection, etc.

Observation 4	For SL unicast, a UE may establish multiple unicast links with a peer UE and use the same or different source Layer-2 IDs for these unicast links. Impacts to access stratum design are foreseen with respect to UE capability exchange, RLM/RLF procedure, and CSI report.

Based on the discussion in section 2 we propose the following:

Proposal 1	L1 source and destination IDs are conveyed in SCI, as such MAC PDU subheader does not convey L2 IDs.

Proposal 2	L1 performs packet filtering for SL unicast/groupcast/broadcast.

Proposal 3	L2 determines L1 IDs based on given L2 IDs taking into account other aspects such as the link information for unicast.

Proposal 4	For SL groupcast/broadcast L1 source/destination IDs are the same as the corresponding L2 source/destination IDs.

Proposal 5	RAN2 investigates the impacts of allowing one UE to use multiple L2 source IDs for communication with the same peer UE. If needed, RAN2 sends LS to SA2 to clarify and feedback RAN2’s view.

Proposal 6	RAN2/RAN1 supports using shortened local L1 IDs in SCI for SL unicast after link establishment.