3GPP RAN WG2 Meeting #105bis	R2-1904984

Xi’an, China, April 8-12, 2019                                        (Resubmission of R2-1901546)

Agenda item:	11.9.3

Source:	Apple Inc.

Title:	On NR Mobility in FR2

Document for:	Approval

# Introduction

The NR Mobility Enhancement Work Item [1] was approved in 3GPP RAN #80 meeting.  An objective of [1] is “to study solution(s) to improve HO/SCG change reliability and robustness especially considering challenges in high/med frequency”. In this contribution, we evaluate the performance of existing mobility solutions when used in the millimeter frequency range, FR2.

# Discussion

## Background

The Rel. 15 NR work item [2] adopts basic mobility procedures defined for LTE. In the existing NR mobility mechanism, the network controls UE mobility based on UE measurement reports. The serving gNB receives the measurement reports from the UE, and selects a target cell for handover (HO). The serving gNB triggers the handover by sending a HO request to the target cell, and after receiving a HO ACK from the target gNB, the serving gNB initiates the HO by sending a HO command to the UE. The UE performs PRACH to the target cell after RRC is reconfigured with the target cell configuration.

NR is expected to work in the frequency range higher than 24 GHz known as FR2 [3]. UE beams are directional in FR2 and therefore when a UE moves or rotates it may experience very fast signal degradation. Such rapid degradation will result in increased signalling, a high frequency of HO and associated failure rate, and finally, more ping-pong between cells. Therefore, HO is more challenging in NR at FR2 than in LTE.

In this paper, we present simulation results evaluating the performance of existing mobility solutions when used in FR2. Specifically, we study the impact of slow rotation of the UE on the frequency of HO when operating in FR2.

## Simulation Model

In order to study the mobility performance of NR UEs operating at FR2, it is crucial to incorporate the impact of antenna radiation patterns in simulations to more accurately model the impact of such movements on the received/transmitted signal power. In addition to antenna radiation patterns the number of antenna panels and their locations (front and/or back) also impact mobility performance.

The modeling of antenna radiation patterns in FR2 was conducted by RAN1 and RAN4 for NR SI in Rel. 14 and the reports are available in TR 38.802 [5] and TR 38.803 [4] respectively. RAN4 has also conducted a network simulation study for co-existence in FR2 incorporating antenna and RF models. The details of simulation models are given in [4].

Both TR 38.802 [5] and TR 38.803 [4] provide similar network layout and propagation models for three different deployment scenarios considered for FR2 system simulations: Urban Macro (UMa), Dense Urban, Indoor hotspot (InH) environments. Furthermore, RAN4 has updated relevant simulation assumptions in TR 38.817 [6]. Figure 1 shows the network configuration for UMa and InH.

- UMA														 b) InH

Figure 1: Network Configurations from TR 38.803

Figure 2 shows a 2-D view of the UE antenna radiation pattern as introduced in [4-5] for azimuth in [-180, 180] range. As shown in Figure 2, the antenna radiation pattern is directional with 25 dB signal attenuation at the sides compared to the antenna boresight.

Figure 2: Antenna Radiation Pattern from TR 38.803

Although both TR 38.802 [5] and TR 38.803 [4] have provided similar models for simulating FR2 networks, we found that simulation models provided in TR 38.803 [4] is sufficient for simulating mobility performance in FR2. Furthermore, RAN4 has used the same model for studying the spherical coverage requirement [3, 6] which is relevant to studying impact of UE rotation on mobility in FR2. We therefore adopted the antenna radiation pattern, network layout, and propagation models provided in TR 38.803 [4] and TR 38.817 [6].

A summary of relevant parameters from TR 38.803 [4] is given in Table 1:

Table 1: Summary of Simulation Model for FR2

| Scenario (in TR38.803) | Indoor hotspots | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| Network Layout Model | Table 5.2.1.3-1 | Table 5.2.1.2-1 | Table 5.2.1.1-1 |
| Propagation Model | Section 5.2.2 |  |  |
| BS Antenna Model | Table 5.2.3.2.3-1 | Table 5.2.3.2.2-1 | Table 5.2.3.2.1-1 |
| UE antenna element pattern | Table 5.2.3.3-1 |  |  |
| UE Elevation distribution | Fixed at 90 |  |  |
| Indoor ratio | 100% | 0%(baseline), 20%, 100% |  |
| Blockage Modeling | No body blockage and hand grip modelling |  |  |
| Traffic model | Full buffer | Full buffer | Full buffer |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 9dB |  |  |
| UE Noise figure in dB | 9dB |  |  |
| Minimum SINR (for outage) | -10dB |  |  |

Proposal 1: RAN2 adopts the network simulation models provided in TR 38.803 for FR2 mobility simulations as summarized in Table 1.

### Simulation Results

In this paper, we simulated a UMa network as shown in Figure 1a comprised of 19 gNB and 57 cells operating at the 28 GHz band. We also simulated an Indoor Hotspot network as shown in Figure 1b. We adopted the simulation parameters provided in TR38.803 as summarized in Table 1. The goal of our simulations is to study the impact of slow rotation of a UE on mobility by measuring the received RSRP at the UE.

We assumed UEs are at fixed locations but rotating at a speed of 0.2 Hz. We therefore measure received RSRP from all surrounding cells over a 5 seconds duration. Furthermore, we assumed 100 ms is required to sweep across all Rx beams to find the best Rx beam from each of the neighboring cells maximizing the RSRP. The UE sends measurement reports every 100 ms. Therefore, consecutive HOs are within 100 ms of each other.

We varied the handover margin (HoM) from 0 dB to 3 dB and measured the number of HO attempts for each UE. Figures 3 and 4 show the CDF of HO attempts in UMa and InH networks respectively. As shown in in Figure 3, more than 50% of UEs will have at least 3 extra HO within 5 seconds in UMa networks. We note that since the UEs are only rotating at fixed locations, most of the attempted HO is due to the operation at FR2 and incorporating directional antenna patterns. In other words, we don’t expect to see any HO if UEs operate in FR1 and use omni-directional antennae in similar deployment scenarios.

Figure 3: CDF of HO attempts in UMa network

Figure 4: CDF of HO attempts in InH network

Observation 1: NR UEs using directional antenna and operating in FR2 will attempt more HO due to change of antenna direction.

Figure 5 shows a snapshot of RSRP at a sample UE. It is observed that RSRP degradation due to rotation of UE can be more 20dB.

Figure 5: RSRP variation due to rotation at a sample NR UE

Observation 2: RSRP degradation due to rotation of NR UE using directional antenna and operating in FR2 can be significant.

Given the large number of HO attempts at FR2, we recommend that RAN2 consider the performance and feasibility of proposed mobility solutions on required signalling and interruption time of HO for NR UEs using directional antenna and operating in FR2.

Proposal 2: RAN2 to consider the performance of proposed mobility solutions on required signalling and interruption time of HO for NR UEs using directional antenna and operating in FR2.

# Proposals (for approval)

In this paper, we presented simulation results showing the performance of existing mobility solutions for NR UEs using directional antenna and operating in FR2. We particularly studied the impact of slow rotations of UE on the number of HO attempts and showed some challenges of supporting mobility in NR at FR2.

Based on the simulation results, the following observations and proposal are made:

Observation 1: NR UEs using directional antenna and operating in FR2 will attempt more HO due to change of antenna direction.

Observation 2: RSRP degradation due to rotation of NR UE using directional antenna and operating in FR2 can be significant.

Proposal 1: RAN2 adopts the network simulation models provided in TR 38.803 for FR2 mobility simulations as summarized in Table 1.

Proposal 2: RAN2 to consider the performance of proposed mobility solutions on required signalling and interruption time of HO for NR UEs using directional antenna and operating in FR2.

# References

- RP-181433, “New WID: NR mobility enhancements,” Intel Corporation, 3GPP RAN #80, May 2018

- RP-172834, “New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #78, December 2017

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- TR38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects,” 3GPP, September 2017

- TR38.802 v14.2.0, “Study on new radio access technology Physical layer aspects,” 3GPP, Sep.  2017

- TR38.817-1 v15.2.0, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR,” 3GPP, January 2019