3GPP TSG-RAN WG2 Meeting #105bis	R2-1904997

# Xi’an, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE capability of lch-ToSCellRestriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the definition for the MAC parameter lch-ToSCellRestriction in 4.2.6 in TS 38.306, it is specified that a UE supporting pdcp-Duplication (see PDCP-Config) shall also support lch-ToSCellRestriction. However, there are four pdcp-Duplication parameters defined in 4.2.4 as presented below, where pdcp-DuplicationMCG-OrSCG-DRB and pdcp-DuplicationSRB are for CA based duplication while pdcp-DuplicationSplitDRB and pdcp-DuplicationSplitSRB refer to DC duplication. pdcp-DuplicationMCG-OrSCG-DRB Indicates whether the UE supports CA-based PDCP duplication over MCG or SCG DRB as specified in TS 38.323 [16].; UE; No; No. pdcp-DuplicationSplitDRB Indicates whether the UE supports PDCP duplication over split DRB as specified in TS 38.323 [16].; UE; No; No. pdcp-DuplicationSplitSRB Indicates whether the UE supports PDCP duplication over split SRB1/2 as specified in TS 38.323 [16].; UE; No; No. pdcp-DuplicationSRB Indicates whether the UE supports CA-based PDCP duplication over SRB1/2 and/or, if EN-DC is supported, SRB3 as specified in TS 38.323 [16].; UE; No; No. As a common understanding, the restriction specified in lch-ToSCellRestriction should only be applied to CA based duplication but not necesasry to DC based duplication. In other words, if the UE only supports DC based PDCP duplication but not the CA-based, then there is no need to make the UE mandatorily support lch-ToSCellRestriction. |
| Summary of change: |  | Clarify in the definition for lch-ToSCellRestriction that a UE supporting pdcp-DuplicationMCG-OrSCG-DRB and pdcp-DuplicationSRB shall support lch-ToSCellRestriction. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: PDCP duplication Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue foreseen, as the capability lch-ToSCellRestriction and other duplication related capabilities are reported separately. The UE may be implemented to mandatorily support lch-ToSCellRestriction if it supporting DC based duplication even if CA based duplication is not supported, but it will cause confusion to the network. 2. If the UE is implemented according to the CR and the network is not, there could be an inter-operability issue. The UE supporting DC based duplication only may not report the support of lch-ToSCellRestriction. Although the two capabilities are separately report, this could lead to a confusion to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The linkage between the capability lch-ToSCellRestriction and duplication based capabities are incorrect. The UE may be required to support lch-ToSCellRestriction when it only supports DC based duplication only but it is actually unnecessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on an old spec |  |  |  |  |  |  |  |  |

START OF CHANGE

4.2.6	MAC parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| lch-ToSCellRestriction Indicates whether the UE supports restricting data transmission from a given LCH to a configured (sub-) set of serving cells (see allowedServingCells in LogicalChannelConfig). A UE supporting pdcp-DuplicationMCG-OrSCG-DRB or pdcp-DuplicationSRB (see PDCP-Config) shall also support lch-ToSCellRestriction. | UE | No | No | No |
| lcp-Restriction Indicates whether UE supports the selection of logical channels for each UL grant based on RRC configured restriction. | UE | No | No | No |
| logicalChannelSR-DelayTimer Indicates whether the UE supports the logicalChannelSR-DelayTimer as specified in TS 38.321 [8]. | UE | No | Yes | No |
| longDRX-Cycle Indicates whether UE supports long DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| multipleConfiguredGrants Indicates whether UE supports more than one configured grant configurations (including both Type 1 and Type 2) in a cell group. For each cell, the UE supports at most one configured grant per BWP and the maximum number of configured grant configurations per cell group is 2. If absent, for each configured cell group, the UE only supports one configured grant configuration on one serving cell. | UE | No | Yes | No |
| multipleSR-Configurations Indicates whether the UE supports TS 38.321 [8] SR configurations per cell group. | UE | No | Yes | No |
| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 [8]. | UE | No | No | No |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321 [8]. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| shortDRX-Cycle Indicates whether UE supports short DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| skipUplinkTxDynamic Indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as specified in TS 38.321 [8]. | UE | No | Yes | No |

END OF CHANGE