3GPP TSG-RAN WG2 #105bis			        R2-1905084

Xi’an, China, April 8th – 12th, 2019

Agenda Item	: 9.7

Source	: LG Electronics Inc.

Title	: Consideration on IDLE measurement in RRC_INACTIVE

Document for	: Discussion and Decision

1.	Introduction

In this contribution, we would like to consider whether IDLE measurement is applied to RRC_INACTIVE or not.

2.	Discussion

2.1 IDLE measurement in TS 36.331

In legacy specification [1], idle measurement can be configured in RRCConnectionRelease message. Hence, measIdleConfig can be configured when a UE is moving from RRC_CONNECTED to RRC_INACTIVE.

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond UP-EDT

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

After idle measurement is performed, an indication of availability can be reported in MSG5 while a UE is moving from RRC_INACTIVE to RRC_CONNECTED.

RRCConnectionResumeComplete-v1530-IEs ::= SEQUENCE {

logMeasAvailableBT-r15					ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15				ENUMERATED {true}				OPTIONAL,

idleMeasAvailable-r15					ENUMERATED {true}				OPTIONAL,

flightPathInfoAvailable-r15				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

In legacy specification [1], applies RRC_IDLE procedures unless specified otherwise:

RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	A RAN-based notification area is configured by RRC layer;

-	The UE stores the UE Inactive AS context;

-	The UE:

-	Applies RRC_IDLE procedures unless specified otherwise;

-	Monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI;

-	Performs periodic RAN-based notification area update;

-	Performs RAN-based notification area update when moving out of the configured RAN-based notification area;

5.3.3	RRC connection establishment

Figure 5.3.3.1-3: RRC connection resume (suspended RRC connection or RRC_INACTIVE), or UP-EDT fallback to RRC connection resume, successful

2.1 IDLE measurement in TS 36.133

According to section 4A [2], measurement requirements in RRC_INACTIVE state follows requirements in RRC_IDLE. Hence, it seems that there is no issue to follow idle measurement in RRC_INACTIVE.

Observation 1. There is no specification issue to perform IDLE measurement in RRC_INACTIVE.

Observation 2. It is confusing whether to support IDLE measurement in RRC_INACTIVE or not

Proposal 1. It is necessary to clarify whether idle measurement can be performed in RRC_INACTIVE or not.

2.2 Possible approaches

According to legacy specification [1][2], there is no issue to perform IDLE measurement in RRC_INACTIVE. However, performing the idle measurement is assuming that a UE in RRC_IDLE so far. In order to clarify this issue, we propose two options and CRs.

Option 1. Adopt idle measurement in RRC_INACTIVE. Hence, idle measurement can be performed in RRC_IDLE or RRC_INACTIVE [3].

Option 2. Insert description to perform idle measurement only while a UE is in RRC_IDLE [4].

Proposal 2. RAN2 needs to consider adopting one of the proposed option.

3.	Conclusion

Observation 1. There is no specification issue to perform IDLE measurement in RRC_INACTIVE.

Observation 2. It is confusing whether to support IDLE measurement in RRC_INACTIVE or not

Proposal 1. It is necessary to clarify whether idle measurement can be performed in RRC_INACTIVE or not.

Proposal 2. RAN2 needs to consider adopting one of the proposed option.

References

[1] 3GPP TS 36.331, v15.5.0.

[2] 3GPP TS 36.133, v16.0.0

[3] 36331_R2-1905107_Correction on Idle mode measurement in RRC_INACTIVE (option 1)

[4] 36331_R2-1905113_Correction on Idle mode measurement in RRC_INACTIVE (option 2)