3GPP TSG-RAN WG2 Meeting #105bis	R2-1905101

# Xi’an, China, 08 Apr - 12 Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | Draft | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on periodicityAndOffset in ZP-CSI-RS-Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For periodicityAndOffset field in IE ZP-CSI-RS-Resource, it is optional with Cond PeriodicOrSemiPersistent. But the corresponding description of PeriodicOrSemiPersistent is missing, it may lead to some confusion about when this field is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of Cond PeriodicOrSemiPersistent in IE ZP-CSI-RS-Resource. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: ZP-CSI-RS Inter-operability: If the UE is implemented according to the CR and the network is not, network may not configure periodicityAndOffset for periodic or semi-persistent ZP-CSI-RS-Resource (as indicated in PDSCH-Config), so UE cannot obtain periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS, it may lead to a reconfiguration failure. If the network is implemented according to the CR and the UE is not, there is interoperability issue, since the network will provide this periodicityAndOffset configuration when needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may not configure periodicityAndOffset for periodic and semi-persistent ZP-CSI-RS-Resource (as indicated in PDSCH-Config), so UE cannot obtain periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS, it may lead to a reconfiguration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The baseline specification is draft TS 38.331-f50 |  |  |  |  |  |  |  |  |

CHANGE START

6.3.2	Radio resource control information elements

<unnecessary parts omitted>

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource (see TS 38.214 [19], clause 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                        OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=            INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS (see TS 38.214 [19], clause 5.1.4.2).

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot.

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID (see TS 38.214 [19], clause 5.1.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is mandatory present for periodic and semi-persistent ZP-CSI-RS-Resource (as indicated in PDSCH-Config). The field is absent otherwise. |

<unnecessary parts omitted>

CHANGE END