3GPP TSG RAN WG2 #105bis	R2-1905178

Xi’an, China, 8th – 12th April, 2019					  (was R2-1904497)

Agenda item:	11.4.6

Source: 	Qualcomm Incorporated

Title: 	NR Uu control of LTE sidelink

Document for:	Discussion and Decision

# Introduction

LTE V2X was envisioned to support BSM messages. NR V2X in contrast supports primarily advanced applications. In previous evaluations, it has been considered by RAN and SA that NR V2X is not intended to completely replace LTE V2X applications but instead to compliment LTE especially when the requirements are not supported by LTE. This therefore requires LTE and NR to coexist with each other in different channels and also interact with each other implicitly through the application layer. This inter-operability potentially also requires network support from NR to control and manage the LTE sidelink.

As communication networks move to NR gradually replacing LTE, it becomes necessary for the NR Uu link to be able to control the LTE sidelink as the LTE sidelink is expected to carry at least BSM messages for the foreseeable future. At the introduction of the NR V2X WI, RAN has also explicitly asked RAN1 to introduce and specify support for NR Uu to control LTE sidelink operation for both mode-4 and mode-3 as shown below.

Specify support for NR Uu to provide control for LTE sidelink - Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and - Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2]. - RAN1 to make a decision on which option is supported until RAN#84.

RAN1 is also tasked with concluding whether RRC-based SPS scheduling would be adequate or whether DCI-based scheduling would be additionally required. These topics are discussed in more detail in the following sections.

# NR Uu Control of LTE V2X Sidelink

LTE Uu control of the LTE sidelink has been specified and supports both mode-3 and mode-4 operation with multiple options for both. For the most part, these can be duplicated in NR in order to achieve the same purpose.

## 2.1	Introducing NR Uu control of LTE sidelink for Mode-4 operation

For mode-4 operation when the UE is in coverage, the LTE Uu provides the necessary configuration information (including the configured resource pool) for autonomous resource allocation and scheduling. This configuration information is provided by either a dedicated RRC configuration message or through the SIB signaling (SIB 21 in the case of LTE). The UE then performs sensing and resource selection operations in a fairly autonomous fashion similar to UEs out-of-coverage.

Providing for two mechanisms to convey the necessary information allows the UE to access the information when in RRC connected mode or even in RRC idle state. A similar approach can be followed for NR control of LTE mode-4 as well. RRC signaling in the NR context needs to be introduced to convey the sidelink configuration (including resource pool information) for operation of the LTE sidelink. This signaling can be introduced to be within the framework of NR and can be interpreted by the UE appropriately for application to the LTE sidelink. Introduction of the RRC configuration details into an appropriate SIB in NR is also needed with a reference to indicate that the configuration pertains to the LTE sidelink for the UE to interpret accordingly. Since additionally functionality needs to be implemented in the UE to enable this feature, this needs to be a separate UE capability indication.

These signaling details can be introduced as part of the RAN2 work on introducing RRC configurations for the NR sidelink. An LS asking RAN2 to look into the introduction of this signaling has already been sent during the study item.

Proposal 1: Introduce RRC signaling and SIB signaling similar to LTE specifications in NR to enable NR Uu control of LTE mode-4 operation in the sidelink.

Proposal 2: Introduce UE capability signaling for NR Uu control of LTE mode-4 operation.

## 2.2	Introducing NR Uu control of LTE sidelink for Mode-3 operation

For mode-3 operation, the LTE Uu controls the LTE sidelink through both dynamic resource allocation as well as SPS resource allocation. For both types of operation, appropriate RRC messages are communicated by the UE requesting for necessary configuration parameters. Following this exchange, DCI is transmitted (DCI-5A in the case of LTE). In the case of dynamic resource allocation, BSR messages are needed so that resources are allocated in a need-based dynamic fashion. These mechanisms were functionally feasible within the context of LTE but in the context of NR much more complexity is introduced if all the functionality to be adopted.

In NR Uu specification, there are Type 1 and Type 2 types of grant-free signaling defined. Type 1 enables UL data transmission for NR Uu based on RRC signaling to indicate the configuration. Type 2 enables UL transmissions based on both RRC and DCI based signaling.

It is considered that RRC signalling similar to Type 1 defined in NR Uu may be used for controlling mode-3 operation of LTE sidelink. Some modifications are necessary since Type-1 was introduced for each UE and not for the entire cell. Additionally, the activation and release mechanism for the configured resources needs to be included in the specification along. Timing information also needs to be provided as part of this signalling. The timing information included should consider potential inter-RAT communications delays and so appropriate delays need to be built in to the specification.

With regards to the introduction of DCI based signalling, several problems arise. First, the content or size of the DCI structure defined in LTE is different from the DCI content that will be fined for the NR specification. Any additional DCI format introduced would have a different content size as a result. This increases the complexity of blind DCI detection considerably since the UE would have to account for different sizes in addition to the different types of DCI formats. Second, the DCI would need to be differentiated between LTE and NR and so unique RNTIs would have to be used. Additionally, due to the delay in inter-RAT signalling, the delays incurred for the application of the DCI would result in the operation being similar to that defined by RRC signalling procedure anyway. So, the purpose of introducing DCI signalling would essentially be defeated. Due to these reasons, during the study item, only an RRC based mechanism was agreed to be supported during the WI phase.

Some modification to the LTE procedure is also likely since the RRC specification would be received through inter-RAT communication. However, since these aspects are internal to UE implementation, explicit specification change to the LTE specifications may not be needed.

Due to the introduction of this additional functionality, UE capability signalling is needed to enable this feature for UEs capable of both LTE and NR V2X.

Proposal 3: Introduce RRC signalling (similar to Type-1) in NR Uu for SPS scheduling for LTE mode-3 operation

Proposal 4: Introduce UE capability signalling for NR Uu control of LTE mode-3 operation.

## 2.3	Introduction of UE assistance signaling to gNB

In LTE V2X, the UE sends assistance information through RRC signaling for the network to allocate an appropriate amount of resources.

A similar approach can be followed in NR as well. The RRC signaling message could also be combined with the request for NR sidelink resources. The details of how the signaling is to be defined could be left to RAN2.

In LTE, BSR messages are also transmitted to enable dynamic resource allocation. If we rely on RRC messaging for NR Uu control of LTE sidelink, then BSR messages would not be required. Therefore, it is proposed that BSR messages specific to LTE sidelink scheduling are not included as part of the NR specification.

Proposal 5: Introduce NR UL RRC signaling for the UE to convey requests for sidelink access.

Proposal 6: No BSR signaling is introduced as a result of NR Uu controlling LTE sidelink.

# Conclusion

In this contribution, we discuss the control of LTE sidelink by NR Uu both for mode-3 and mode-4. The following is proposed:

Proposal 1: Introduce RRC signaling and SIB signaling similar to LTE specifications in NR to enable NR Uu control of LTE mode-4 operation in the sidelink.

Proposal 2: Introduce UE capability signaling for NR Uu control of LTE mode-4 operation.

Proposal 3: Introduce RRC signalling (similar to Type-1) in NR Uu for SPS scheduling for LTE mode-3 operation

Proposal 4: Introduce UE capability signalling for NR Uu control of LTE mode-3 operation.

Proposal 5: Introduce NR UL RRC signaling for the UE to convey requests for sidelink access.

Proposal 6: No BSR signaling is introduced as a result of NR Uu controlling LTE sidelink.

# References

- RP-190766, “New WID on 5G V2X with NR sidelink”