3GPP TSG-RAN WG2 Meeting #105bis	 R2-1905298

Xi’An, China, 8-12th April 2019

Source:	Thales, Vodafone

Title:	Proposed NR – NTN target performances

Agenda Item:       11.6.2

Document for:     Discussion

# 1 Introduction

This paper propose initial target performances for the NTN NG-RAN.

# 2 Discussion

Performance requirements relative to NR-NTN are needed for WGs to carry out analysis.

This TDOC provides target performances figures as well as the related environment and terminal characteristics assumptions mainly derived from TS 22.261 and TR 38.913 and the related sources.

In addition, it proposes to add a usage scenarios for narrowband IoT connectivity as suggested by Vodafone in RAN2#105 meeting.

Proposal 1: Please consider these initial target performance figures as well as the related environment and terminal characteristics assumptions mainly derived from TS 22.261 and TR 38.913.

# 4 Proposed text for TR 38.821 v0.4.0

----- start of text proposal -----

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[10]	AIAA 2006-6753, Revisiting Spacetrack Report #3 , Models for Propagation of NORAD Element Sets, David A. Vallado, Paul Crawford, Richard Hujsak, T. S. Kelso, presented at the AIAA/AAS Astrodynamics Specialist Conference, Keystone, CO, 2006 August 21–24, 2006

[11]	3GPP TS 22.261 “Service requirements for the 5G system; Stage 1 (Release 16)”

[12]	3GPP TR 38.913 “Study on Scenarios and Requirements for  Next Generation Access Technologies; (Release 15)”

[13]	3GPP TR 45.820 “Cellular system support for ultra-low complexity and  low throughput Internet of Things (CIoT) (Release 13)”

[14]	R2-1901404 “IoT Device Density Models for Various Environments”, Vodafone, submitted to RAN2#105

----- end of text proposal -----

----- start of text proposal -----

# B.2	Performance targets for evaluation purposes

This table includes performance values that may be used for theoretical analysis or simulations.

The values relate to targeted performances, but should not be considered as strict requirements.

Table B.2-1: Non-Terrestrial network target performances per usage scenarios

| Usage scenarios | Peak data rate (note 1) |  | Experience data rate (note 2) |  | Overall UE density per km2 | Activity factor (note 3) | Max UE speed | Environment | UE categories | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | Downlink | Uplink |  |  |  |  |  |  |
| Pedestrian |  |  | 2 Mbps | 60 kbps | TBD | 1,50% | 3 km/h | Extreme coverage | Handheld | Data rate => see 7.10.1 "extreme coverage performance" in [12]; UE density => TBD % of rural macro in [11]; |
|  |  |  |  |  |  |  |  |  |  | Activity factor: see table 6.1.6-1 "extreme rural" in [12] |
| Vehicular connectivity |  |  | 50 Mbps | 25 Mbps | TBD | 50% | 250 km/h | Along roads in low population densirty areas | Vehicular mounted | data rate and activity factor in TS 22.261; UE density => TBD % of High-speed vehicle in [11] |
| Stationary |  |  | 50 Mbps | 25 Mbps | TBD | TBD | 0km/h | Extreme coverage | Building mounted | Data rate =>assuming per end-user 50/25 Mbps data rate and an average of 5 end-user devices per stationary UE) rate and 20% activity factor per end-user device |
| Airplanes connectivity |  |  | 360 Mbps | 180 Mbps | TBD | TBD | 1 000 km/h | Open area | Airplane mounted | Data rate =>assuming per end-user 15/7.5 Mbps data rate and 20% activity factor per end-user devices (See [11]); nber of users per airplane: average aircraft size (assuming 120 users per plane) |
| IoT connectivity (Note 4) |  |  | 2 kbps | 10 kbps | 400 | 1,00% | 0 km/h | Extreme coverage | IoT | Device density => [14] ; Data rate and activity factor => derived from [13] annex E.2 "Traffic models for Cellular IoT" |

NOTE 1: Peak Data rate is defined in [12]

NOTE 2: Experienced data rate is defined in [12]

NOTE 3: Activity factor is defined in [11]

NOTE 4: This refers to low power wide area service capability

NOTE 5: This does not preclude the definition of performance parameters for other usage scenarios in future stages

Signalling (control plane) and data (User plane) Mobility interruption time should be made as much as possible equivalent as in terrestrial systems except in the case of handover between satellite and terrestrial access. In the latter case, this interruption time will depend on the satellite orbit.

----- end of text proposal -----