3GPP TSG-RAN WG2 Meeting #105bis	R2-1905322

Xi’an, China, 8th – 12th of April 2019

Title:	LS on Conditional Handover in E-UTRAN and NR

Response to:	-

Release:	Release 16

Work Item:	LTE_feMob-Core, NR_Mob_enh-Core

Source:	Nokia

To:	TSG RAN WG3

Cc:	-

Contact Person:

#### Name:	Jedrzej Stanczak

E-mail Address:	jedrzej.stanczak [at] nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

As a part of Rel-16 LTE_feMob-Core and NR_Mob_enh-Core work items, RAN2 works on the details of Conditional Handover (CHO). CHO is the solution, wherein the UE does not immediately execute the HO command (i.e. RRC Connection Reconfiguration with mobilityControlInfo in case of E-UTRAN) but stores it until a triggering condition is met which has been configured along with the HO command. When the condition is met, the UE immediately executes the HO command and accesses the target cell. It has been agreed there could be a single or multiple candidate target cells for CHO and it is up to network’s decision how many cells to prepare.

RAN2#105 has additionally agreed that late packet forwarding (i.e. not done immediately when CHO target candidate cells become prepared) could be more suitable for CHO, especially when there are multiple candidate target cells. However, early packet forwarding can be also considered as an option.

RAN2 still discusses the details of HO command and triggering conditions (e.g. whether it is the source eNB/gNB, target eNB/gNB or source + target eNB/gNB that decide on such condition, and as a result, whether the condition is included in HO command or included directly by the source eNB/gNB in the reconfiguration message). This may impact the HO Preparation phase. Additionally, RAN2 already agreed the network can inform the UE to release the CHO configurations after CHO command. This may happen also due to target eNB’s/gNB’s decision to cancel the preparation. Thus, the impact on X2/Xn interface is foreseen.

Based on the CHO details described above, RAN3 is kindly asked to study and potentially define suitable mechanisms for E-UTRAN’s and NR’s CHO, at least in the following areas:

- Data forwarding

- HO preparation

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to study at least data forwarding and HO preparation mechanisms or enhancements to the existing ones, suitable for Conditional Handover in E-UTRAN and NR.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#106		13 – 17 May 2019		Reno, USA

3GPP RAN2#107		26 – 30 August 2019		Prague, Czechia