3GPP TSG-RAN WG2 Meeting #105bis                                                                       R2-1905434

Xi’an, China, 8th  – 12th  April 2019

Agenda Item:	    10.4.1.3.4

Source: 	    MediaTek Inc.

Title:		    SCG configurable fields in RRC Reconfiguration

Document for:	    Discussion and decision

# 1 Introduction

This is report for the following offline discussion.

R2-1904468	Restriction of SCG configurable fields in  RRCReconfiguration	Intel Corporation	draftCR	Rel-15	38.331	15.5.0	F	NR_newRAT-Core

-	Ericsson think this is already clear. This is specified in the RRC reconfiguration procedure that only certain fields are included. LG have the same view. MediaTek think this is correct for SRB1 but it is not so clear for SRB3 case.

-	Nokia think even for the SRB3 case there are requirements on what can be configured via SRB3.

-	Intel now understand that this is being added by tee rapporteur CR for the late drop but this needs to be clear for EN-DC.

=>	Offline activity to check that the spec is clear on which fields of the reconfiguration message can be included for reconfiguration sent via the embedded message and send via SRB3 (Offline discussion 17, MediaTek)

R2-1903653	RRC Reconfiguration via SRB3 in EN-DC	MediaTek Inc.	draftCR	Rel-15	38.331	15.5.0	F	NR_newRAT-Core

=>	Discussed jointly with the previous CR.

# 2 Discussion

## 2.1 Background

In RAN2#105bis, two companies propose to clarify what should be included in RRCReconfiguration for SCG configuration in EN-DC [1] [2]. We discuss this in 2 cases:

- The NR RRCReconfiguration embedded in LTE RRC message (sent via SRB1)

- The NR RRCReconfiguration sent via SRB3

## 2.2 NR RRCReconfiguration embedded in LTE

In case that the NR RRCReconfiguration embedded in LTE RRC message (sent via SRB1), we have the following description in field description in 36.331.

RRCConnectionReconfiguration field descriptions

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroup and/ or measConfig. If nr-SecondaryCellGroupConfig is configured, the network always includes this field upon MN handover to initiate an NR SCG reconfiguration with sync and key change.

It seems clearly specified which fields of the reconfiguration message can be included for reconfiguration sent via the embedded message.

Question 1: Does companies agree that only secondaryCellGroup and/or measConfig could be included for NR reconfiguration embedded in LTE RRC message and it is clear in current specification?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | We think that the description in 36.331 is clear. Only secondaryCellGroup and measConfig could be included in this case. |
| Intel | Yes | It is clear from TS36.331 |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Huawei | Yes |  |

Proposal 1: RAN2 confirms that, In EN-DC, only secondaryCellGroup and/or measConfig could be included for NR RRCReconfiguration embedded in LTE RRC message.

## 2.3 NR RRCReconfiguration sent via SRB3

Regarding to the NR RRCReconfiguration sent via SRB3, 38.331 specifies that the about the usage of SRB3 in EN-DC in 5.3.5.1 as below:

-----------------------------------------------------------------

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

<Remove unrelated parts>

In EN-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, provided that the (re-)configuration does not require any MeNB involvement.

-------------------------------------------------------------------

The description does not say anything about IE name, some companies may think the above text is not as clear as in the LTE field description (SRB1 case). We would like establish the common understand on which configurations could be included in RRCReconfiguration sent via SRB3 first. And then discuss whether further clarification is needed. The following is the ASN.1 define for NR RRCReconfiguration message. The key question would be if there any configurations other than secondaryCellGroup, measConfig, and/or radioBearerConfig would be included in SRB3.

RRCReconfiguration-IEs ::=          SEQUENCE {

radioBearerConfig    RadioBearerConfig       				   OPTIONAL, -- Need M

secondaryCellGroup   OCTET STRING (CONTAINING CellGroupConfig) OPTIONAL, -- Need M

measConfig           MeasConfig                                OPTIONAL, -- Need M

lateNonCriticalExtension    OCTET STRING                       OPTIONAL,

nonCriticalExtension        RRCReconfiguration-v1530-IEs       OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup   OCTET STRING (CONTAINING CellGroupConfig)  OPTIONAL, -- Need M

fullConfig        ENUMERATED {true}                          OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList    SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message   OPTIONAL, -- Cond nonHO

masterKeyUpdate             MasterKeyUpdate                  OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery      OCTET STRING (CONTAINING SIB1)   OPTIONAL, -- Need N

dedicatedSystemInformationDelivery  OCTET STRING (CONTAINING SystemInformation)   OPTIONAL, -- Need N

otherConfig       OtherConfig                                OPTIONAL, -- Need M

nonCriticalExtension        RRCReconfiguration-v1540-IEs     OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=       SEQUENCE {

otherConfig-v1540              OtherConfig-v1540        OPTIONAL, -- Need M

nonCriticalExtension           SEQUENCE {}              OPTIONAL

}

Question 2: Does companies agree that only secondaryCellGroup, measConfig, and/or radioBearerConfig could be included in NR RRCReconfiguration sent via SRB3? If no, which additional filed could be included?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | In the 1st release of EN-DC in 38.331, it is clear that only the radioBearerConfig, secondaryCellGroup, and/ or measConfig could be sent via SRB3 because there is no other extension at that time. We think that all IE(s) added after RRCReconfiguration-v1530-IEs extension is for NR SA. |
| Intel | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Huawei | Yes |  |

Proposal 2: RAN2 confirms that, In EN-DC, only secondaryCellGroup, measConfig, and/or radioBearerConfig could be included in NR RRCReconfiguration sent via SRB3.

Question 3: If answer of question 2 is yes, do you think current specification is clear enough on this aspect? If no, any suggestion on how to clarify it?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | We think the description on 5.3.5.1 is a little bit unclear. Especially, it does not prevent network to send the fullConfig flag. One can consider that this fullConfig is used to reset the configure of MAC/RLC/PHY in SCG. We would prefer to have a clear statesment on which fields could be included in SRB3, as what we have done in SRB1 case. We also put a draft CR in the folder. The suggestion is simply to add “In EN-DC, only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3.” in 5.3.5.1. |
| Intel | No | Agree with the addition from Mediatek |
| Samsung | No | Agree with the suggestion from MediaTek |
| Ericsson | No | We are fine with the changes proposed my Mediatek |
| Huawei | No |  |

Proposal 3: Agree the CR in R2-1905435 to clarify the SRB3 configuration restriction for EN-DC.

# 3 Conclusions

Base on the discussion in section 2, we have the following proposals:

Proposal 1: RAN2 confirms that, in EN-DC, only secondaryCellGroup and/or measConfig could be included in NR RRCReconfiguration embedded in LTE RRC message.

Proposal 2: RAN2 confirms that, in EN-DC, only secondaryCellGroup, measConfig, and/or radioBearerConfig could be included in NR RRCReconfiguration sent via SRB3.

Proposal 3: Agree the CR in R2-1905435 to clarify the SRB3 configuration restriction for EN-DC.

# 4 References

[1] R2-1904468, “Restriction of SCG configurable fields in RRCReconfiguration”, Intel

[2] R2-1903653, “RRC Reconfiguration via SRB3 in EN-DC”, MediaTek