3GPP TSG-RAN WG2 Meeting #105bis	R2-1905435

# Xi’an, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | NNN | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Reconfiguration via SRB3 in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Nokia, Nokia Shanghai Bell, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 04/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 38.331 does not clearly say, which fields can be present in RRCReconfiguration received via SRB3 in EN-DC. For SRB1 case (i.e. RRCReconfiguration received via LTE RRCConnectionReconfiguration), it is said for nr-SecondaryCellGroupConfig in 36.331 that: "In this version of the specification, the NR RRC message only includes fields secondaryCellGroup and/ or measConfig". In the 1st release of EN-DC in 38.331, it is clear that only the radioBearerConfig, secondaryCellGroup, and/ or measConfig could be sent via SRB3 because there is no other extension at that time. RRCReconfiguration-IEs ::= SEQUENCE { radioBearerConfig RadioBearerConfig OPTIONAL, -- Need M secondaryCellGroup OCTET STRING (CONTAINING CellGroupConfig) OPTIONAL, -- Need M measConfig MeasConfig OPTIONAL, -- Need M lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RRCReconfiguration-v1530-IEs OPTIONAL } However, with further extensions added in RRCReconfiguration due to NR SA, it is not clear whether the later IEs could be sent via SRB3 in EN-DC. We think that the new IEs such as masterCellGroup, fullConfig, dedicatedNAS-MessageList, otherConfig are not designed for EN-DC. Thus, the network should not send this IEs via SRB3. Therefore, we propose to clarify that only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.3.5.1, specify that only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3. Impact analysis Impacted 5G architecture options: EN-DC Impacted functionality: RRC Reconfiguration procedure received via SRB3 in EN-DC Inter-operability: If the network implements the CR but the UE does not, there is no Inter-operability issue. The network will not include the IEs other than measConfig, radioBearerConfig and secondaryCellGroup and this does not cause any problem to legacy UE. If the UE implements the CR but the network does not, there is Inter-operability issue. The IEs other than measConfig, radioBearerConfig and secondaryCellGroup are not designed for EN-DC. The UE behaviour is unclear if those configurations are received via SRB3 in EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour is unclear if the configuration other than measConfig, radioBearerConfig and secondaryCellGroup are received via SRB3 in EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on draft 38.331-f50 |  |  |  |  |  |  |  |  |

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

In EN-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, provided that the (re-)configuration does not require any MeNB involvement. In EN-DC, only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3.