3GPP TSG-RAN WG2 Meeting #105bis			Tdoc R2-1905442

Xian, China, 8th– 12th April 2019

Title:	[DRAFT] Draft LS on Release-with-Redirect in 2-step resume procedure

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson [To be RAN2]

To:	CT1

Cc:

Contact Person:

#### Name:	Oscar Ohlsson

E-mail Address:	oscar.ohlsson@ericsson.com

Send any reply LS to:3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

For a NAS triggered resume procedure the network may release and redirect the UE, i.e. after entering RRC_CONNECTED the UE receives a RRCRelease message containing redirectedCarrierInfo and suspendConfig to move the UE back to RRC_INACTIVE and select a new cell (redirection). For this case RAN2 understanding is that NAS will re-trigger the NAS procedure (if still needed) when the UE enters the new cell.

However, RAN2 has also discussed another case where the release and re-direct is performed in 2-steps when the UE starts from RRC_INACTIVE state (2-step release and redirect). In this case the UE does not enter RRC_CONNECTED but is directly released with redirect information after sending the resume request. This means that the UE will remain in RRC_INACTIVE state and then select the new cell.

It is unclear to RAN2 whether the scenario has been addressed in current NAS specification, and what NAS behavior is, e.g. if NAS will re-trigger the NAS procedure also for 2-step release and re-direct?

If the 2-step resume procedure with redirection as described above is not supported by NAS then RAN2 will add a restriction into RRC spec, i.e. the network shall not use 2-step resume procedure with redirection for NAS triggered events. From RAN2 side there is no strong need to support this case in Rel-15 and therefore does not require any NAS changes.

2. Actions:

To CT1 group.

ACTION: CT1 is kindly requested to answer if the 2-step resume procedure with redirection described above is supported by NAS.

3. Date of Next TSG WG RAN2 Meetings:

RAN2#106	13 - 17 May 2019		Reno, US

RAN2#107	26 - 30 Aug 2019		Prague, Czech Republic