3GPP TSG-RAN WG2 Meeting#105bis          							R2-1905459

Xi'an, China, 8th – 12th April 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Report of offline discussion #21 Clarification on Msg4 delivery during resume and reestablishment procedure

Agenda item:		10.4.1.3.6

Document for: 	Discussion and Decision

- Introduction

This is the report of the following offline discussion:

R2-1903616	Clarification on msg4 delivery during the resume and reestablishment procedure	ZTE corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

-	 Ericsson also think there is no requirements that MAC CE and RRC message need to be in the same TB. Nokia have the same view.

-	Samsung think this was discussed in LTE Rel-8 and think they can be sent in different TBs. But that was for establishment so we need more time to check for resume procedure.

-	Qualcomm would also like time to check. Intel explain in LTE it was specified in MAC spec.

-	OPPO think a CR would be good to make this clear.

=>	Time given for more checking until Friday. On Friday we will try to take a principle decision and then a CR related to that decision can be prepared for the next meeting (Offline discussion 21, ZTE)

- Discussion

2.1 Background

As described in TS38.300, the RRCRelease message with suspend indication will be sent to UE after the path switch procedure is finished.

Figure 1: RNA update procedure with UE context relocation

However, as the path switch procedure via the NG interface may, in some cases take longer than the maximum value of the ra-ContentionResolutionTimer (which is 64ms), by the time PATH SWITCH REQUEST RESPONSE is received, the ra-ContentionResolutionTimer may have already expired and UE will consider the contention resolution unsuccessful and indicate a Random Access problem to upper layers. UE may have to re-try the RNAU procedure again and again until the PATH SWITCH REQUEST RESPONSE is received in the network side.

2.1 Delivering Msg4 and RRC messages in different TBs

The following solution is considered to help with the RANU procedure and prevent UE from trying it again and again.

Figure 2: Send Msg4 containing contention resolution ID to UE after the context retrieve procedure

As shown in figure 2, Msg4 (i.e. a UE Contention Resolution Identity MAC CE) is sent to UE before the ra-ContentionResolutionTimer expires. The RRCRelease with suspend indication is then sent to UE after the path switch procedure is finished.

Since the intention of such solution is to reduce the unsuccessful contention resolutions caused by the latency in path switch procedure, the most essential part in the first downlink message from serving gNB to UE is the contention resolution ID. Other information is not necessary as it will anyway be provided by the following RRCRelease with suspend indication message.

In addition to the RNAU procedure, the same situation happens during the resume procedure when UE resumes from RRC_INACTIVE to RRC_CONNECTED as well as the RRC Reestablishment procedure in which the RRCResume message or RRCReestablishment message is sent to UE after successful context retrieve procedure. A successful context retrieve procedure may take longer than the typical value range of the ra-ContentionResolutionTimer (which is 8ms - 64ms). Thus, the ra-ContentionResolutionTimer may expire even before the context retrieve procedure is finished. Thus, the solution (i.e. delivering Msg4 and RRC message in different TBs) may also be applied during RRC Resume procedure and Reestablishment procedure to reduce the unsuccessful contention resolutions caused by the latency in context retrieve.

We want to confirm that delivering Msg4 and RRC messages in different TBs is allowed and should be supported by all the UEs.

Question 1: Can companies confirm that delivering Msg4 and RRC messages in different TBs is allowed in NR and should be supported by all the UEs for the following procedures?

- RRC connection establishment;

- RRC connection resume;

- RRC connection re-establishment.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| CMCC | Yes |  |
| NTT DOCOMO | Yes |  |
| MediaTek | Yes |  |
| OPPO | Yes |  |
| NEC | Yes |  |
| vivo | Yes |  |

Question 2: If the answer to Question 1 is “Yes”, do companies think a CR is needed? If companies think a CR is needed for clarification, a stage 2 CR (e.g. a 38.300 CR) or a stage 3 CR (e.g. a 38.321 CR)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | An agreement captured in the chairman-notes will be sufficient. |
| Ericsson | No |  |
| Nokia | No | Agree with ZTE |
| Huawei, HiSilicon | No | Agree with ZTE |
| CATT | No | Agree with ZTE |
| CMCC | No | Agree with ZTE |
| NTT DOCOMO | No | Agree with ZTE. It is noted that there was a CR proposed in UP session in NR AH meeting Montreal (R2-1810007) to capture tables of MAC header structure for contention resolution (as in LTE MAC) and this was not agreed since the structure of MAC PDU has changed from LTE. |
| MediaTek | No |  |
| OPPO |  | No strong opinion. No CR is OK. |
| NEC | No | Agree with ZTE |
| vivo | No | Agree with ZTE |

- Conclusion and proposals

Since the majority of the companies confirm that delivering Msg4 and RRC messages in different TBs is allowed and should be supported by all the UEs, we suggest to agree the following proposal:

Proposal 1: RAN2 confirm that delivering Msg4 and RRC messages in different TBs is allowed and should be supported by all the UEs for the following procedures:

- RRC connection establishment;

- RRC connection resume;

- RRC connection re-establishment.