3GPP TSG RAN WG2#105bis	R2-1903010

Xi'an, China, 8 – 12, April, 2019

3GPP TSG RAN WG1 Meeting #96	R1-1903697

Athens, Greece, 25th February – 1st March, 2019

Title:	LS on support of Enhancements on multi-TRP/panel transmission

Response to:

Release:	Rel-16

Study Item:	NR_eMIMO

Source:	RAN WG1

To:	RAN WG2

Cc:	-

Contact Person:

Name:	                 	Min Zhang

E-mail Address:	Min.zhang1 at Huawei.com

1. Overall Description:

RAN1 has discussed multi-TRP/panel transmission enhancement in Rel 16 to support different scenarios including TRPs connected with ideal and non-ideal backhaul, TRPs from intra-cell (same cell ID) and inter-cell (different cell ID), and also TRPs based eMBB and URLLLC. So far RAN1 has made the following agreements which may have RAN2 specification impact.

For multi-TRP/panel transmission, both multiple PDCCH and single PDCCH designs are supported in Rel-16

- Applies for eMBB

For multiple PDCCH based multi-TRP/panel transmission,

- The total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to 2 and also the total number of MIMO layers of scheduled PDSCHs is up to reported UE MIMO capability, if resource allocation of PDSCHs are overlapped.

- To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs. One CORESET in a “PDCCH-config” corresponds to one TRP.

- Separate ACK/NACK payload/feedback for received PDSCHs is supported. For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used, PUCCH resources conveying ACK/NACK feedback can be TDM with separated HARQ-ACK codebook.

For single PDCCH based multi-TRP/panel transmission,

- TCI indication framework shall be enhanced in Rel-16 at least for eMBB. Each TCI code point in a DCI can correspond to 1 or 2 TCI states. When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1 and FFS design for DMRS type 2. MAC-CE can be enhanced to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2.

2. Actions:

RAN1 respectfully asks RAN2 to take into account above RAN1 agreements for future RAN2 work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #96bis	08 – 12 April 2019, Xian, China

TSG RAN WG1 Meeting #97	13 – 17 May 2019,  Reno, US