3GPP TSG RAN WG2#106	R2-1905505

Reno, Nevada, USA, 13 – 17 May, 2019

3GPP TSG RAN WG1 #96bis		R1-1905577

Xi’an, China, 8th – 12th April, 2019

Title:	LS on FD-MIMO signalling capabilities

Release:	Rel-13

Work Item:	LTE_EBF_FDMIMO-Core

Source:	RAN1

To:	RAN2

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

Attachments:	None

# 1. Overall Description:

RAN1 reached the following agreement on FD-MIMO capabilities:

Agreement:

Adopt the following baseband capability signalling for Rel-13 FD-MIMO:

For a UE configured with FD-MIMO, for a band combination for which the UE does not report FD-MIMO capabilities, a configuration related to a set of CCs is supported by the UE if the following inequality is met

i ∈ configured CCswi∙li≤y

where

-  li is the maximum number of DL layers configured for CC i

- wi=x0,  if CC i is configured with FD-MIMO and li=2x1,  if CC i is configured with FD-MIMO and li=4x2,  if CC i is configured with FD-MIMO and li=81,   if CC i  is not configured with FD-MIMO

- y is the “total number of weighted layers” the UE can process

Notes:

- Legacy capability signalling can still be used after this capability is introduced (to e.g. signal a subset of supported band combinations).

- Detailed design of the capability signalling is left to RAN2

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to introduce following parameters in UE capability signalling (per UE):

- xi (i=0,1,2): Relative weight of processing FD-MIMO with {2, 4, 8} layers with respect to non-FD-MIMO with the same number of layers. xi  has a range of {1, 1.25, 1.5, 1.75, 2, 2.5, 3, 4}

- y: “Total number of weighted layers”, which is an integer between 2 and 128.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #97	13 - 17 May 2019	Reno, USA

TSG RAN WG1 Meeting #98	26 - 30 Aug 2019	Prague, CZ