3GPP TSG-RAN WG2 Meeting #106	R2-1905824

Reno, USA, 13th –17th May 2019	(Resubmission of R2-1903337)

Source:	vivo

Title:	Configuration of INACTIVE state for MR-DC

Agenda item:	11.10.4.3

Document for:	Discussion and Decision

1	Introduction

As described in [1], one of the objectives of this WI is:

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

The feasibility of coexistence between RRC_INACTIVE and DC has been discussed and confirmed [2][3]. At RAN2#103bis meeting, RAN2 has further discussed MR-DC configuration in INACTIVE [4]. RAN2 agreed as follows:

Agreements

1:	For NE-DC, NGEN-DC, and NR-DC, the UE stores the PDCP/SDAP configuration when moving to INACTIVE.  When resuming, the UE applies the stored PDCP/SDAP configuration as in NR SA/LTE.

2	A UE releases its lower-layer SCG configuration in RRC_INACTIVE

3	RAN2 understand that discussion on keeping the lower layer SCG configuration in Inactive would fit within the existing scope of the DC/CA enhancements WI

4	RAN2 understand that before suspend or in the resume message, the network should reconfigure any SCG only bearers so that at completion of the resume the UE is not left with any DRB without any RLC bearer (may not need to be captured in the specification)

As highlighted above, RAN2 agreed that for rel-15, UE releases its lower-layer SCG configuration in RRC_INACTIVE. But for the Rel-16 DC/CA enhancements WI, RAN2 hints to the necessity of UE keeping the lower layer SCG configuration in INACTIVE.

Further during euCA WI, it has been discussed whether to keep SCell configuration upon Resume procedure, but because of time limit, the discussion could not be carried on.

This contribution further discusses configuration of INACTIVE state for DC/CA enhancement and data arrival in case of INACTIVE state of MR-DC.

2	Discussion

  - CA configuration on Resume

    - Maintaining SCell configuration

According to TS 38.331 [5], CA resumption is not supported in NR Release 15, hence SCell configuration will be released upon initiation of the RRC resume procedure as follows.

***********************************From 38.331********************************************

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

[some text omitted]

- release the MCG SCell(s) from the UE Inactive AS context, if stored;

[some text omitted]

***********************************From 38.331********************************************

Observation 1: MCG SCell configuration is released upon initiation of the RRC resume procedure in NR R15.

In RAN2#102 meeting, whether to support RRC suspend and resume CA establishment was discussed, and the discussion history can be tracked down as follows.

R2-1808165	RRC Suspend Resume CA establishment	Ericsson	discussion	Rel-15	LTE_euCA-Core

=>	Noted

R2-1808166	Text proposal RRC Suspend Resume CA establishment	Ericsson	discussion	Rel-15	LTE_euCA-Core

- CB on Friday:=>	Indicate whether it should be supported or not and will not spend online for technical discussion on Friday. (Offline#299, R2-1808810, Ericsson)

R2-1808166	Text proposal RRC Suspend Resume CA establishment	Ericsson	discussion	Rel-15	LTE_euCA-Core

=>	Postponed to next meeting and should as TEI and not euCA WI

Some companies pointed out to the necessity of having SCell configuration kept upon resume would allow to directly configure SCells to be activate or dormant at RRC resume and may further reduce the CA establishment delay. The discussion on this topic has been postponed due to time limit for euCA WI.

The main motivation to support RRC suspend and resume CA establishment is to activate the stored SCell configuration directly as soon as the RRC connection is resumed. Therefore, enhancing RRC suspend and resume procedure for CA configuration is beneficial in reducing latency and signalling overhead.

Observation 2: Support of RRC suspend and resume CA establishment was discussed in LTE euCA WI but postponed due time limit.

    - SCell measurement for resuming

During LTE euCA WI, it is proposed [6] to enhance the RRCConnectionResume message, by adding an indication of which state, active, dormant or deactivated, to configure SCells into at RRC resume. Absence of this indication means the SCell should not be configured at RRC resume. But, the network may probably not be able to get full knowledge of whether the configured SCell quality is good enough, before sending RRC resume message, due to Msg3 size limitation. Thus the indication in RRC resume message may be blind and not enough to support proper resumption of SCell configuration. The consequence is that, if the INACTIVE UE moves far away from the cell where it is suspended, the configured SCell, via RRC resume message, may fail to be resumed. However, if the INACTIVE UE has available measurement result for the configured SCell(s), it can be considered, as solid basis, that the SCell configuration is proper or not. When the UE receives the RRC resume message, it can judge if the configured SCell quality is above certain threshold for successful resuming of the SCell. Otherwise, the UE will release the dedicated SCell configuration and fallback to work on non-CA mode. The failure of SCell resumption can be indicated to the network via RRC resume complete message. In such case, the network may configure CA to the UE only when the UE report available measurement results after entering RRC Connected.

Observation 3: Without valid measurement result(s) from UE, indication of keep/release MCG SCell configuration in RRC resume message may be set blindly by the NW and leads to failure of SCell configuration.

Proposal 1: MCG SCell configuration will be maintained upon initiation of the RRC resume procedure.

Proposal 2: The UE restores the SCell configuration as far as the measurement result(s) of SCell configured in RRC resume message is valid when resuming to RRC_CONNECTED.

  - DC configuration

    - Maintaining low layer configuration

One of the motivations for maintaining the SCG configuration in RRC_INACTIVE as described in [7] is:

- The state and "endpoints" (in case of Dual Connectivity configuration) of the N2 and N3 reference points are not changed by the UE entering CM-CONNECTED with RRC Inactive state. A UE in RRC inactive state is aware of the RAN Notification area.

According to RAN2 to agreement, UE releases its lower-layer SCG configuration in RRC_INACTIVE, i.e. in RRC_INACTIVE UE will release low layer configuration of:

- MN-terminated SGC bearer

- MN-terminated split bearer

- SN-terminated SCG bearer

- SN-terminated split bearer

Observation 4: In case of DC, UE releases its lower-layer SCG configuration in RRC_INACTIVE in NR R15.

Similar to discussion on CA topic, maintaining lower-layer SCG configuration in RRC Inactive can give opportunity of resuming DC upon entering RRC Connected, which will bring benefit of signalling overhead reduction and enable delta configuration via RRC resume message.

Therefore,

Proposal 3: The whole SCG configuration is stored as part of the Inactive UE AS Context and not released upon initiation of the resume procedure.

    - SCG measurement for resuming

Given UE mobility, the available SCG measurement can also be taken into account to further decide whether the stored SCG configuration can be reconfigured or not. When the UE receives the RRC resume message, it judges if the configured SCG cell quality is above certain threshold based on SCG measurement in RRC Inactive, then the SCG configuration can be resumed by the UE successfully. Otherwise, the UE will release the dedicated SCG configuration and fallback to work on non-DC mode. The failure of SCG resumption can be indicated to the network via RRC resume complete message. In such case, the network may configure DC to the UE only when the UE report available SCG cell measurement result after entering RRC Connected.

Therefore,

Proposal 4:  The UE restores the SCG configuration as far as the measurement result(s) of SCG serving cell configured in RRC resume message is valid when resuming to RRC_CONNECTED.

3	Conclusion

This contribution discusses remaining issues related to CA and DC in RRC resume procedure. The contribution concludes with the following observations and proposals:

Observation 1: MCG SCell configuration is released upon initiation of the RRC resume procedure in NR R15.

Observation 2: Support of RRC suspend and resume CA establishment was discussed in LTE euCA WI but postponed due time limit.

Observation 3: Without valid measurement result(s) from UE, indication of keep/release MCG SCell configuration in RRC resume message may be set blindly by the NW and leads to failure of SCell configuration.

Observation 4: In case of DC, UE releases its lower-layer SCG configuration in RRC_INACTIVE in NR R15.

Proposal 1: MCG SCell configuration will be maintained upon initiation of the RRC resume procedure.

Proposal 2: The UE restores the SCell configuration as far as the measurement result(s) of SCell configured in RRC resume message is valid when resuming to RRC_CONNECTED.

Proposal 3: The whole SCG configuration is stored as part of the Inactive UE AS Context and not released upon initiation of the resume procedure.

Proposal 4:  The UE restores the SCG configuration as far as the measurement result(s) of SCG serving cell configured in RRC resume message is valid when resuming to RRC_CONNECTED.

4	References

- RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.

- R2-1806651, Reply LS on coexistence between RRC inactive and dual connectivity (Source: SA2).

- R3-181557, Reply LS on coexistence between RRC inactive and dual connectivity.

- R2-181402, Summary of [103#50][NR late drop] - MR-DC configuration in INACTIVE (Interdigital).

- 3GPP TS 38.331 V15.4.0 (2018-12).

- R2-1808165, RRC Suspend Resume CA establishment, Ericsson.

- S2-184343, Coexistence of RRC Inactive and Dual Connectivity, Qualcomm Incorporated.