3GPP TSG-RAN WG2 Meeting #106    		        R2-1905937

Reno, NV, USA, 12th – 16th May 2019                         Revision of R2-1904329

Source: 	CMCC

Title:	Discussion on power saving in inter-frequency measurements

Agenda item:	11.11.5

Document for:	Discussion & Decision

- Introduction

At RAN2 #105bis meeting, the agreements about power consumption reduction in RRM measurements was approved as the following [1],

Agreements: - RAN2 evaluates RRM measurement relaxation in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE. - RAN2 to evaluate relaxed monitoring criteria under which the UE may relax RRM measurements. The relaxed monitoring criteria may include the following aspects, but are not limited to: • UE mobility status (e.g. serving cell variation, speed, movement, direction, cell (re-selection, UE type …) • Link quality (e.g. serving cell threshold/quality, position in cell, …) • Serving cell beam status (e.g. beam change, direction, beam specific link condition…) - Serving cell measurements are not excluded from the evaluation of relaxed RRM measurements for now.

In this contribution, power consumption reduction in inter-frequency RRM measurement will be discussed.

- Reduce number of intra-band measured frequencies in inter-frequency measurement

In this section, the reduction of number of measured frequencies in inter-frequency RRM measurements to reduce UE power consumption will be discussed.

## 2.1 Measurement relaxation criteria

In NR, RRM measurements are performed in RRC_IDLE/RRC_INACTIVE for cell selection and reselection and in RRC_CONNECTED for handover. However, the current RRM measurements configuration and procedures can be further optimized for some scenarios, e.g., for inter-frequency RRM measurement with some intra-band frequencies co-site deployed, current Rel-15 procedure in this case will cause some unnecessary measurements and increases the UE power consumption.

Specifically, for the intra-band carriers deployed on the same site, the propagation characteristics is similar, and it is very likely for UE to get almost the same measurement results, e.g., at least for RSRP. In current Rel-15 inter-frequency RRM measurement framework, if all the carriers in the same band are informed by SIB to UE for measurement, UE has to measure all the carriers and chooses the strongest cell to camp on or handover, even the RSRP measurement results of various intra-band carriers are almost the same. Therefore, in this case, measurements on all the intra-band frequencies are meaningless to UE and this will increase UE power consumption, the number of frequency layers in this case could be reduced without any impact on the system performance. In addition, there may be a UE capability limitation on the number inter-frequency measurements, if UE can reduce the number of measured frequency layers in the same band in this case, UE will have more chance to measure other frequencies in other bands.

Therefore, in the scenario which multiple intra-band frequencies are co-site deployed, the number of measured frequencies in inter-frequency RRM measurements can be reduced to reduce the UE power consumption.

Proposal 1. For co-site deployed intra-band frequencies criteria, the number of measured frequencies can be reduced in inter-frequency RRM measurement relaxation.

## 2.2 Relaxation procedure

As discussed in the above section, the number of intra-band measured frequencies can be reduced in inter-frequency measurements.

In RRC_CONNECTED state, gNB can configure UE only some of the intra-band frequencies to be measured through dedicated RRC signalling to reduce the UE power consumption. However, in RRC_IDLE, the inter-frequency measurement configuration is carried in system information, and gNB cannot only broadcast some of the measured frequencies.

One way to realize reducing the measured frequency layers is that gNB can configure UE the relationship between different intra-band measured frequencies or measured frequency groups. UE can random selects only one frequency from the associated frequencies or in the same frequency group to measure, and can assume the measurement results, e.g., RSRP, of the frequencies in the same group are the same. The configuration of measured frequency groups or the association relationships can be indicated in measurement configuration information such as system information or measurement object for UEs in RRC_IDLE and RRC_CONNECTED respectively.

For example, there are three co-site deployed intra-band frequencies A, B and C with the same priority. In order to reduce the power consumption on measuring all the frequency layers, gNB could configure UE the association among three frequencies. UE can random select one of the frequency layers such as frequency B to measure, and then UE selects the strongest cell to camp on or handover. In this way, UE can avoid measuring some unnecessary frequencies to realize power saving.

In addition, in current measurement procedure, whether RSRP or RSRQ should be used for cell reselection or handover depends on the network configuration, this inter-frequency measurement relaxation method is suitable at least for the case that RSRP is used for cell reselection and handover. For the case that RSRQ is used for cell reselection and handover, anyway network can control whether to use this inter-frequency measurement relaxation or not based on the practical network conditions.

Proposal 2. For inter-frequency RRM measurement reduction, the measured frequency group and association relationship between the measured frequencies can be indicated to UE in system information or measurement configuration information, and UE can randomly select one of the frequency layers in the same group to perform RRM measurement.

## 2.3 Impact on mobility and measurement accuracy

Another issue to be discussed is the impact of relaxation of inter-frequency measurements on mobility and measurement accuracy required by RAN4.

In this inter-frequency measurement relaxation scheme, it is up to gNB to control whether the frequency layers can be reduced. Besides, UE can random select one frequency layer to measure under the condition that RSRP measurement results on different frequency layers are almost the same and UE will select the strongest cell to camp on or handover on the measured frequency layer. That is, the relaxation of inter-frequency measurements will have no impact on mobility performance.

For measurement accuracy, since the measurement samples and periods are the same as the Rel-15 RRM measurements, it is expected that there is no impact on the measurement accuracy required by RAN4. For information, the same idea and design principle have already been adopted in NR, e.g., in R4-1801307 [3], RAN4 agreed that NR supports to configure enabling/disabling intra-frequency measurement on each NR SCell frequency layer considering that, for two serving cell frequency layers within the same band, the intra-frequency measurement results on each serving cell frequency layer could be almost same.

Proposal 3. The relaxation of inter-frequency measurements will have no impact on mobility and measurement accuracy.

## 2.4 Evaluation results

In this section, the evaluation results is provided considering the case that reducing 2 measured frequency layers to 1 frequency layer in RRC_IDLE state. The power model and simulation parameters are according to TR 38.840 [2] and summarized in Table 1. UE power consumption in RRC_IDLE may contain several sections: monitoring Paging PDCCH power, receive Paging PDSCH power, Idle power, switching power, cell search power and measurement power. We assume that the paging rate is 10% and cell search is performed every three measurements in this simulation.

Table 1 Simulation parameters

| Frequency Range | FR1 |
| --- | --- |
| SCS | 30kHz |
| DRX cycle | 1280 ms |
| SMTC | 5ms |
| Pre-sync duration | 1 slot |
| Paging duration | 1 slot |
| Switching time | 1 slot |
| Idle power | 1 |
| Pre-sync power | 100 |
| Paging rate | 10% |
| Cell search power | 150 |
| Cell search + measure power | 8 neighbor cell: 200 (sync) |
| Measure power | 8 neighbor cell: 150 (sync) |
| Switching power | 45 |

The evaluation results are provided in Table 2.

Table 2. Simulation results

| Case | 2 frequency layers to be measured | 1 frequency layer to be measured |
| --- | --- | --- |
| Power unit per slot | 2.43 | 1.77 |
| Power saving gain | baseline | 27.2% |

From the evaluation results, we can see that if the measured frequency layer can be reduced from 2 layers to 1 layer, the power saving gain will be about 27.2%. If the reducing ratio increases such as only 1/3 frequency layers need to be measured or the total number of frequency layers increase such as reducing 6 frequency layers to 3 frequency layers, the power saving gain will also increase.

Observation 1. The power saving gain is about 27.2% if the number of measured frequency layers reduced from 2 to 1.

- Conclusions

In this contribution, for inter-frequency measurement, relaxing the number of intra-band measured frequencies is discussed, and the following proposals are made.

Proposal 1. For co-site deployed intra-band frequencies criteria, the number of measured frequencies can be reduced in inter-frequency RRM measurement relaxation.

Proposal 2. For inter-frequency RRM measurement reduction, the measured frequency group and association relationship between the measured frequencies can be indicated to UE in system information or measurement configuration information, and UE can randomly select one of the frequency layers in the same group to perform RRM measurement.

Proposal 3. The relaxation of inter-frequency measurements will have no impact on mobility and measurement accuracy.

Observation 1. The power saving gain is about 27.2% if the number of measured frequency layers reduced from 2 to 1.

- References

[1]	Report of 3GPP TSG RAN WG2 meeting #105bis, April, 2019

[2] 3GPP TR38.840, “NR; Study on UE Power Saving (Release 16)”

[3] R4-1801307, LS for intra-frequency measurement on NR Scell, NTTDOCOMO

- Appendix

Table 18: UE power consumption model for FR1 [2]

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

#### 8.1.4.	UE power consumption model for RRM measurements [2]

For inter-frequency measurement:

The inter-frequency measurement is represented as function of the number of frequency layers:

UE needs to monitor Nf frequency layers within a measurement gap.

E3=i=0Nf-1Ei+Et∙Nf+1

Where

- Ei is either Pfr1*Ns (for FR1) or Pfr2*Ns (for FR2) for each frequency layer i

  - Ns is the number of slot over which measurements (for each frequency layer i) are carried out

- Nf is the number of frequency layers measured

- Et = Pt * Tt, where

  - Pt is the switching power consumption

    - Assume micro sleep power for Pt

  - Tt is 0.5ms for FR1 and 0.25ms for FR2 (from RAN4)

It can be simplified to the following if Ei is same across frequency layers (i.e. Ei = E for all i).

E3 = E*Nf + Et*(Nf+1)

Note: RAN4 requirement assumes only one frequency layer per measurement gap.