3GPP TSG-RAN WG2 Meeting #106                              	　　　　　　　　    　　　　　　R2-1906085

Reno, USA, 13rd –17th May 2019

Source: 	KDDI Corporation

Title:	Consideration on BAP-layer routing tables

Tdoc Type:	Discussion

Agenda Item:	11.1.3 Adapt function

Document for:	Discussion and decision

Release:	Rel-16

Introduction

RAN2 discussed “routing” and “bearer mapping” via email [1] [2]. However how the IAB routing works in whole picture is still unclear. So, in this contribution, we try to clarify the whole routing mechanism, specifically we discuss and try to clarify the following aspects.

- Routing table

- Routing configuration

- Adaptation layer header

Discussion

  - Routing table

In IAB system, the following two types of routing table should be considered, a routing table owned by the donor-CU, and the other is a routing table owned by each IAB node. Each routing table contains information for both Uplink and Downlink.

- The donor-CU routing table

The donor-CU routing table contains all the path information of all the IAB nodes under the IAB donor-CU, specifically information such as, Route ID, Destination ID, and list of the next hop node which indicates a path (travel route). The donor-CU updates its routing table, when a new IAB node connects/disconnects to the donor-CU, when an IAB node creates a new connection to other IAB node within an IAB donor-CU, when an IAB node releases an existing connection.

- The IAB node routing table

The IAB node routing table contains only the path information related to that IAB node. The table contains information such as Route ID, next hop address for Uplink, next hop address for Downlink. Each IAB node updates it table when it receives the route configuration messages from the Donor-CU.

Figure1 Example deployment

We will show how those routing tables (IAB node4 related) looks like in below, using an example case depicted as figure1. With regard to bearer mapping, in the column “bearer mapping to next hop”, (1:1) means 1:1 mapping, (1:N) means 1:N mapping. We list up routing tables related to only downlink, and don’t show the uplink. However, routing tables for uplink just looks like as Downlink.

The donor-CU routing table (Downlink)

| Route ID | Destination address | bearer mapping to next hop | Next hop address 1 | bearer mapping to next hop | Next hop address 2 |
| --- | --- | --- | --- | --- | --- |
| 1 | IAB node4 | -(1:1) | IAB node2 | TEID of Leg2(1:2) | IAB node1 |
| 2 | IAB node4 | -(1:1) | IAB node2 | TEID of Leg3(1:2) | IAB node1 |
| 3 | IAB node4 | TEID of Leg7(1:2) | IAB node3 | -(1:1) | IAB node1 |
| 4 | IAB node4 | TEID of Leg8(1:2) | IAB node3 | -(1:1) | IAB node1 |

The IAB node1 routing table (Downlink)

| Route ID | Destination address | Next hop address 1 | bearer mapping to next hop |
| --- | --- | --- | --- |
| 1 | IAB node4 | IAB node2 | TEID of Leg2(1:2) |
| 2 | IAB node4 | IAB node2 | TEID of Leg3(1:2) |
| 3 | IAB node4 | IAB node3 | -(1:1) |
| 4 | IAB node4 | IAB node3 | -(1:1) |

The IAB node2 routing table (Downlink)

| Route ID | Destination address | Next hop address 1 | bearer mapping to next hop |
| --- | --- | --- | --- |
| 1 | IAB node4 | IAB node4 | -(1:1) |
| 2 | IAB node4 | IAB node4 | -(1:1) |

The IAB node3 routing table (Downlink)

| Route ID | Destination address | Next hop address 1 | bearer mapping to next hop |
| --- | --- | --- | --- |
| 1 | IAB node4 | IAB node4 | TEID of Leg7(1:2) |
| 2 | IAB node4 | IAB node4 | TEID of Leg8(1:2) |

Above the discussion, we make the following proposals on routing table,

Proposal1: For BAP-layer routing, agree to have two types of routing table, one is a routing table owned by the donor-CU, and the other is a routing table owned by each IAB node

Proposal2: Agree that the donor-CU routing table contains all the path information of the all IAB nodes under the IAB donor-CU for both uplink and downlink, and the table includes Route ID, Destination ID, and list of the next hop node which indicates a path (travel route)

Proposal3: Agree that the IAB node routing table contains only the path information which is related to that IAB node, and the table includes Route ID, next hop address for Uplink, next hop address for Downlink

  - Routing configuration

Based on the above routing table discussion, the routing configuration message is used to develop the routing table in each IAB node side. It is proposed

Proposal4: Agree that routing configuration is used to develop the routing table in each IAB node side, and the message includes Route ID, next hop address for Uplink, next hop address for Downlink.

Proposal5: Discuss and decide how to configure routing information, using RRC signaling or using F1-AP message

  - Adaptation layer header

Based on the above routing table and routing configuration discussion, it is proposed

Proposal6: Agree that adaptation layer header includes Route ID and Destination ID, so that the each IAB node can understand the next link/RLC channel based on the routing table.

Conclusion

In this conclusion, we make the following proposals.

Proposal1: For BAP-layer routing, agree to have two types of routing table, one is a routing table owned by the donor-CU, and the other is a routing table owned by each IAB node

Proposal2: Agree that the donor-CU routing table contains all the path information of the all IAB nodes under the IAB donor-CU for both uplink and downlink, and the table includes Route ID, Destination ID, and list of the next hop node which indicates a path (travel route)

Proposal3: Agree that the IAB node routing table contains only the path information which is related to that IAB node, and the table includes Route ID, next hop address for Uplink, next hop address for Downlink

Proposal4: Agree that routing configuration message is used to develop the routing table in each IAB node side, and the message includes Route ID, next hop address for Uplink, next hop address for Downlink.

Proposal5: Discuss and decide how to configure routing information, using RRC signaling or using F1-AP message

Proposal6: Agree that adaptation layer header includes Route ID and Destination ID, so that the each IAB node can understand the next link/RLC channel based on the routing table.

References

[1]  R2-1905063	Summary of email discussion [105#46][IAB] Routing	LG Electronics Inc.

[2]  R2-1903964  Summary of email discussion report of [105#47][IAB] Bearer Mapping 	Ericsson