3GPP TSG-RAN WG2 Meeting #106	R2-1906108

# Reno, USA, May 13 – 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0121 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unsupported procedures with explicit SMTC configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was found unclear which LTE PCell, the source or the target, the UE should use as the timing reference cell for SMTC configuration in the following procedures. - Simultaneous LTE-to-LTE handover and NR PSCell addition using explicit configuration of SMTC in RRCConnectionReconfiguration message - Simultaneous LTE-to-LTE handover and SN change using explicit configuration of SMTC in RRCConnectionReconfiguration message RAN2 agreed the above procedures are not supported in release-15. It should be noted that the network could still use the procedures when the source LTE PCell and the target LTE PCell are SFN and subframe synchronized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that the following procedures for EN-DC are not supported in this version of the protocol; - Inter-Master Node handover with Secondary Node change using explicit SMTC configuration for PSCell in RRCConnectionReconfiguration message, when the source PCell and the target PCell are not SFN/subframe-synchronized - eNB to Master Node change procedure using explicit SMTC configuration for PSCell in RRCConnectionReconfiguration message, when the source PCell and the target PCell are not SFN/subframe-synchronized, is not supported in this version of the protocol. Impact Analysis: Impacted 5G architecture option: (NG)EN-DC Impacted functionality: - Simultaneous LTE-to-LTE handover and NR PSCell addition using explicit configuration of SMTC in RRCConnectionReconfiguration message - Simultaneous LTE-to-LTE handover and SN change using explicit configuration of SMTC in RRCConnectionReconfiguration message Inter-operability: - If the network is implemented according to the CR and the UE is not; there is no inter-operability problem as the network would not use the unsupported procedures. - If the UE is implemented according to the CR and the network is not; the network may use the unsupported procedures, and the corresponding procedure may fail due to the UE using a different SMTC timing from the one intended by the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may use the unsupported procedures, which may be considered misconfiguration by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.1, 10.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.7	Inter-Master Node handover with/without Secondary Node change

### 10.7.1	EN-DC

Inter-Master Node handover with/without MN initiated Secondary Node change is used to transfer context data from a source MN to a target MN while the context at the SN is kept or moved to another SN. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN (or release the SN, as described in section 10.8).

NOTE 1:	Inter-RAT Inter-Master node handover with/without SN change is not supported in this version of the protocol (i.e. no transition from EN-DC to NR-NR DC).

Figure 10.7.1-1: Inter-MN handover with/without MN initiated SN change

Figure 10.7.1-1 shows an example signaling flow for inter-Master Node handover with or without MN initiated Secondary Node change:

NOTE 2:	For an inter-Master Node handover without Secondary Node change, the source SN and the target SN shown in Figure 10.7.1-1 are the same node.

1.	The source MN starts the handover procedure by initiating the X2 Handover Preparation procedure including both MCG and SCG configuration. The source MN includes the (source) SN UE X2AP ID, SN ID and the UE context in the (source) SN in the Handover Request message.

NOTE 3:	The source MN may send the SgNB Modification Request message (to the source SN) to request the current SCG configuration before step 1.

2.	If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SgNB Addition Request to the target SN including the UE context in the source SN that was established by the source MN.

3.	The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

4.	The target MN includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an RRC message to perform the handover, and may also provide forwarding addresses to the source MN. The target MN indicates to the source MN that the UE context in the SN is kept if the target MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5.	The source MN sends SN Release Request to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. If the indication as the UE context kept in SN is included, the SN keeps the UE context.

6.	The source MN triggers the UE to apply the new configuration.

NOTE x:	Inter-Master Node handover with Secondary Node change using explicit SMTC configuration for PSCell in RRCConnectionReconfiguration message, when the source PCell and the target PCell are not SFN/subframe-synchronized, is not supported in this version of the protocol.

7/8.	The UE synchronizes to the target MN and replies with RRCConnectionReconfigurationComplete message.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SgNB Reconfiguration Complete message.

11a.	The SN sends the Secondary RAT Data Volume Report message to the source MN and includes the data volumes delivered to the UE over the NR radio for the related E-RABs.

NOTE 4:	The order the source SN sends the Secondary RAT Data Volume Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

11b.	The source MN sends the Secondary RAT Report message to MME to provide information on the used NR resource.

12.	For bearers using RLC AM, the source MN sends the SN Status transfer to the target MN.

13.	Data forwarding from the source MN takes place. If the SN is kept, data forwarding may be omitted for SCG bearers and SCG split bearers.

14-17.	The target MN initiates the S1 Path Switch procedure.

NOTE 5:	If new UL TEIDs of the S-GW are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

18.	The target MN initiates the UE Context Release procedure towards the source MN.

19.	Upon reception of the UE Context Release message, the (source) SN can release C-plane related resource associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the indication was included in the SN Release Request in step 5.

[…]

## 10.9	eNB/gNB to Master Node change

### 10.9.1	EN-DC

The eNB to Master Node change procedure is used to transfer context data from a source eNB to a target MN that adds an SN during the handover.

Figure 10.9.1-1: eNB to Master Node change

Figure 10.9.1-1 shows an example signaling flow for eNB to Master Node change:

1.	The source eNB starts the handover procedure by initiating the X2 Handover Preparation procedure.

2.	The target MN sends SgNB Addition Request to the target SN.

3.	The target SN replies with SgNB Addition Request Acknowledge. If data forwarding is needed, the target SN provides forwarding addresses to the target MN.

4.	The target MN includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an E-UTRA RRC message, including a NR RRC configuration message which also includes the SCG configuration, to perform the handover, and may also provide forwarding addresses to the source eNB.

5.	The source eNB triggers the UE to apply the new configuration.

NOTE x:	eNB to Master Node change procedure using explicit SMTC configuration for PSCell in RRCConnectionReconfiguration message, when the source PCell and the target PCell are not SFN/subframe-synchronized, is not supported in this version of the protocol.

6/7.	The UE synchronizes to the target MN and replies with RRCConnectionReconfigurationComplete message.

8.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the target SN.

9.	If the RRC connection reconfiguration procedure was successful, the target MN informs the target SN.

10.	For bearers using RLC AM, the source eNB sends the SN Status transfer to the target MN.

11.	Data forwarding from the source eNB takes place.

12-15.	The target MN initiates the S1 Path Switch procedure.

NOTE:	If new UL TEIDs of the S-GW are included, the target MN performs MN initiated SN Modification procedure to provide them to the target SN.

16.	The target MN initiates the UE Context Release procedure towards the source eNB.