# 3GPP TSG-RAN WG2 Meeting #106 	R2-1906109

Reno, Nevada USA, 13-17 May 2019

Agenda item:		11.15

Source: 		Qualcomm Incorporated

Title: 		CLI-RSSI and SRS-RSRP Measurement

Document for:	Agreement

# Introduction

RAN2 was requested by RAN1 in LS R2-1905203 to define CLI-RSSI and SRS-RSRP measurements. This paper analyzes the configuration, measurement and reporting procedures.

# RAN1 agreements

## On CLI-RSSI Measurement

- Agreements on resource configuration for CLI-RSSI measurement: - In information element of resource configuration for CLI-RSSI measurement, at least following parameters are included - Number of PRBs and start PRB index for subband indication - Note: Configured PRBs are contiguous. - Note: The exact number of PRBs is up to RAN4. - Number of OFDM symbols and first OFDM symbol index in a slot - Note: Configured OFDM symbols are contiguous. - Note: The exact number of OFDM symbol is up to RAN4. - Note: Depending on UE capability discussions, UE may not be required to assume that PDSCH is FDMed with CLI-RSSI measurement resource(s) - In information element of resource configuration for CLI-RSSI measurement, following values for slot configuration is used. - slots10 INTEGER (0..9), slots20 INTEGER (0..19), slots40 INTEGER (0..39), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639)} - Note: Network configures slot duration for CLI-RSSI measurement which is corresponded with one value of periodicity among the periodicity set (i.e. {10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms}) Table. CLI-RSSI measurement periodicity SCS(kHz) Slot duration; 15k Hz; 30 kHz; 60 kHz; 120 kHz. 10 slots; 10 msec. 20 slots; 20 msec; 10 msec. 40 slots; 40 msec; 20 msec; 10 msec. 80 slots; 80 msec; 40 msec; 20 msec; 10 msec. 160 slots; 160 msec; 80 msec; 40 msec; 20 msec. 320 slots; 320 msec; 160 msec; 80 msec; 40 msec. 640 slots; 640 msec; 320 msec; 160 msec; 80 msec. - In information element of resource configuration for CLI-RSSI measurement, reference subcarrier spacing for CLI-RSSI measurement is included. - All subcarrier spacings supported in Rel-15 NR for FR1, FR2 - Subcarrier spacing is a reference unit of time/frequency resource configuration. - UE operates CLI measurement within the active BWP. - The subcarrier spacing for CLI-measurement resource configuration can be same or different from the SCS of the active BWP. - One or multiple resources for CLI-RSSI measurement can be configured - Maximum number of measurement resource for CLI-RSSI measurement is 64.

## On SRS-RSRP measurement

- Agreements and conclusion on UE behaviour for SRS-RSRP: - In order to perform SRS transmission for CLI measurement, The TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols transmitted to the gNB. - For SRS-RSRP measurement, the UE is not required to perform time tracking or time adjustment other than a constant offset relative to its own DL timing. - The constant offset is derived by UE implementation. - Agreement on SRS transmission: - For SRS transmission for SRS-RSRP measurement purpose, no new SRS resource set usage is introduced - Agreement on SRS resource for SRS-RSRP measurement: - For SRS-RSRP measurement, one or more SRS resource per serving cell can be configured - For SRS-RSRP, the total number of SRS resources to be monitored by a UE should not exceed 32.

# Measurement configuration

## Measurement Objects

Current MeasObjectNR is for SSB or CSI-RS based measurements, which are quite different from CLI-RSSI and SRS-RSRP measurement. The existing measurement objects are for UE to measure DL signal. The CLI measurement is for UE to measure UL signal.

CLI measurements are fundamentally different as they are done on UL signals whereas existing measurements are on DL signal.

It is cleaner to define a new measurement object instead of extending existing measurement objects.

Define a new measurement object MeasObjectCLI for CLI-RSSI and SRS-RSRP measurements.

Include CLI-RSSI and SRS-RSRP measurement resource configuration in MeasObjectCLI.

Include L3 filtering configurations for CLI-RSSI and SRS-RSRP measurements in the corresponding measurement configurations.

Example Text Proposal is given below:

MeasObjectCLI ::= SEQUENCE { cli-RSSI-Measurement CLI-RSSI-Measurement OPTIONAL, srs-RSRP-Measurement SRS-RSRP-Measurement OPTIONAL, ... } #### – CLI-RSSI-Measurement The IE CLI-RSSI-Measurement is used to configure CLI-RSSI measurements. CLI-RSSI-Measurement information element -- ASN1START -- TAG-CLI-RSSI-MEASUREMENT-START CLI-RSSI-Measurement ::= SEQUENCE { <<fields TBD based on RAN1 LS>>, filterCoefficientCLI-RSSI FilterCoefficient, ... } -- TAG-CLI-RSSI-MEASUREMENT-STOP -- ASN1STOP #### – SRS-RSRP-Measurement The IE SRS-RSRP-Measurement is used to configure SRS-RSRP measurements. SRS-RSRP-Measurement information element -- ASN1START -- TAG-SRS-RSRP-MEASUREMENT-START SRS-RSRP-Measurement ::= SEQUENCE { <<fields TBD based on RAN1 LS>>, filterCoefficientSRS-RSRP FilterCoefficient, ... } -- TAG-SRS-RSRP-MEASUREMENT-STOP -- ASN1STOP

For better measurement efficiency and accuracy, it should be possible to configure all UEs served by a cell to transmit the same SRS at the same time. Then, it is necessary for neighbour cells to know the SRS measurement window.

It is preferable that the measurement resource configuration is synchronized between infra-frequency neighbour cells. CU/gNB should get its member cells synchronized in the CLI measurement resource. OAM should be responsible to synchronize the measurement configuration of its managed gNBs/CUs.

If neighbour cells are not synchronized for the SRS configuration, the gNB configures CLI-RSSI/SRS-RSRP measurement for serving UEs by putting the neighbour cells’ CLI-RSSI/SRS-RSRP measurement resources as a list into the SRS-RSRP measurement resource configuration.

Coordination of the CLI-RSSI and SRS-RSRP measurement resources between intra-frequency neighbours is beneficial. This may need RAN3 work.

From RAN2’s point of view, coordinating the CLI-RSSI and SRS-RSRP measurement resources between intra-frequency neighbours is beneficial.

## Reporting triggers

Both periodic and event triggered measurement reporting are beneficial for CLI measurements. The gNB may take interference avoidance actions when high cross link interference is detected, for example: gNB may

- adjust the TDD UL/DL configuration

- avoid scheduling on the interfering slots

- inform the other gNB to mitigate the interference.

Both periodic and event triggered measurement reporting are beneficial for CLI measurements.

Both periodical and event triggered reporting are supported for CLI-RSSI and SRS-RSRP measurements.

For event triggered reporting, new events need to be defined because the existing events and thresholds are not applicable.

Define two new events for CLI measurements – 1) L1: CLI-RSSI or SRS-RSRP is above threshold; 2) L2: CLI-RSSI and SRS-RSRP are below threshold.

Include CLI-RSSI threshold, SRS-RSRP threshold, report on leave, hysteresis and time to trigger as configurations for both events.

Reuse reportInterval and reportAmount for both types of reporting triggers.

CLI measurement may not need UE to measure and report both CLI-RSSI and SRS-RSRP. gNB shall be able to indicate to UE which ones to measure. For event triggered, there are two options. One is implicit upon configuration of the corresponding thresholds for events L1 and L2 (i.e., if threshold is configured, then corresponding measurement is reported). Another option is explicit configuration to indicate which quantity to measure and report. The scenario that an event is triggered based on both measured quantities but only one of them is to be reported is unlikely, therefore for event triggered reporting, the configuration of what is to be measured can be implicit.

For periodic measurements, an explicit configuration is required as there are no corresponding thresholds to be configured.

Define CLI-MeasQuantity to indicate which quantity (CLI-RSSI, SRS-RSRP) to measure and report.

A text proposal capturing the above proposals is given below:

ReportConfigNR information element -- ASN1START -- TAG-REPORTCONFIGNR-START ReportConfigNR ::= SEQUENCE { reportType CHOICE { periodical PeriodicalReportConfig, eventTriggered EventTriggerConfig, ..., reportCGI ReportCGI, [[ cli-Periodical CLI-PeriodicalReportConfig, cli-EventTriggered CLI-EventTriggerConfig ]] } } ReportCGI ::= SEQUENCE { cellForWhichToReportCGI PhysCellId, ... } EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventA1 SEQUENCE { a1-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA2 SEQUENCE { a2-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA3 SEQUENCE { a3-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA4 SEQUENCE { a4-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA5 SEQUENCE { a5-Threshold1 MeasTriggerQuantity, a5-Threshold2 MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA6 SEQUENCE { a6-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, ... }, rsType NR-RS-Type, reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), reportQuantityRS-Indexes MeasReportQuantity OPTIONAL, -- Need R maxNrofRS-IndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, -- Need R includeBeamMeasurements BOOLEAN, reportAddNeighMeas ENUMERATED {setup} OPTIONAL, -- Need R ... } CLI-EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventL1 SEQUENCE { l1-CLI-RSSI-Threshold RSSI-Range OPTIONAL, -- Value range TBC by RAN4 l1-SRS-RSRP-Threshold RSRP-Range OPTIONAL, -- Value range TBC by RAN4 reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventL2 SEQUENCE { l2-CLI-RSSI-Threshold RSSI-Range OPTIONAL, -- Value range TBC by RAN4 l2-SRS-RSRP-Threshold RSRP-Range OPTIONAL, -- Value range TBC by RAN4 reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, ..., }, reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, cli-MeasReportQuantity CLI-MeasReportQuantity, ..., } PeriodicalReportConfig ::= SEQUENCE { rsType NR-RS-Type, reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), reportQuantityRS-Indexes MeasReportQuantity OPTIONAL, -- Need R maxNrofRS-IndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, -- Need R includeBeamMeasurements BOOLEAN, useWhiteCellList BOOLEAN, ... } CLI-PeriodicalReportConfig ::= SEQUENCE { reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, cli-MeasReportQuantity CLI-MeasReportQuantity, ... } CLI-MeasReportQuantity ::= SEQUENCE { cli-RSSI BOOLEAN, srs-RSRP BOOLEAN } NR-RS-Type ::= ENUMERATED {ssb, csi-rs} MeasTriggerQuantity ::= CHOICE { rsrp RSRP-Range, rsrq RSRQ-Range, sinr SINR-Range } MeasTriggerQuantityOffset ::= CHOICE { rsrp INTEGER (-30..30), rsrq INTEGER (-30..30), sinr INTEGER (-30..30) } MeasReportQuantity ::= SEQUENCE { rsrp BOOLEAN, rsrq BOOLEAN, sinr BOOLEAN } -- TAG-REPORTCONFIGNR-STOP -- ASN1STOP

Adopt the TP in section 3 as baseline.

# Performing the measurement

For intra-frequency measurement, a gap can be either defined as the regular L3 measurement gap or realized by symbol level rate matching even if L3 measurement gap is not configured. In particular, by symbol level rate matching, PDSCH can be blanked in the symbols for rate matching so that there is no FDM reception with different timing of PDSCH and CLI-RSSI/SRS-RSRP. However, this does not preclude the network transmitting PDCCH and other DL channels to the UE and therefore requires the UE to do FDM reception of these DL channels with CLI. Therefore, the safest way for UE implementation is to allow L3 measurement gap for CLI-RSSI and SRS-RSRP measurement.

Define intra-frequency measurement gap for CLI-RSSI and SRS-RSRP measurement.

It is possible the CLI-RSSI and SRS-RSRP measurement period is outside of DRX activity period. It should be up to UE implementation whether to measure outside of the DRX activity period, as long as RAN4 defined performance requirement can be met.

It is up to UE implementation whether to measure CLI-RSSI and/or SRS-RSRP during off period of DRX cycle (i.e., other than Active Time), as long as performance requirements (to be defined by RAN4) can be met.

# Measurement Report

CLI measurement result is quite different from the MeasResultNR. We should define a new IE for CLI measurement reporting.

Add CLI measurement measResultCLI into MeasResults. CLI measurement is reported together with serving cell measurement result.

In a network with dynamic TDD configuration, strong CLI may occur to a UE if it is close to multiple UEs from another cell that have strong UL transmissions colliding with its DL reception. In this case, the strong interference power may push the UE analog circuit to saturation. For the UE that is configured to measure the SRS-RSRP or the CLI-RSSI, in such case either the measurement is not accurate, or the UE is not even able to detect the SRS sequence ID.

If the interference is too strong, either the measurement is not accurate, or the UE is not even able to detect the SRS sequence ID.

Therefore, the UE should be allowed to transmit an overload flag in the CLI RSRP/RSSI report to indicate that the interference is too strong to measure.

Include ‘too strong signal to measure’ as one of the possible reported values of CLI-RSSI and SRS-RSRP measurement result.

Below is an example ASN.1.

MeasResults ::= SEQUENCE { measId MeasId, measResultServingMOList MeasResultServMOList, measResultNeighCells CHOICE { measResultListNR MeasResultListNR, ..., measResultListEUTRA MeasResultListEUTRA } OPTIONAL, ..., [[ measResultCLI MeasResultCLI OPTIONAL ]] } MeasResultCLI ::= SEQUENCE { measResultCLI-RSSI MeasResultCLI-RSSI OPTIONAL, measResultSRS-RSRP MeasResultSRS-RSRP OPTIONAL, tooStrongToMeasure BOOLEAN }

# SRS configuration for CLI measurement

In R2-1905203, RAN1 shared the list of SRS configuration parameters for the UE to measure SRS-RSRP from UEs from another cell. The following SRS-scs parameter is defined as the subcarrier spacing for SRS to be measured.

| Parameter name in specification | New or existing parameter | Parameter name in text | Description | Value range |
| --- | --- | --- | --- | --- |
| SRS-scs | New | SRS-scs | Subcarrier spacing for SRS UE operates CLI measurement within the active BWP. The subcarrier spacing for CLI-measurement resource configuration can be same or different from the SCS of the active BWP. The UE is not required to measure SRS-RSRP if SCS of the SRS is different than the downlink active BWP SCS of the same carrier. | 15, 30, 60 kHz for FR1 60, 120 kHz for FR2 |

Description of the parameter states that UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier. This is based on the following RAN1 agreements in [2].

- Agreements on SRS-RSRP: - The number of SRS to be monitored by the UE should not exceed 8 within a slot - Network may configure more than 8 SRSs over different slots - The total number of SRSs to be monitored by a UE should not exceed [32] - The UE is not required to perform time tracking or time adjustment other than a constant offset relative to its own DL timing in order to perform SRS-RSRP measurement - FFS: Details on how the constant offset is derived by the UE. The constant offset of 0 is not precluded. - FFS: Whether or not to have measurement accuracy relaxation - FFS: Timing for the transmission point of view - UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier

Whether the “SRS-scs” parameter needs to be explicitly configured by the network depends on whether the SRS-RSRP measurement configuration is cell specific or BWP dependent. If the SRS-measurement configuration is cell specific, the “SRS-scs” parameter should be configured to the UE. Otherwise, the “SRS-scs” parameter can be omitted from the SRS-RSRP configuration because the network should only configure the SRS with SCS same as the downlink active BWP SCS of the same carrier.

If the SRS-RSRP measurement configuration is BWP dependent, network does not need to explicitly configure SRS-scs.

Although, SRS can be transmitted in up to 4 ports from a UE, it should be discussed in RAN2 whether the SRS-RSRP should be measured in up to 4 ports. For SRS-RSRP measurement report, L3 measurement reporting is applied. Current L3 measurement is performed in a single port. To make CLI measurement compatible with existing L3 measurement, the UE should be configured only to measure SRS-RSRP in a single SRS port. Note this does not preclude the UE from measuring multiple SRS ports if each port of the SRS is configured as a separate measurement resource.

Each SRS-RSRP measurement resource only includes a single SRS port. To measure the SRS transmitted from N (N=2, 4) ports from the interfering UE, network can configure N separate SRS resources that correspond to the N ports at the interfering UE.

| Parameter name in specification | New or existing parameter | Parameter name in text | Description | Value range |
| --- | --- | --- | --- | --- |
| nrofSRS-Ports | New | nrofSRS-Ports |  | 1,[2],[4] |

# Summary

Based on the above discussion, we have the following observations:

Observation 1.	CLI measurements are fundamentally different as they are done on UL signals whereas existing measurements are on DL signal.

Observation 2.	Coordination of the CLI-RSSI and SRS-RSRP measurement resources between intra-frequency neighbours is beneficial. This may need RAN3 work.

Observation 3.	Both periodic and event triggered measurement reporting are beneficial for CLI measurements.

Observation 4.	If the interference is too strong, either the measurement is not accurate, or the UE is not even able to detect the SRS sequence ID.

Based on the discussion and observations, we propose:

Proposal 1.	Define a new measurement object MeasObjectCLI for CLI-RSSI and SRS-RSRP measurements.

Proposal 2.	Include CLI-RSSI and SRS-RSRP measurement resource configuration in MeasObjectCLI.

Proposal 3.	Include L3 filtering configurations for CLI-RSSI and SRS-RSRP measurements in the corresponding measurement configurations.

Proposal 4.	From RAN2’s point of view, coordinating the CLI-RSSI and SRS-RSRP measurement resources between intra-frequency neighbours is beneficial.

Proposal 5.	Both periodical and event triggered reporting are supported for CLI-RSSI and SRS-RSRP measurements.

Proposal 6.	Define two new events for CLI measurements – 1) L1: CLI-RSSI or SRS-RSRP is above threshold; 2) L2: CLI-RSSI and SRS-RSRP are below threshold.

Proposal 7.	Include CLI-RSSI threshold, SRS-RSRP threshold, report on leave, hysteresis and time to trigger as configurations for both events.

Proposal 8.	Reuse reportInterval and reportAmount for both types of reporting triggers.

Proposal 9.	Define CLI-MeasQuantity to indicate which quantity (CLI-RSSI, SRS-RSRP) to measure and report.

Proposal 10.	Adopt the TP in section 3 as baseline.

Proposal 11.	Define intra-frequency measurement gap for CLI-RSSI and SRS-RSRP measurement.

Proposal 12.	It is up to UE implementation whether to measure CLI-RSSI and/or SRS-RSRP during off period of DRX cycle (i.e., other than Active Time), as long as performance requirements (to be defined by RAN4) can be met.

Proposal 13.	Add CLI measurement measResultCLI into MeasResults. CLI measurement is reported together with serving cell measurement result.

Proposal 14.	Include ‘too strong signal to measure’ as one of the possible reported values of CLI-RSSI and SRS-RSRP measurement result.

Proposal 15.	If the SRS-RSRP measurement configuration is BWP dependent, network does not need to explicitly configure SRS-scs.

Proposal 16.	Each SRS-RSRP measurement resource only includes a single SRS port. To measure the SRS transmitted from N (N=2, 4) ports from the interfering UE, network can configure N separate SRS resources that correspond to the N ports at the interfering UE.

# References

- R2-1905203/R1-1903836, RAN1 LS on RRC parameters for NR CLI-handling

- RP-190667, Status Report to TSG, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR