# 3GPP TSG RAN WG2 Meeting #106                                            R2-1906112

# Reno, USA, May 13th – May 17th, 2019

Agenda item:	11.6.3.1

Source:	ZTE Corporation, Sanechips

Title:	Ambiguity on preamble reception in NTN

Document for: 	   Discussion and decision

- Introduction

In this contribution, we address the observation on the ambiguity of preamble reception at gNB side caused by large transmission delay, discuss potential solutions and impact on RACH capacity.

- Discussion

  - Ambiguity on preamble reception

It’s captured in TS 38821 that transmission delay in NTN can be up to 541.14 ms (GEO) and 25.76 ms (LEO), with up to 16 ms (GEO) and 4.44 ms (LEO) differential delay separately, where differential delay equals to minimum one way delay minus maximum one way delay.

Observation 1: The differential delay within a satellite beam in Non-GEO can be up to 4.44 ms while for Non-GEO can be up to 16 ms.

Figure 1 Transmission of preamble

Such significant transmission delay might lead to the case where preambles transmitted in two consecutive ROs with interval less than 2*maximum differential delay can fall within the same preamble processing window at gNB side as depicted in Figure 1. Therefore gNB can not determine the exact RO in which received preambles were transmitted. If gNB can not identify in which RO the received preamble is transmitted on, then gNB is unable to estimate the correct timing advance, leading to uncertain behavior in both UE and gNB side.

- Based on the current specs, the ambiguity of preamble reception can only be avoided by the configuration of RACH resource, in which case the NW ensure the time interval between two consecutive RO is larger than the maximum delay difference*2 within the cell, which is 32 ms considering the worst case in GEO. Referring to Table 6.3.3.2-2 to Table 6.3.3.2-2 in TS 38.821, only limited PRACH configuration can meet the requirement on RO interval at time domain, e.g. the PRACH index highlighted in yellow in Annex 1, which can significantly impact the RACH density to be supported in time domain. Therefore, an analysis on RACH capacity taking into account the restriction above restriction is given in following section.

Observation 2: Insufficient time interval between two consecutive ROs will lead to ambiguity of preamble reception at gNB side, which can be resolved based on network implementation, by configuring consecutive ROs with time interval larger than maximum delay difference*2.

  - RACH capacity in current NR specifications

- The capacity of RACH is determined by sparsity of time and frequency resources, which is configured by higher-layer parameters prach-ConfigurationIndex and msg1-FDM defined in TS 38.331. Considering the worst case in GEO where the time interval of consecutive ROs can be larger than 32 ms, according to Table 6.3.3.2-2 to Table 6.3.3.2-3 in TS 38211, the max PRACH density in time domain for “FR1 and paired spectrum” and “FR1 and unpaired spectrum” is 0.025 number/ms among the PRACH configuration that meets this requirement, while no PRACH configuration in “FR2 and paired spectrum” can provide such large time interval.

- Assuming msg1-FDM is 8 and all 64 preambles are used for contention-based RACH, then we can obtain the maximum RACH attempts supported per cell with , and the user density supported for “FR1 and paired spectrum” and “FR1 and unpaired spectrum” as given in following tables. (The details of the calculation is provided in Annex 2)

- Table 1. Maximum RACH attempts supported per cell, with

| FR1/FR2 and spectrum type | Max RACH attempt supported (attempts/second/cell) |
| --- | --- |
| FR1 and paired spectrum | -12800*ln(1-0.01) = 128.64 |
| FR1 and unpaired spectrum | -12800*ln(1-0.01) = 128.64 |

- Table 2. User density supported for “FR1 and paired spectrum” and “FR1 and unpaired spectrum”

| - Cell coverage - (km2) | - User density can be supported - (UE number/km2) |  |  |
| --- | --- | --- | --- |
|  | - RACH requirement per UE = 0.001667 - attempts per second | - RACH requirement per UE = 0.0001667 - attempts per second | - RACH requirement per UE = 0.00001667 - attempts per second |
| - 10000 | - 7.7 | - 77.2 | - 771.7 |
| - 50000 | - 1.5 | - 15.4 | - 154.3 |
| - 100000 | - 0.8 | - 7.7 | - 77.2 |
| - 400000 | - 0.2 | - 1.9 | - 19.3 |
| - 800000 | - 0.1 | - 1.0 | - 9.6 |

Based on the table above, it can be observed that the solution based on RO configuration will decrease the RACH capacity significantly, leading to very limited user density in NTN, which is not future proof for supporting IoT service.

Observation 3: If the NW ensure the time interval between two consecutive RO is larger than the Maximum delay difference*2 based on PRACH configuration, then the RACH capacity will decrease significantly,  leading to very limited user density in NTN, which is not future proof for supporting IoT service.

Proposal 1: RAN2 is kindly asked to confirm the issue that, based on the current specs, the RACH capacity in NTN can not be satisfied in case the Maximum delay difference is large.

Proposal 2: Agree the text proposal provided, which captures the observation on ambiguity on preamble reception as well as the analysis on RACH capacity.

- Conclusion and proposals

With the above analysis, we have the following observation and proposals:

Observation 1: The differential delay within a satellite beam in Non-GEO can be up to 4.44 ms while for Non-GEO can be up to 16 ms.

Observation 2: Insufficient time interval between two consecutive ROs will lead to ambiguity of preamble reception at gNB side, which can be resolved based on network implementation, by configuring consecutive ROs with time interval larger than maximum delay difference*2.

Observation 3: If the NW ensure the time interval between two consecutive RO is larger than the Maximum delay difference*2 based on PRACH configuration, then the RACH capacity will decrease significantly, leading to very limited user density in NTN, which is not future proof for supporting IoT service.

Proposal 1: RAN2 is kindly asked to confirm the issue that, based on the current specs, the RACH capacity in NTN can not be satisfied in case the Maximum delay difference is large.

Proposal 2: Agree the text proposal provided, which captures the observation on ambiguity on preamble reception as well as the analysis on RACH capacity.

- Text Proposal

/* Text unchanged omitted /*

#### 7.2.1.1	Random Access

/* Text unchanged omitted /*

7.2.1.1.3		 RACH Capacity

Problem statement

Due to the significant transmission delay in NTN, preambles transmitted in two consecutive ROs with interval less than 2*maximum differential delay can fall within the same preamble processing window. Therefore gNB can not determine from which RO the received preambles were transmitted, which makes it unable to estimate the correct timing advance, leading to uncertain behavior in both UE and gNB side.

The ambiguity of preamble reception can be resolved if the network ensures the time interval between two consecutive ROs is larger than the Maximum delay difference based on PRACH configuration. Considering the worst case in GEO, the time interval of consecutive ROs can be larger than 32 ms. According to Table 6.3.3.2-2 to Table 6.3.3.2-3 in TS 38211, the max PRACH density in time domain for “FR1 and paired spectrum” and “FR1 and unpaired spectrum” is 0.025 number/ms among the PRACH configuration that meets this requirement, while no PRACH configuration in “FR2 and paired spectrum” can provide such large time interval.

Assuming msg1-FDM is 8 and all 64 preambles are used for contention-based RACH, then we can obtain the maximum RACH attempts supported per cell with [4] and the user density can be supported for “FR1 and paired spectrum” and “FR1 and unpaired spectrum” as given in the following tables.

Table 1. Maximum RACH attempts supported per cell, with

| FR1/FR2 and spectrum type | Max RACH attempt supported (attempts/second/cell) |
| --- | --- |
| FR1 and paired spectrum | -12800*ln(1-0.01) = 128.64 |
| FR1 and unpaired spectrum | -12800*ln(1-0.01) = 128.64 |

Table 2. User density supported for “FR1 and paired spectrum” and “FR1 and unpaired spectrum”

| Cell coverage (km2) | User density can be supported (UE number/km2) |  |  |
| --- | --- | --- | --- |
|  | RACH requirement per UE = 0.001667 attempts per second | RACH requirement per UE = 0.0001667 attempts per second | RACH requirement per UE = 0.00001667 attempts per second |
| 10000 | 7.7 | 77.2 | 771.7 |
| 50000 | 1.5 | 15.4 | 154.3 |
| 100000 | 0.8 | 7.7 | 77.2 |
| 400000 | 0.2 | 1.9 | 19.3 |
| 800000 | 0.1 | 1.0 | 9.6 |

Based on the figures in Table 2 , it can be observed that the solution based on RO configuration will decrease the RACH capacity significantly, leading to very limited user density in NTN, which is not future proof for supporting IoT service.

- References

- R2-1903391,Consideration on Random Access for NTN, ZTE

- TS 38.331, NR - Radio Resource Control (RRC) protocol specification, R15

- TR 38.811, Study on New Radio (NR) to support non terrestrial networks, R15

- R1-061369, LTE random-access capacity and collision probability, Ericsson

- R1-1709895, NR-PRACH formats: capacity enhancements and other use cases, ZTE

- Annex2: PRACH configuration in TS 38.821

Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/supplementary uplink.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 16 | 1 | 1 | 0 | - | - | 0 |
| 1 | 0 | 16 | 1 | 4 | 0 | - | - | 0 |
| 2 | 0 | 16 | 1 | 7 | 0 | - | - | 0 |
| 3 | 0 | 16 | 1 | 9 | 0 | - | - | 0 |
| 4 | 0 | 8 | 1 | 1 | 0 | - | - | 0 |
| 5 | 0 | 8 | 1 | 4 | 0 | - | - | 0 |
| 6 | 0 | 8 | 1 | 7 | 0 | - | - | 0 |
| 7 | 0 | 8 | 1 | 9 | 0 | - | - | 0 |
| 8 | 0 | 4 | 1 | 1 | 0 | - | - | 0 |
| 9 | 0 | 4 | 1 | 4 | 0 | - | - | 0 |
| 10 | 0 | 4 | 1 | 7 | 0 | - | - | 0 |
| 11 | 0 | 4 | 1 | 9 | 0 | - | - | 0 |
| 12 | 0 | 2 | 1 | 1 | 0 | - | - | 0 |
| 13 | 0 | 2 | 1 | 4 | 0 | - | - | 0 |
| 14 | 0 | 2 | 1 | 7 | 0 | - | - | 0 |
| 15 | 0 | 2 | 1 | 9 | 0 | - | - | 0 |
| 16 | 0 | 1 | 0 | 1 | 0 | - | - | 0 |
| 17 | 0 | 1 | 0 | 4 | 0 | - | - | 0 |
| 18 | 0 | 1 | 0 | 7 | 0 | - | - | 0 |
| 19 | 0 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 20 | 0 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 21 | 0 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 22 | 0 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 23 | 0 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 24 | 0 | 1 | 0 | 3, 6, 9 | 0 | - | - | 0 |
| 25 | 0 | 1 | 0 | 0,2,4,6,8 | 0 | - | - | 0 |
| 26 | 0 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 27 | 0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | - | - | 0 |
| 28 | 1 | 16 | 1 | 1 | 0 | - | - | 0 |
| 29 | 1 | 16 | 1 | 4 | 0 | - | - | 0 |
| 30 | 1 | 16 | 1 | 7 | 0 | - | - | 0 |
| 31 | 1 | 16 | 1 | 9 | 0 | - | - | 0 |
| 32 | 1 | 8 | 1 | 1 | 0 | - | - | 0 |
| 33 | 1 | 8 | 1 | 4 | 0 | - | - | 0 |
| 34 | 1 | 8 | 1 | 7 | 0 | - | - | 0 |
| 35 | 1 | 8 | 1 | 9 | 0 | - | - | 0 |
| 36 | 1 | 4 | 1 | 1 | 0 | - | - | 0 |
| 37 | 1 | 4 | 1 | 4 | 0 | - | - | 0 |
| 38 | 1 | 4 | 1 | 7 | 0 | - | - | 0 |
| 39 | 1 | 4 | 1 | 9 | 0 | - | - | 0 |
| 40 | 1 | 2 | 1 | 1 | 0 | - | - | 0 |
| 41 | 1 | 2 | 1 | 4 | 0 | - | - | 0 |
| 42 | 1 | 2 | 1 | 7 | 0 | - | - | 0 |
| 43 | 1 | 2 | 1 | 9 | 0 | - | - | 0 |
| 44 | 1 | 1 | 0 | 1 | 0 | - | - | 0 |
| 45 | 1 | 1 | 0 | 4 | 0 | - | - | 0 |
| 46 | 1 | 1 | 0 | 7 | 0 | - | - | 0 |
| 47 | 1 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 48 | 1 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 49 | 1 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 50 | 1 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 51 | 1 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 52 | 1 | 1 | 0 | 3,6,9 | 0 | - | - | 0 |
| 53 | 2 | 16 | 1 | 1 | 0 | - | - | 0 |
| 54 | 2 | 8 | 1 | 1 | 0 | - | - | 0 |
| 55 | 2 | 4 | 0 | 1 | 0 | - | - | 0 |
| 56 | 2 | 2 | 0 | 1 | 0 | - | - | 0 |
| 57 | 2 | 2 | 0 | 5 | 0 | - | - | 0 |
| 58 | 2 | 1 | 0 | 1 | 0 | - | - | 0 |
| 59 | 2 | 1 | 0 | 5 | 0 | - | - | 0 |
| 60 | 3 | 16 | 1 | 1 | 0 | - | - | 0 |
| 61 | 3 | 16 | 1 | 4 | 0 | - | - | 0 |
| 62 | 3 | 16 | 1 | 7 | 0 | - | - | 0 |
| 63 | 3 | 16 | 1 | 9 | 0 | - | - | 0 |
| 64 | 3 | 8 | 1 | 1 | 0 | - | - | 0 |
| 65 | 3 | 8 | 1 | 4 | 0 | - | - | 0 |
| 66 | 3 | 8 | 1 | 7 | 0 | - | - | 0 |
| 67 | 3 | 4 | 1 | 1 | 0 | - | - | 0 |
| 68 | 3 | 4 | 1 | 4 | 0 | - | - | 0 |
| 69 | 3 | 4 | 1 | 7 | 0 | - | - | 0 |
| 70 | 3 | 4 | 1 | 9 | 0 | - | - | 0 |
| 71 | 3 | 2 | 1 | 1 | 0 | - | - | 0 |
| 72 | 3 | 2 | 1 | 4 | 0 | - | - | 0 |
| 73 | 3 | 2 | 1 | 7 | 0 | - | - | 0 |
| 74 | 3 | 2 | 1 | 9 | 0 | - | - | 0 |
| 75 | 3 | 1 | 0 | 1 | 0 | - | - | 0 |
| 76 | 3 | 1 | 0 | 4 | 0 | - | - | 0 |
| 77 | 3 | 1 | 0 | 7 | 0 | - | - | 0 |
| 78 | 3 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 79 | 3 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 80 | 3 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 81 | 3 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 82 | 3 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 83 | 3 | 1 | 0 | 3, 6, 9 | 0 | - | - | 0 |
| 84 | 3 | 1 | 0 | 0,2,4,6,8 | 0 | - | - | 0 |
| 85 | 3 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 86 | 3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | - | - | 0 |
| 87 | A1 | 16 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 88 | A1 | 16 | 1 | 4 | 0 | 2 | 6 | 2 |
| 89 | A1 | 8 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 90 | A1 | 8 | 1 | 4 | 0 | 2 | 6 | 2 |
| 91 | A1 | 4 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 92 | A1 | 4 | 1 | 4,9 | 0 | 1 | 6 | 2 |
| 93 | A1 | 4 | 0 | 4 | 0 | 2 | 6 | 2 |
| 94 | A1 | 2 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 95 | A1 | 2 | 0 | 1 | 0 | 2 | 6 | 2 |
| 96 | A1 | 2 | 0 | 4 | 0 | 2 | 6 | 2 |
| 97 | A1 | 2 | 0 | 7 | 0 | 2 | 6 | 2 |
| 98 | A1 | 1 | 0 | 4 | 0 | 1 | 6 | 2 |
| 99 | A1 | 1 | 0 | 1,6 | 0 | 1 | 6 | 2 |
| 100 | A1 | 1 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 101 | A1 | 1 | 0 | 1 | 0 | 2 | 6 | 2 |
| 102 | A1 | 1 | 0 | 7 | 0 | 2 | 6 | 2 |
| 103 | A1 | 1 | 0 | 2,7 | 0 | 2 | 6 | 2 |
| 104 | A1 | 1 | 0 | 1,4,7 | 0 | 2 | 6 | 2 |
| 105 | A1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 6 | 2 |
| 106 | A1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 6 | 2 |
| 107 | A1 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 6 | 2 |
| 108 | A1/B1 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 109 | A1/B1 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 110 | A1/B1 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 111 | A1/B1 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 112 | A1/B1 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 113 | A1/B1 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 114 | A1/B1 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 115 | A1/B1 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 116 | A1/B1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 117 | A2 | 16 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 118 | A2 | 16 | 1 | 4 | 0 | 2 | 3 | 4 |
| 119 | A2 | 8 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 120 | A2 | 8 | 1 | 4 | 0 | 2 | 3 | 4 |
| 121 | A2 | 4 | 0 | 2,6,9 | 0 | 1 | 3 | 4 |
| 122 | A2 | 4 | 0 | 4 | 0 | 2 | 3 | 4 |
| 123 | A2 | 2 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 124 | A2 | 2 | 0 | 1 | 0 | 2 | 3 | 4 |
| 125 | A2 | 2 | 0 | 4 | 0 | 2 | 3 | 4 |
| 126 | A2 | 2 | 0 | 7 | 0 | 2 | 3 | 4 |
| 127 | A2 | 1 | 0 | 4 | 0 | 1 | 3 | 4 |
| 128 | A2 | 1 | 0 | 1,6 | 0 | 1 | 3 | 4 |
| 129 | A2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 130 | A2 | 1 | 0 | 1 | 0 | 2 | 3 | 4 |
| 131 | A2 | 1 | 0 | 7 | 0 | 2 | 3 | 4 |
| 132 | A2 | 1 | 0 | 2,7 | 0 | 2 | 3 | 4 |
| 133 | A2 | 1 | 0 | 1,4,7 | 0 | 2 | 3 | 4 |
| 134 | A2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 3 | 4 |
| 135 | A2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 3 | 4 |
| 136 | A2 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 3 | 4 |
| 137 | A2/B2 | 2 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 138 | A2/B2 | 2 | 0 | 4 | 0 | 2 | 3 | 4 |
| 139 | A2/B2 | 1 | 0 | 4 | 0 | 1 | 3 | 4 |
| 140 | A2/B2 | 1 | 0 | 1,6 | 0 | 1 | 3 | 4 |
| 141 | A2/B2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 142 | A2/B2 | 1 | 0 | 1 | 0 | 2 | 3 | 4 |
| 143 | A2/B2 | 1 | 0 | 7 | 0 | 2 | 3 | 4 |
| 144 | A2/B2 | 1 | 0 | 1,4,7 | 0 | 2 | 3 | 4 |
| 145 | A2/B2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 3 | 4 |
| 146 | A2/B2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 3 | 4 |
| 147 | A3 | 16 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 148 | A3 | 16 | 1 | 4 | 0 | 2 | 2 | 6 |
| 149 | A3 | 8 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 150 | A3 | 8 | 1 | 4 | 0 | 2 | 2 | 6 |
| 151 | A3 | 4 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 152 | A3 | 4 | 0 | 4 | 0 | 2 | 2 | 6 |
| 153 | A3 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 154 | A3 | 2 | 0 | 1 | 0 | 2 | 2 | 6 |
| 155 | A3 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 156 | A3 | 2 | 0 | 7 | 0 | 2 | 2 | 6 |
| 157 | A3 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 158 | A3 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 159 | A3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 160 | A3 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 161 | A3 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 162 | A3 | 1 | 0 | 2,7 | 0 | 2 | 2 | 6 |
| 163 | A3 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 164 | A3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 165 | A3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 166 | A3 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 2 | 6 |
| 167 | A3/B3 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 168 | A3/B3 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 169 | A3/B3 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 170 | A3/B3 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 171 | A3/B3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 172 | A3/B3 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 173 | A3/B3 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 174 | A3/B3 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 175 | A3/B3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 176 | A3/B3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 177 | B1 | 16 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 178 | B1 | 16 | 1 | 4 | 0 | 2 | 7 | 2 |
| 179 | B1 | 8 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 180 | B1 | 8 | 1 | 4 | 0 | 2 | 7 | 2 |
| 181 | B1 | 4 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 182 | B1 | 4 | 1 | 4,9 | 0 | 1 | 7 | 2 |
| 183 | B1 | 4 | 0 | 4 | 0 | 2 | 7 | 2 |
| 184 | B1 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 185 | B1 | 2 | 0 | 1 | 0 | 2 | 7 | 2 |
| 186 | B1 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 187 | B1 | 2 | 0 | 7 | 0 | 2 | 7 | 2 |
| 188 | B1 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 189 | B1 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 190 | B1 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 191 | B1 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 192 | B1 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 193 | B1 | 1 | 0 | 2,7 | 0 | 2 | 7 | 2 |
| 194 | B1 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 195 | B1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 196 | B1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 7 | 2 |
| 197 | B1 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 7 | 2 |
| 198 | B4 | 16 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 199 | B4 | 16 | 1 | 4 | 0 | 2 | 1 | 12 |
| 200 | B4 | 8 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 201 | B4 | 8 | 1 | 4 | 0 | 2 | 1 | 12 |
| 202 | B4 | 4 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 203 | B4 | 4 | 0 | 4 | 0 | 2 | 1 | 12 |
| 204 | B4 | 4 | 1 | 4,9 | 0 | 2 | 1 | 12 |
| 205 | B4 | 2 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 206 | B4 | 2 | 0 | 1 | 0 | 2 | 1 | 12 |
| 207 | B4 | 2 | 0 | 4 | 0 | 2 | 1 | 12 |
| 208 | B4 | 2 | 0 | 7 | 0 | 2 | 1 | 12 |
| 209 | B4 | 1 | 0 | 1 | 0 | 2 | 1 | 12 |
| 210 | B4 | 1 | 0 | 4 | 0 | 2 | 1 | 12 |
| 211 | B4 | 1 | 0 | 7 | 0 | 2 | 1 | 12 |
| 212 | B4 | 1 | 0 | 1,6 | 0 | 2 | 1 | 12 |
| 213 | B4 | 1 | 0 | 2,7 | 0 | 2 | 1 | 12 |
| 214 | B4 | 1 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 215 | B4 | 1 | 0 | 1,4,7 | 0 | 2 | 1 | 12 |
| 216 | B4 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 1 | 12 |
| 217 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 1 | 12 |
| 218 | B4 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 1 | 12 |
| 219 | C0 | 8 | 1 | 4 | 0 | 2 | 7 | 2 |
| 220 | C0 | 4 | 1 | 4,9 | 0 | 1 | 7 | 2 |
| 221 | C0 | 4 | 0 | 4 | 0 | 2 | 7 | 2 |
| 222 | C0 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 223 | C0 | 2 | 0 | 1 | 0 | 2 | 7 | 2 |
| 224 | C0 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 225 | C0 | 2 | 0 | 7 | 0 | 2 | 7 | 2 |
| 226 | C0 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 227 | C0 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 228 | C0 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 229 | C0 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 230 | C0 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 231 | C0 | 1 | 0 | 2,7 | 0 | 2 | 7 | 2 |
| 232 | C0 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 233 | C0 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 234 | C0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 7 | 2 |
| 235 | C0 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 7 | 2 |
| 236 | C2 | 16 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 237 | C2 | 16 | 1 | 4 | 0 | 2 | 2 | 6 |
| 238 | C2 | 8 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 239 | C2 | 8 | 1 | 4 | 0 | 2 | 2 | 6 |
| 240 | C2 | 4 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 241 | C2 | 4 | 0 | 4 | 0 | 2 | 2 | 6 |
| 242 | C2 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 243 | C2 | 2 | 0 | 1 | 0 | 2 | 2 | 6 |
| 244 | C2 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 245 | C2 | 2 | 0 | 7 | 0 | 2 | 2 | 6 |
| 246 | C2 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 247 | C2 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 248 | C2 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 249 | C2 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 250 | C2 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 251 | C2 | 1 | 0 | 2,7 | 0 | 2 | 2 | 6 |
| 252 | C2 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 253 | C2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 254 | C2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 255 | C2 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 2 | 6 |

- Annex1: Calculation of RACH capacity and user density

- For FR1 and paired spectrum :

- Table 1. Max PRACH density in time domain, for FR1 and paired spectrum/supplementary uplink

| - Preamble format | - PRACH Configuration Index | - Max PRACH density in time domain (number/ms) |
| --- | --- | --- |
| - 0 | - 8 | - 0.025 |
| - 1 | - 36 | - 0.025 |
| - 2 | - 55 | - 0.025 |

- For FR1 and unpaired spectrum :

- Table 2. Max PRACH density in time domain, for FR1 and unpaired spectrum

| - Preamble format | - PRACH Configuration Index | - Max PRACH density in time domain (number/ms) |
| --- | --- | --- |
| - 0 | - 2 | - 0.025 |
| - 1 | - 30 | - 0.025 |
| - 2 | - 36 | - 0.025 |

- (Noted that above table only gives some feasible examples, not all PARCH configuration suitable is provided. )

For FR2 and unpaired spectrum, not suitable PRACH configuration can satisfy the condition that RO allocation within the time domain is larger than 16 ms.

To simplify analysis, assuming FDM is 8 and all 64 preamble used for CBRA. Thus we obtain:

- Table 3. Summary of max PRACH density in time/freq domain

| FR1/FR2 and spectrum type | Max PRACH density in time/frequency domain (number/second) |
| --- | --- |
| FR1 and paired spectrum/supplementary uplink | 0.025*8*1000*64 = 12800 |
| FR1 and unpaired spectrum | 0.025*8*1000*64 = 12800 |

The collision probability can be expressed as  according to [4]. In this formula,  is the total number of RACH opportunities per second (corresponding to values in Table 3), and  is the RACH attempt density with unit of attempts/second/cell. To achieve a reasonable collision probability, e.g., , supportable RACH attempt density is calculated as in Table 4.

- Table 4. Maximum RACH attempts supported per cell, with

| FR1/FR2 and spectrum type | Max RACH attempt supported (attempts/second/cell) |
| --- | --- |
| FR1 and paired spectrum | -12800*ln(1-0.01) = 128.64 |
| FR1 and unpaired spectrum | -12800*ln(1-0.01) = 128.64 |

With the “Max RACH attempt supported per cell” calculated in Table 4, the maximum UE density supported should fulfill the equation as follows:

“cell coverage” * “user density” * “RACH requirement per UE” <= “Max RACH attempt supported”

Therefore, the maximum user density supported can be calculated with the formula as follows:

Maximum User density = “Max RACH attempt supported” / (“cell coverage” * “RACH requirement per UE”)

For the RACH requirement per UE, the following assumption are used in the RACH capacity estimation in RAN1 [5]:

- Initial access: 1 every 10min

- Handover: 1 every 10min (For LEO, considering the handover is still required even the UE is static)

- Paging: 1 every 10min

- Other SI: 1every 10min

Based on the assumption above, the overall RACH requirement per UE is 0.001667 attempts/second/user.

Considering the service mode for NTN may be different from normal UE, besides the 0.001667, the 0.0001667 and 0.00001667 will be evaluated as well.

With the assumption above, the evaluation result are summarized in the table as follow:

- Table 5. User density supported for “FR1 and paired spectrum” and “FR1 and unpaired spectrum”

| - Cell coverage - (km2) | - User density can be supported - (UE number/km2) |  |  |
| --- | --- | --- | --- |
|  | - RACH requirement per UE = 0.001667 - attempts per second | - RACH requirement per UE = 0.0001667 - attempts per second | - RACH requirement per UE = 0.00001667 - attempts per second |
| - 10000 | - 7.7 | - 77.2 | - 771.7 |
| - 50000 | - 1.5 | - 15.4 | - 154.3 |
| - 100000 | - 0.8 | - 7.7 | - 77.2 |
| - 400000 | - 0.2 | - 1.9 | - 19.3 |
| - 800000 | - 0.1 | - 1.0 | - 9.6 |