3GPP TSG RAN WG2 #106	 R2-1906219

Reno, US, 13 – 17 May, 2019

Agenda Item:	11.9.3.1 Conditional handover

Source: 	Ericsson

Title:  	TP to 38.300 on Conditional Handover in NR

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements one objective is to improve the robustness at handover. In RAN2#105bis in Xi’an, CHO has been agreed to be introduced and some high level progress has been reached.

Agreements

0:	CHO is introduced in NR to solve robustness/reliability issue.

. . .

Herein we start a TP to start capturing stage-2 agreements for CHO in NR. We show changes on top of the NR handover section, so it becomes visible how we propose to capture CHO agreements in 38.300.

# TP to 38.300

#### 9.2.3.3	Conditional Handover

##### 9.2.3.2.1	C-Plane Handling

As in the case of handover, in conditional handover the intra-NR RAN handover performs the preparation and execution phase of the conditional handover procedure performed without involvement of the 5GC, i.e. preparation messages are directly exchanged between the gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target candidate gNB associated to the cell that the UE selects (in case of multiple triggering cells). The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover

0.	The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to configure conditional handover to the UE, possibly based on MeasurementReport and RRM information.

3.	The source gNB issues a Handover Request message indicating a conditional handover to a target candidate gNB passing a transparent RRC container with necessary information to prepare the conditional handover at the target candidate side. The information includes at least the target candidate cell ID, KgNB*, the C-RNTI of the UE in the source gNB, RRM-configuration including UE inactive time, basic AS-configuration including antenna Info and DL Carrier Frequency, the current QoS flow to DRB mapping rules applied to the UE, the SIB1 from source gNB, the UE capabilities for different RATs, PDU session related information, and can include the UE reported measurement information including beam-related information if available. The PDU session related information includes the slice information (if supported) and QoS flow level QoS profile(s).

FFS 	After issuing a Handover Request for conditional handover, the source gNB should not reconfigure the UE, including performing Reflective QoS flow to DRB mapping.

4.	Admission Control may be performed by the target candidate gNB. Slice-aware admission control shall be performed if the slice information is sent to the target gNB. If the PDU sessions are associated with non-supported slices the target gNB shall reject such PDU Sessions.

5.	The target candidate gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE for conditional handover to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the conditional handover.

6.	The source gNB configured the Uu conditional handover by sending an RRCConditionalReconfiguration to the UE, containing the configuration of a triggering condition (e.g. like an A3 event in reportConfig) and an RRCReconfiguration message for each target cell candidate, containing the information required to access the target cell candidate: at least the target candidate cell ID, the new candidate C-RNTI, the target candidate gNB security algorithm identifiers for the selected security algorithms. It can also include the association between RACH resources and SSB(s), common RACH resources, and system information of each target candidate cell, etc. Upon reception of the configurations, the UE starts monitoring the CHO triggering conditions.

FFS Whether CHO configuration may contain dedicated RACH resources.

7.	The source gNB sends the SN STATUS TRANSFER message to the target gNB.

8.	Upon the fulfilment of a triggering condition for CHO, the UE synchronises to the selected target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

9.	The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

10.	5GC switches the DL data path towards the target gNB. The UPF sends one or more "end marker" packets on the old path to the source gNB per PDU session/tunnel and then can release any U-plane/TNL resources towards the source gNB.

11.	The AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

12.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

FFS RACH configuration allowed to be configured in CHO configuration.