3GPP TSG-RAN WG2 Meeting #106	R2-1906260

# Reno, USA, 13th- 17th May

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3978 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the applied freqBandList filter in UE-EUTRA-Capability_option 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following NOTE is added in TS36.331 to clarify that even when network requests only capabilities for eutra, the requestedFreqBandsNR-MRDC is also included in the UECapabilityEnquiry message. NOTE 3: Even if the network requests (only) capabilities for eutra, it may include NR band numbers in the requestedFreqBandsNR-MRDC in order to ensure that the UE includes all necessary feature sets (i.e. E-UTRA and NR) needed for subsequently requested eutra-nr capabilities. Usually,after requesting the capabilities for eutra, a subsequent UECapabilityEnquiry message which contains requestedFreqBandsNR-MRDC will be sent to UE to enquire capabilities for eutra-nr and the appliedFreqBandListFilter will be echoed back to network. However, it is also possible that network release the RRC connection after successfully enquiring the capabilities for eutra without sending the subsequent UECapabilityEnquiry message to enquire capabilities for eutra-nr. In this case, the network receives the capabilities for eutra but has no idea how it has been filtered. When handover happens, the source gNB will provide the UE capabilities for eutra to the target gNB. Without the applied filter, the target gNB has no idea how the received capabilities have been filtered in the source gNB and has no choice but to enquire the capabilities for eutra again. To reduce unnecessary UE capability enquiry procedure, the above NOTE 3 is deleted and capabilities for eutra and eutra-nr are always enquired in the same UECapabilityEnquiry message which includes requestedFreqBandsNR-MRDC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NOTE 3 in 5.6.3 is deleted. Impact analysis Impacted 5G architecture options: Standalone, MR-DC Impacted functionality: UE capability filtering Inter-operability: 1. If the UE is implemented according to this CR but the network is not, UE cannot understand the NR band numbers in the requestedFreqBandsNR-MRDC. 2. If the network is implemented according to this CR but the UE is not, no inter-operability is forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | unnecessary UE capability enquiry procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.6.3	UE capability transfer

NOTE 2:	The network must include the requestedFreqBandsNR-MRDC in order to obtain feature sets for E-UTRA and MR-DC.

End of change