3GPP TSG-RAN WG2 Meeting #106	R2-1906382

Reno, Nevada, USA May 13th – May 17th 2019					   (rev: R2-1903192)

Agenda Item:	12.3.3.1 (LTE_feMob)

Source:	Charter Communications

Title:	On Maximum Number of CHO Candidate Targets

Document for: Discussion and Agreement

# Introduction

A new WI [1] for even further Mobility enhancement in E-UTRAN was approved at RAN#80. The WI scope covers the following:

- Specify further enhancements to achieve following targets, [RAN2/3]

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover,

- Specify necessary core requirements for the identified solutions [RAN4]

This work has been split into the following phases:

- Study Phase, to evaluate the proposed solutions, e.g. simultaneous connectivity with both source and target eNB, conditional handover and enhancements to make-before-break, including support of carrier aggregation in source and carrier aggregation in target eNB during handover, and do down selection or merger, if necessary.

- Work Phase, to specify the chosen solution(s)

The following was agreed for LTE Conditional HO (CHO) at RAN2#104 [2]:

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

In this contribution, we focus on the issue of number of candidate cells for CHO, and provide a way forward.

# Discussion

At RAN2#105, contributions in [3], [4], [5], [6], [7], [8] discussed the issue of number of candidate cells to be supported for CHO. Whilst the authors of this paper (we) acknowledge that technical complexities are of paramount importance when making design decisions on CHO, we’d like to highlight the usage of this CHO feature from a practical stand point.

Development of LTE specifications was centered around Macro eNBs. Although, HeNB support was added in later releases of LTE, given the challenges (e.g. cost, business models etc) associated with small cells, its deployment didn’t really achieve the expected penetration rate.

With developments such as inclusion of FR2 as a valid frequency range for deployment of 3GPP’s 5G NR technology, FCC’s R&O [8] for opening of 3.5GHz CBRS spectrum in USA which allows indoor (Cat A CBSDs) and outdoor (Cat B CBSDs) mode of service operation etc, small cell-based deployments are either already becoming or are soon to be a reality. An LTE network which uses a combination of macro nodes together w/ small cells or a small-cell only network (for regional deployments) is expected to benefit the most from CHO.

The authors believe that neighbor cell relationship for macro nodes looks drastically different to that of small cell nodes. The density of target users adds further layer of complexity e.g. a deployment targeted for a downtown area is expected to be denser (more neighbor cells) than one which targets sub-urban or rural areas.

Observation 1: The number of target cells for CHO will vary depending on the type of targeted deployment (e.g. sparse v/s dense area).

Our initial radio planning for Multi Dwelling Units (MDUs ala low/mid/high-rise residential or enterprise buildings) extremely dense areas such as downtown NewYork City (NY, USA) reveals that the number of candidate target cells (assuming omnidirectional cell for indoor small-cell eNBs, and sectorized cells for outdoor small-cell eNBs) could range from 8-20. Whereas, the number of candidate target cells for a sub-urban/rural macro-based deployment may be no more than 2-8.

Observation 2: The number of target (small-)cells for CHO for urban/sub-urban/rural macro- deployment could be from 2-8, whereas for dense urban small cell-deployment could be up to 20.

We do acknowledge that various technical aspects, such as the ones mentioned below, associated with supporting candidate target cells for CHO need to be taken into considerations:

- How are the candidate target cells communicated to the UE? E.g. in a single message, or one message per target cell etc

- How are updates to the candidate targets cells done? E.g. multiple single messages, delta messages etc

- How often are such updates expected once CHO starts?

- How to balance resource reservation against cell’s performance?

- etc…

We understand that resolution of above questions is questions. However, we assert that more important than the “hows” is the question of “why”? Which ties in with usability of this feature. An LTE small-cell deployment is likely to derive more benefit from CHO feature than a macro-cell based deployment.

We understand the following options have been proposed thus far:

- Support multiple candidate cells but only within a single candidate target node e.g. in [11]

- Support multiple candidate cells with a number ranging from 1 through 6 e.g. in [3], [10]

- Support multiple candidate cells but leave the exact number to network implementation e.g. in [13]

- Support multiple candidate cells but determine the exact number during Stage 3 e.g. in [7], [12]

/a/ and /b/ whilst the simplest of the options would discourage deployment of CHO itself because it offers a limiting solution (not helpful for deployments w/ LTE small-cell type); Ergo, negating the value of a feature which has good potential to make a real difference.

/d/ whilst a sound approach from an engineering perspective lacks a design goal. This carries the risk of making it a subject of technical limitations imposed by selected solutions/mechanisms.

/c/ does offers flexibility to different operators based on their needs. However, it is no different to /d/ as it keeps the CHO feature a slave to protocol limitations e.g. if RAN2 agrees that only a single CHO command is allowed towards a UE, and such a command contain “full” configuration (not “delta” configuration), then it will implicitly limit the number of candidate cells any network implementation can support.

Given the above, we propose to set, as a design target, the maximum number of candidate cells for CHOfor this release (Rel-16) be 16 (which if needed can be represented by 4-bits over RRC).

Conclusion 1: The design target for maximum number of candidate target cells to be supported for Rel-16 Conditional HO should be set to 16.

Some companies had mentioned to us offline that setting a CHO design target of 16 (per Conclusion#1) may place additional RRM requirements towards RAN4. We understand that RAN4 has defined various limits for the number of simultaneous cells which can be monitored by UE for LTE (E-UTRA) per frequency layer (distinction between intra- and inter- frequency), together with the maximum number of frequency layers supported by current specifications. While we think RAN4 specs already allow for simultaneous monitoring of 16 CHO target cells, to address their concerns, we suggest to liaise with RAN4 to communicate RAN2’s design intent for CHO. A companion LS (in [14]) is also submitted for RAN2’s consideration.

Conclusion 2: RAN2 should liaise with RAN4 (and RAN1, RAN3) to communicate its design intent for maximum number of CHO candidates a UE is required to monitor.

# Conclusion

The following proposals are provided in this contribution:

Observation 1: The number of target cells for CHO will vary depending on the type of targeted deployment (e.g. sparse v/s dense area).

Observation 2: The number of target (small-)cells for CHO for urban/sub-urban/rural macro- deployment could be from 2-8, whereas for dense urban small cell-deployment could be up to 20.

Conclusion 1: The design target for maximum number of candidate target cells to be supported for Rel-16 Conditional HO should be set to 16.

Conclusion 2: RAN2 should liaise with RAN4 (and RAN1, RAN3) to communicate its design intent for maximum number of CHO candidates a UE is required to monitor.

# References

- RP-181475 New Work Item on even further Mobility enhancement in E-UTRAN

- R2-1818528, “Report from break-out session on Rel-16 NR Industrial IoT SID and Rel-16 LTE even further mobility enhancements WID”, Nokia

- R2-1900276, “Support of One or More Candidate Cells for Conditional HO”, CATT

- R2-1900511, “Discussion on the number of prepared cells for CHO”, vivo

- R2-1900924, “Number of Prepared Cells in Conditional Handover”, ETRI

- R2-1900988, “On Supporting Conditional HO”, Samsung

- R2-1901082, “Further details of conditional handover in LTE”, Ericsson

- R2-1901378, “Configuration of CHO”, China Telecom

- FCC 15-47, REPORT AND ORDER AND SECOND FURTHER NOTICE OF PROPOSED RULEMAKING

- R2-1903435, “Discussion on the number of prepared cells for CHO”, vivo

- R2-1905136, Consideration of multiple candidate target cell for LTE CHO”, Samsung

- R2-1903884, “Configuration of Conditional handover in LTE“, Ericsson

- R2-1904343, “Remaining Issues of LTE Conditional Handover”, CMCC

- R2-1906399, “LS on Maximum Number of Candidate Target Cells for Conditional Handover in E-UTRAN”, Charter Communications

Small cell refers to either micro, pico, and femto cells