3GPP TSG-RAN WG2 Meeting #106	R2-1906400

Reno, Nevada, USA May 13th – May 17th 2019

Agenda Item:	12.3.3.1 (LTE_feMob)

Source:	Charter Communications

Title:	CHO “Bye” Indication on the source link

Document for: Discussion and Agreement

# Introduction

A new WI [1] for even further Mobility enhancement in E-UTRAN was approved at RAN#80. The WI scope covers the following:

- Specify further enhancements to achieve following targets, [RAN2/3]

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover,

- Specify necessary core requirements for the identified solutions [RAN4]

This work has been split into the following phases:

- Study Phase, to evaluate the proposed solutions, e.g. simultaneous connectivity with both source and target eNB, conditional handover and enhancements to make-before-break, including support of carrier aggregation in source and carrier aggregation in target eNB during handover, and do down selection or merger, if necessary.

- Work Phase, to specify the chosen solution(s)

The following was agreed for LTE Conditional HO (CHO) at RAN2#104 [2]:

Agreements

1  Existing Ax measurement events can be used for executing CHO. FFS which Ax events can be used.

2  Conventional handover overrides any configured conditional handover command

3  The network can inform the UE to release CHO configurations (e.g. candidate cells) by RRC signaling.

=> FFS how “CHO cmd” is formulated in Stage-3 signalling

=> FFS whether UE continues to receive source cell while executing CHO cmd.

=> FFS what UE does if it receives HO cmd while executing CHO cmd.

=> FFS what UE does if NW removes CHO cmd while executing the same CHO cmd.

=> FFS whether UE stores CHO commands in failure cases

=> FFS whether CHO candidates can be released via other means.

The UE behavior if it receives legacy HO command whilst executing CHO was FFS. This assumes that UE is capable of receive RRC commands from source node between the time UE triggers CHO and CHO completion with target node. In this contribution, we focus not on the FFS per se, but on the fact that if UE is still able to have a link (even if its at cell edge, and even if link is fading fast) to the source cell (to be able to receive legacy HO command), then does it make sense for the UE to have the possibility to send a CHO “bye” message to source node?

# Discussion

At RAN2#105bis, several contributions e.g. [3] proposed that if UE receives a legacy HO command whilst CHO is in progress then legacy HO command overrides CHO. As a result, the highlighted FFS in section 1 was agreed.

Observation 1: Link conditions notwithstanding, it is possible for the source cell to send legacy HO command to UE which has triggered CHO and is the process of executing the triggered CHO.

The consequence of the above assertion (validated during RAN2#105bis) is that it is possible for a UE that is executing CHO to receive L3 (RRC) signaling. If case be, then corollary should stand true as well in that it should be possible for UE to send L3 (RRC) signaling to the source cell whilst it is middle of CHO execution with a candidate cell.

Observation 2: Link conditions and capabilities notwithstanding, based on observation#1 it is also possible for the UE to send L3 (RRC) signaling messages to source cell while UE is in the process of executing the triggered CHO.

If above are undisputed, the question which begs an answer is whether and in what form does it make sense for UE to send a “bye” message prior to UE attempting RACH on candidate cell. At RAN2#105bis various options were called out:

- Contributions such as [4], [6] advocate a “bye” message sent from UE to source (eNB) cell. Upon receipt of which the source eNB can start SN status transfer and packet forwarding, as in legacy handover. Even though not explicitly stated we assume such a message to be an RRC-level message.

- Contributions such as [5] advocate a light-weight “bye” message sent from UE to source (eNB) cell on pre-configured resources (e.g. PUCCH or dedicated PRACH preamble) of the source eNB. We equate this to mean a message for which a maximum number of L1- retransmissions occur; but no L2 (HARQ)/L3 (RLC) retransmissions.

- Contributions such as [3] advocate not taking any action on the source link immediately prior to, during or after a successful RACH procedure w/ candidate cell. In this option, it is expected that the target node informs the source node (e.g. via PATH SWITCH or similar message) of the HO completion.

In our opinion it is important to first understand whether a “bye” message (regardless of whether its L3, L2 or L1- based) for the source link (UEsource eNB) is beneficial. A key tenet of CHO is the ability to reserve resources on multiple candidate cells in advance. Resources reserved (including a CFRA RACH) for a specific UE implies resources not available to some other UE(s). It stands to reason that the sooner the fate of such resources is known (either employ or discard) the better the expected system performance (spectral efficiency).

Observation 3: The sooner the source eNB can release unutilized resources across target cell(s) the better the expected system performance and utilization.

Any mechanism which aids achieving observation#3 is therefore warranted.

Then it becomes a question of gains vs pains:

- /c/ provides the least of pain as no new system functionality is expected. However, it also provides the least of gains. Source eNB doesn’t know of CHO execution until upon CHO completion. While the source eNB’s effort to perform resource cleanup on candidate target cell(s) remains the same, it does allow less(er) candidate target resources to be available for potential CHO and non-CHO UEs on the said target cell(s).

- /a/ can be implemented with minimal effort - restricted mostly to L3 and perhaps an L2 (if new RRC msg is used) but it may end up delaying CHO execution if L1 (PHY, RV based) + L2 (HARQ) + L3 (possibly RLC-based) retransmissions occur due to degrading source link quality.

- /b/ requires the most implementation effort (including RAN1 enhancements) but provides the most gains. One might argue that if source eNB starts the clean-up efforts for this UE on both itself and candidate cell(s) which UE didn’t end up using and if RACH w/ intended candidate cell fails then UE will not be allowed to execute this w/ other candidates either. What is important to understand is that RACH failures operate at a timescale much shorter than Xn interactions. Hence, we don’t expect this to be an issue which would warrant a solution at this time.

It is therefore the opinion of the authors that a light-weight “CHO bye” solution (with PHY enhancements) (similar to /b/) is worth pursuing i.e. adopting a “fire-and-forget” or “one-shot” CHO “L1 bye” message without HARQ should be considered. Such an L1 message could use PUCCH resource configuration and can be investigated by RAN1. Having information about the target cell (e.g. PCI) towards which the UE is executing the CHO can also be beneficial for the source eNB to perform actions such as faster initiation of resource cleanup on candidate target cell(s) other than the one UE is initiating CHO towards.

Conclusion 1: RAN2 should agree to defining a Condition Handover “Bye” procedure between UE and source eNB with the following characteristics:

    - It is a L1- only message sent without any retransmissions

    - It is triggered by the UE when a CHO triggering criterion has been satisfied (i.e. a triggering condition evaluates to true)

    - Upon sending of which, UE executes CHO procedures with target node

    - Optionally, allow the UE to provide target cell’s PCI towards which CHO is being executed

Note, this mechanism is assumed to be in addition to current HO behavior wherein upon successful completion of RACH procedure on target cell the target cell will inform the source cell of the CHO completion (details are FFS and for RAN3 to decide).

Conclusion 2: If conclusion#1 is agreeable, then source eNB behavior upon receipt of light-weight L1 CHO bye msg should be defined by RAN3. Actions include e.g. source eNB starting cleanup of reserved resources on other target cell(s), or leave it implementation dependent.

Conclusion 3: RAN2 should liaise with RAN1 and RAN3 to define functionality mentioned in Conclusion#1 and Conclusion#2. Draft LS is provided in [7].

# Conclusion

The following proposals are provided in this contribution:

Observation 1: Link conditions notwithstanding, it is possible for the source cell to send legacy HO command to UE which has triggered CHO and is the process of executing the triggered CHO.

Observation 2: Link conditions and capabilities notwithstanding, based on observation#1 it is also possible for the UE to send L3 (RRC) signaling messages to source cell while UE is in the process of executing the triggered CHO.

Observation 3: The sooner the source eNB can release unutilized resources across target cell(s) the better the expected system performance and utilization.

Conclusion 1: RAN2 should agree to defining a Condition Handover “Bye” procedure between UE and source eNB with the following characteristics:

    - It is a L1- only message sent without any retransmissions

    - It is triggered by the UE when a CHO triggering criterion has been satisfied (i.e. a triggering condition evaluates to true)

    - Upon sending of which, UE executes CHO procedures with target node

    - Optionally, allow the UE to provide target cell’s PCI towards which CHO is being executed

Note, this mechanism is assumed to be in addition to current HO behavior wherein upon successful completion of RACH procedure on target cell the target cell will inform the source cell of the CHO completion (details are FFS and for RAN3 to decide).

Conclusion 2: If conclusion#1 is agreeable, then source eNB behavior upon receipt of light-weight L1 CHO bye msg should be defined by RAN3. Actions include e.g. source eNB starting cleanup of reserved resources on other target cell(s), or leave it implementation dependent.

Conclusion 3: RAN2 should liaise with RAN1 and RAN3 to define functionality mentioned in Conclusion#1 and Conclusion#2. Draft LS is provided in [7].

# References

- RP-190272, New Work Item on even further Mobility enhancement in E-UTRAN

- R2-1905188, “Report from session on Rel-16 LTE Mobility Enhancements WI”, Nokia

- R2-1903443, “Discussion of conditional handover”, Intel

- R2-1903240, “Discussions on Conditional Handover Procedures”, MediaTek

- R2-1904068, “Additional aspects in conditional handover”, NEC

- R2-1904343, “Remaining Issues of LTE Conditional Handover”, CMCC

- R2-1906399, “LS on Maximum Number of Candidate Target Cells for Conditional Handover in E-UTRAN”, Charter Communications

PUCCH is nominally used to transmit uplink control information (UCI) such as HARQ ACK/NACK when configured/triggered to do so by the gNB. As PUCCH format to be employed is a function of message payload size, either existing PUCCH format(s) or a new PUCCH format could be employed. Details are under RAN1 purview.