# 3GPP TSG-RAN WG2 Meeting #106 	R2-1906446

Reno, Nevada, USA 13-17 May 2019

Agenda Item:	12.2.4, 12.1.4

Source: 	Qualcomm Incorporated

Title: 	Retransmission and fallback for data transmission on PUR

Document for:     Discussion/Decision

# Introduction

The WI description for additional MTC enhancements for LTE is given in [1]. The WI description for additional NB-IoT enhancements is given in [2]. One of the common objectives of both of the WIs is:

Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes

The QoS requirements for the UL data may be different depending on scenarios. Therefore, the PUR mechanism should be able to support retransmissions and fallback.

# Background

RAN1 has agreed the following (partial list) in RAN1#94 for both eMTC and NB-IoT enhancements [3], [4]:

Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

Furthermore, RAN1#94bis agreed the following (partial list of relevant agreements) [5] [6]:

Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

During RAN1#95, further progress was made (partial list) [7]:

- Agreement - For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect - an UL GRANT for retransmission on the MPDCCH, or - FFS: a NACK, or - FFS: no explicit ACK

During RAN1#96, further progress was made (partial list) [8]:

Agreement For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration. - PUR search space is the search space where UE monitors for MPDCCH - FFS: Whether PUR search space is common or UE specific

So far, while a general understanding within RAN2 has been that fallback and retransmission mechanisms will be needed, explicit agreements has not been made.

# Discussion

As shown in section 2 above, RAN1 has agreed to support fallback mechanism to RACH/EDT procedures for UL transmission in PUR.

Fallback mechanisms from UL transmission on PUR to EDT and legacy RACH procedures are required, and RAN1 already agreed to support it.

It is important to note that the procedures for (HARQ) retransmission and/or fallback to another scheme will depend significantly on whether one is dealing with RRC-configured UE-specific PUR or SIB-configured CBS PUR. This is because an RRC-configured UE-specific PUR may have an in-built mechanism for contention-resolution (and hence ACK/NACK the transmission) – for example, via a uniquely assigned UE-specific DMRS sequence or by allocating different time/frequency resources. On the contrary, corresponding procedures for SIB-configured resources may be more involved due to the possibility of multiple UEs selecting the same resource from a pool of resources that are broadcast.

The aspects to be considered for retransmission include how ACK/NACK is indicated by eNB for the UL transmission, what is the UE behaviour when neither ACK nor NACK is received, etc. RAN1#96b has agreed the following:

- Agreement - The UE monitors the MPDCCH for at least a time period after a PUR transmission - FFS: Details of the time period - FFS: UE behaviour if nothing is received in the time period - FFS: If and how often UE monitors MPDCCH after a PUR allocation in which it has not transmitted

UE may wait for a certain period within which it expects to receive any possible response from the eNB (explicit ACK/NACK or further grant in MPDCCH) before taking further action which is still FFS in RAN1. This waiting period may be similar to the RAR window in legacy random access (i.e., the UE starts monitoring after a certain time period and continues monitoring for ACK/NACK/retx UL grant for a certain duration until one is received). It can be signalled separately from legacy RAR window and possibly configured per CE level.

UE behaviour needs to be defined upon expiry of such waiting period. For example, the UE may be allowed to retransmit the UL data. How to select the UL resources used for retransmission needs to be specified, e.g., whether PUR can be used for retransmissions and, if so, whether the retransmission UL resources is same (i.e., as indicated by the same PUR) or different (i.e., UL resources specifically for retx) from original transmission.   Further, the retransmission may involve the UE using a power ramp-up i.e., different transmission attempts may use different—possibly increased transmit power.

On the other hand, UE may fallback to EDT/legacy RA upon expiry of the waiting period. This decision may be based on the number of retransmissions, the power-level used at the UE exceeding a certain threshold, etc.

In summary, it is clear that there are various aspects that need to be discussed and specified to support the retransmissions and fallback mechanisms.

Supporting retransmissions and fallback involves discussion on various aspects and scenarios.

While a number of these aspects are marked as FFS in RAN1 agreements, as shown in section 2 above, RAN1 has already agreed that upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK at least on MPDCCH, whereas upon unsuccessful decoding by eNB of a PUR transmission the UE can expect an UL GRANT for retransmission on the MPDCCH. This means at least a procedure similar to adaptive HARQ retransmission is already envisioned and agreed by RAN1.

At least a procedure similar to adaptive HARQ retransmission for PUR transmissions is already envisioned/agreed by RAN1.

Therefore, we propose that RAN2 take the following agreement as starting point for further discussion on this aspect:

Mechanisms for HARQ retransmissions for transmissions over PUR are supported.

Mechanisms for fallback to RACH/EDT procedures for transmissions over PUR are supported.

# Summary

In this contribution, we discussed some aspects of supporting quality of service for UL data transmission on preconfigured resources in IDLE mode. Based on the above discussion, we made the following observations:

Observation 1.	Fallback mechanisms from UL transmission on PUR to EDT and legacy RACH procedures are required, and RAN1 already agreed to support it.

Observation 2.	Supporting retransmissions and fallback involves discussion on various aspects and scenarios.

Observation 3.	At least a procedure similar to adaptive HARQ retransmission for PUR transmissions is already envisioned/agreed by RAN1.

Based on the discussion and observations, we propose:

Proposal 1.	Mechanisms for HARQ retransmissions for transmissions over PUR are supported.

Proposal 2.	Mechanisms for fallback to RACH/EDT procedures for transmissions over PUR are supported.

# References

- RP-181878, Revised WID: Additional MTC enhancements for LTE, Ericsson

- RP-181674, Revised WID: Additional enhancements for NB-IoT, Huawei

- R1-1809672, RAN1#94 Chairman’s notes of AI 6.2.1 Additional MTC Enhancements, Ad-hoc Chair (Samsung)

- R1-1809673, RAN1#94 Chairman’s notes of AI 6.2.2 Additional Enhancements for NB-IoT, Ad-hoc Chair (Samsung)

- R1-1811725, RAN1#94bis Chairman’s notes of AI 6.2.1 Additional MTC Enhancements, Ad-hoc Chair (Samsung)

- R1-1811726, RAN1#94bis Chairman’s notes of AI 6.2.2 Additional Enhancements for NB-IoT, Ad-hoc Chair (Samsung)

- R1-1813770, RAN1#95 Chairman's notes of AI 6.2.1 Additional MTC Enhancements, Ad-hoc chair (Samsung)

- R1-1903664, RAN1#96 Chairman's notes of AI 6.2.1 Additional MTC Enhancements, Ad-hoc chair (Samsung)