3GPP TSG-RAN WG2 Meeting #106	R2-1906499

# Reno, NV, USA, 13th - 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1047 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Paging Monitoring for SI Change Indication and PWS Notification in RRC Connected |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-05-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UEs in RRC_IDLE/INACTIVE monitor paging in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging. How RRC_IDLE paging monitoring occasion (MO) is associated to Tx-beam was addressed in TS 38.304, v15.3.0, sec. 7.1: “A PO is a set of 'S' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. The Kth PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB.” In RRC_CONNECTED, however, in case the active BWP is not the initial DL BWP, it is unclear how paging MO is associated with Tx-beam. Unlike data reception where UE-specific PDCCH search space is configured, RRC_CONNECTED UE has no idea where to find the paging MO corresponding to a specific SSB. For example, assume TCI shows QCL to SSB#3, UE has no idea where the paging MO corresponding to SSB#3 is. Notice that the firstPDCCH-MonitoringOccasionOfPO indicates the start of one beam-sweeping round of paging transmission, but not the SSB and paging MO association. Therefore, we suggest adding descriptions about SSB and paging MO association in RRC specification, similar to that in TS 38.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | <1> In Clause 5.2.2.2.2, clarify that the for SI change indication and PWS notification monitoring, the Kth PDCCH monitoring occasion in the PO corresponds to the Kth transmitted SSB. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: SI change indication and PWS notification Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, UE in RRC_CONNECTED may misunderstand the Tx beam used in the MO it is monitoring, and thus fails to receive SI change indication and PWS notification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE in RRC_CONNECTED may need to monitor all MOs in a PO for SI change indication and PWS notification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Change]

5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI (other than for ETWS and CMAS) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space on the active BWP to monitor paging.

For SI change indication and PWS notification monitoring, a paging occasion is a set of 'S' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. The Kth PDCCH monitoring occasion for paging in the paging occasion corresponds to the Kth transmitted SSB.

[End of Change]