3GPP TSG-RAN2 Meeting #106	R2-1906516

# Reno, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3987 | rev | 1 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the extended Time Alignment Timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Institute for Information Industry (III) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB-IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing value on Time Alignment Timer is not enough. The maximum value 10.24s besides the value of infinity would be too short in NB-IoT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.3.2, create an IE to expand Time Alignment Timer. - In clause 6.7.1, add parameter name. - In clause 6.7.3.1, add parameter name and reference to TS 36.321[6]. - In clause 6.7.3.2, add parameter name and reference to TS 36.321[6]. Impact Analysis: Impacted functionality: Time Alignment Timer for NB-IoT Inter-operability: 1. if the Network supports the change and the UE does not: Have inter-operability issues. 2. if the UE supports the change and the network does not: Have inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification refers to a parameter which does not exist in the presented reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.7.1, 6.7.3.1, 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

6.3.2	Radio resource control information elements

<omitted text>

–	TimeAlignmentTimer-Long

The IE TimeAlignmentTimer-Long is used to extendedly control how long the UE considers the serving cells belonging to the associated TAG to be uplink time aligned. Corresponds to the extended Timer for time alignment in TS 36.321 [6]. Value in number of sub-frames. Value sf20480 corresponds to 20480 sub-frames, sf40960 corresponds to 40960 sub-frames and so on.

TimeAlignmentTimer-Long information element

-- ASN1START

TimeAlignmentTimer-Long ::=					ENUMERATED {

sf20480, sf40960, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

Next change

6.7.1	General NB-IoT message structure

-- ASN1START

NBIOT-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

RRCConnectionReestablishmentReject,

SecurityModeCommand,

SecurityModeComplete,

SecurityModeFailure,

AdditionalSpectrumEmission,

ARFCN-ValueEUTRA-r9,

CellIdentity,

C-RNTI,

DedicatedInfoNAS,

DRB-Identity,

InitialUE-Identity,

IntraFreqBlackCellList,

IntraFreqNeighCellList,

maxBands,

maxCellBlack,

maxCellInter,

maxCellIntra,

maxFBI2,

maxFreq,

maxMultiBands,

maxPageRec,

maxPLMN-r11,

maxSAI-MBMS-r11,

maxSIB,

maxSIB-1,

MBMS-SAI-r11,

MBMS-SAI-List-r11,

MBMSSessionInfo-r13,

NextHopChainingCount,

PagingUE-Identity,

PLMN-Identity,

P-Max,

PowerRampingParameters,

PreambleTransMax,

PhysCellId,

Q-OffsetRange,

Q-QualMin-r9,

Q-RxLevMin,

ReestabUE-Identity,

RegisteredMME,

ReselectionThreshold,

ResumeIdentity-r13,

RRC-TransactionIdentifier,

RSRP-Range,

ShortMAC-I,

S-TMSI,

SystemInformationBlockType16-r11,

SystemInfoValueTagSI-r13,

T-Reordering,

TimeAlignmentTimer,

TimeAlignmentTimer-Long,

TMGI-r9,

TrackingAreaCode,

DataInactivityTimer-r14

FROM EUTRA-RRC-Definitions;

-- ASN1STOP

Next change

6.7.3.1	NB-IoT System information blocks

–	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

SystemInformationBlockType2-NB information element

-- ASN1START

SystemInformationBlockType2-NB-r13 ::=	SEQUENCE {

radioResourceConfigCommon-r13			RadioResourceConfigCommonSIB-NB-r13,

ue-TimersAndConstants-r13				UE-TimersAndConstants-NB-r13,

freqInfo-r13							SEQUENCE {

ul-CarrierFreq-r13						CarrierFreq-NB-r13			OPTIONAL,	-- Need OP

additionalSpectrumEmission-r13			AdditionalSpectrumEmission

},

timeAlignmentTimerCommon-r13			TimeAlignmentTimer,

multiBandInfoList-r13	SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	cp-Reestablishment-r14				ENUMERATED {true}				OPTIONAL		-- Need OP

]],

[[	servingCellMeasInfo-r14				ENUMERATED {true} 				OPTIONAL,		-- Need OR

cqi-Reporting-r14					ENUMERATED {true} 				OPTIONAL		-- Need OR

]],

[[	enhancedPHR-r15						ENUMERATED {true}		OPTIONAL,	-- Need OR

freqInfo-v1530						SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15

}	OPTIONAL,		-- Cond TDD

cp-EDT-r15							ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-r15							ENUMERATED {true}		OPTIONAL,	-- Need OR

timeAlignmentTimerCommon-r15		TimeAlignmentTimer-Long OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType2-NB field descriptions

additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], clause 6.2.4F.

cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT, see 5.3.3.1b.

cp-Reestablishment This field indicates if the NB-IoT UE is allowed to trigger RRC connection re-establishment when AS security has not been activated.

cqi-Reporting For FDD: This field indicates if downlink channel quality reporting in RRCConnectionReestablishmentRequest-NB, RRCConnectionRequest-NB and RRCConnectionResumeRequest-NB message is allowed.

enhancedPHR For FDD: This field indicates if the NB-IoT UE is allowed to report enhanced PHR in MSG3 as specified in TS 36.321 [6].

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1-NB, listed in the same order.

servingCellMeasInfo This field indicates if serving cell idle mode measurement reporting in RRCConnectionReestablishmentComplete-NB, RRCConnectionResumeComplete-NB and RRCConnectionSetupComplete-NB is allowed.

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the anchor carrier, see TS 36.211 [21].

timeAlignmentTimer-Long Indicates the extended value of the time alignment timer, see TS 36.321 [6].

ul-CarrierFreq For FDD: Uplink carrier frequency as defined in TS 36.101 [42], clause 5.7.3F. If operationModeInfo in the MIB-NB is set to standalone and the field is absent, the value of the carrier frequency is determined by the TX-RX frequency separation defined in TS 36.101 [42], table 5.7.4-1, and the value of the carrier frequency offset is 0. If operationModeInfo in the MIB-NB is not set to standalone, the field is mandatory present. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

up-EDT For FDD: This field indicates whether the UE is allowed to initiate UP-EDT, see 5.3.3.1b.

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

Next change

6.7.3.2	NB-IoT Radio resource control information elements

<omitted text>

MAC-MainConfig-NB

The IE MAC-MainConfig-NB is used to specify the MAC main configuration for signalling and data radio bearers.

MAC-MainConfig-NB information element

-- ASN1START

MAC-MainConfig-NB-r13 ::=			SEQUENCE {

ul-SCH-Config-r13					SEQUENCE {

periodicBSR-Timer-r13				PeriodicBSR-Timer-NB-r13		OPTIONAL,	-- Need ON

retxBSR-Timer-r13					RetxBSR-Timer-NB-r13

}																	OPTIONAL,	-- Need ON

drx-Config-r13						DRX-Config-NB-r13				OPTIONAL,	-- Need ON

timeAlignmentTimerDedicated-r13		TimeAlignmentTimer,

logicalChannelSR-Config-r13			CHOICE {

release								NULL,

setup								SEQUENCE {

logicalChannelSR-ProhibitTimer-r13	ENUMERATED {

pp2, pp8, pp32, pp128, pp512,

pp1024, pp2048, spare}

}

}																	OPTIONAL,	-- Need ON

...,

[[	rai-Activation-r14						ENUMERATED {true}			OPTIONAL,	-- Need OR

dataInactivityTimerConfig-r14	CHOICE {

release								NULL,

setup								SEQUENCE {

dataInactivityTimer-r14				DataInactivityTimer-r14

}

}																OPTIONAL	-- Need ON

]],

[[	drx-Cycle-v1430					ENUMERATED {

sf1280, sf2560, sf5120, sf10240}	OPTIONAL	-- Need ON

]],

[[	ra-CFRA-Config-r14				ENUMERATED {true}				OPTIONAL	-- Need ON

]],

[[	timeAlignmentTimerDedicated-r15		TimeAlignmentTimer-Long		OPTIONAL	-- Need ON

]]

}

PeriodicBSR-Timer-NB-r13 ::=		ENUMERATED {

pp2, pp4, pp8, pp16, pp64, pp128, infinity, spare}

RetxBSR-Timer-NB-r13 ::=			ENUMERATED {

pp4, pp16, pp64, pp128, pp256, pp512, infinity, spare}

DRX-Config-NB-r13 ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

onDurationTimer-r13					ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, spare},

drx-InactivityTimer-r13				ENUMERATED {

pp0, pp1, pp2, pp3, pp4, pp8, pp16, pp32},

drx-RetransmissionTimer-r13			ENUMERATED {

pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24,

pp33, spare7, spare6, spare5,

spare4, spare3, spare2, spare1},

drx-Cycle-r13						ENUMERATED {

sf256, sf512, sf1024, sf1536, sf2048, sf3072,

sf4096, sf4608, sf6144, sf7680, sf8192, sf9216,

spare4, spare3, spare2, spare1},

drx-StartOffset-r13					INTEGER (0..255),

drx-ULRetransmissionTimer-r13		ENUMERATED {

pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24,

pp33, pp40, pp64, pp80, pp96,

pp112, pp128, pp160, pp320}

}

}

-- ASN1STOP

MAC-MainConfig-NB field descriptions

drx-Config Used to configure DRX as specified in TS 36.321 [6].

drx-Cycle longDRX-Cycle in TS 36.321 [6]. The value of longDRX-Cycle is in number of sub-frames. Value sf256 corresponds to 256 sub-frames, sf512 corresponds to 512 sub-frames and so on. In case drx-Cycle-v1430 is signalled, the UE shall ignore drx-Cycle-r13.

drx-StartOffset drxStartOffset in TS 36.321 [6]. Value is in number of sub-frames by step of (drx-cycle / 256).

drx-InactivityTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

drx-RetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

drx-ULRetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

logicalChannelSR-ProhibitTimer Timer used to delay the transmission of an SR. See TS 36.321 [6]. Value in number of PDCCH periods. Value pp2 corresponds to 2 PDCCH periods, pp8 corresponds to 8 PDCCH periods and so on.

periodicBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of PDCCH periods. Value pp2 corresponds to 2 PDCCH periods, pp4 corresponds to 4 PDCCH periods and so on.

ra-CFRA-Config Activation of contention free random access (CFRA), see TS 36.321 [6].

rai-Activation Activation of release assistance indication (RAI) in TS 36.321 [6].

retxBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of PDCCH periods. Value pp4 corresponds to 4 PDCCH periods, pp16 corresponds to 16 PDCCH periods and so on.

onDurationTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

timeAlignmentTimer Indicates the value of the time alignment timer, see TS 36.321 [6].

timeAlignmentTimer-Long Indicates the extended value of the time alignment timer, see TS 36.321 [6].

End of changes