3GPP TSG-RAN2 Meeting #106	R2-1906597

# Reno, USA, 13 - 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0768 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to mobility state thresholds |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | REL-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a descrepancy between the definition of medium/high mobility state threshold in 36.331 and 36.304. In 36.331 the medium/high mobility state threshold is defined as the number of cell changes to enter medium/high mobility state. In 36.304 the medium/high mobility state threshold is defined as the “maximum” number of cell reselections to enter medium/high mobility state. NCR_M and NCR_H are normal thresholds for which no maximum is applied. The criteria to enter medium/high mobility state are defined in section 5.2.4.3 (state detection criteria), i.e. dependent on the number of cell reselections during time period TCRmax and no maximum is used: Medium-mobility state criteria: - If number of cell reselections during time period TCRmax exceeds NCR_M and not exceeds NCR_H High-mobility state criteria: - If number of cell reselections during time period TCRmax exceeds NCR_H |  |  |  |  |  |  |  |  |
| Summary of change: |  | The word “maximum” is removed from the medium/high mobility state threshold. Impact Analysis Impacted functionality: UE mobility state Inter-operability: There is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is a descrepancy between the definition of medium/high mobility state threshold in 36.331 and 36.304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.304 CR 0124 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 5.2.4.7.1	Speed dependant reselection parameters

TCRmax

This specifies the duration for evaluating allowed amount of cell reselection(s).

NCR_M

This specifies the number of cell reselections to enter Medium-mobility state.

NCR_H

This specifies the number of cell reselections to enter High-mobility state.

TCRmaxHyst

This specifies the additional time period before the UE can enter Normal-mobility state.

Speed dependent ScalingFactor for Qhyst

This specifies scaling factor for Qhyst in sf-High for High-mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionNR

This specifies scaling factor for TreselectionNR in sf-High for High-mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionEUTRA

This specifies scaling factor for TreselectionEUTRA in sf-High for High-mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionUTRA

This specifies scaling factor for TreselectionUTRA in sf-High for High-mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionGERA

This specifies scaling factor for TreselectionGERA in H sf-High for High-mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionCDMA_HRPD

This specifies scaling factor for TreselectionCDMA_HRPD in sf-High for High mobility state and sf-Medium for Medium-mobility state

Speed dependent ScalingFactor for TreselectionCDMA_1xRTT

This specifies scaling factor for TreselectionCDMA_1xRTT in sf-High for High mobility state and sf-Medium for Medium-mobility state

<End of modified section>