3GPP TSG-RAN2 Meeting #106	R2-1906599

Reno, USA, 13 - 17 May 2019	Revision of R2-1904141

Agenda Item:	10.4.5.2 Cell selection/reselection

Source: 	Ericsson

Title:  	P-Max applicable in FR2

Document for:	Discussion and Decision

# Introduction

The P-Max IE applicable in FR2 is discussed in this contribution.

# Background

RAN4

RAN4 discussed the applicability of P-Max in FR2, but did not reach an agreement, i.e. the CRs were postponed [1-3].

RAN2

Idle mode:

The P-Max IE is used in SIB1 to indicate the maximum power on NUL or SUL carrier, and in SIB2 on an intra-frequency carrier, and in SIB4 on an inter-frequency carrier. The P-Max IE is also used for the additionalPmax IE (OP) in the NS parameters for the MFBI signalling.

The P-Max IE is used for the Pcompensation in cell selection criterion S, as part of the cell suitability, for the serving, intra-frequency neighbour or inter-frequency neighbour cell(s).

Connected mode:

The P-Max IE is used in several RRC messages to configure the maximum uplink power on NUL or SUL carrier as part of the configuration CellGroupConfig IE.

The P-Max IE in 38.331 has the following value range:

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, in TS 38.101-1 [15] and is used to calculate the parameter Pcompensation defined in TS 38.304 [20].

P-Max information element

P-Max ::=                           INTEGER (-30..33)

Correction to the P-Max value range for FR2 was discussed in RAN2#104 [4], but postponed, due to lack of progress in RAN4.

# Discussion

P-Max in FR1:

RAN4 agreed corrections to 38.101-1 which specify that if P-Max is present in system information the UE uses min (P-Max, PowerClass), otherwise the UE uses PowerClass [5]. This principle applies to all NR bands in FR1. RAN4 has defined Power Class 2 (26 dBm) and Power Class 3 (23 dBm) in FR1, see table Table 6.2.1-1 in 38.101-1.

P-Max in FR2:

The NW should be able to limit the maximum output power for NR UE in FR2 by signalling an explicit P-Max value in system information. Even though RAN4 has not decided on the applicable P-max in FR2 (i.e. use of TRP or EIRP), it is likely that the required signalling range is outside the signalling range of P-Max IE, see tables 6.2.1.x-2 in 38.101-2 for Power Class 1, 2, 3 and 4 defined for FR2:

| Power Class | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| 1 | 35 | 55 |
| 2 | 23 | 43 |
| 3 | 23 | 43 |
| 4 | 23 | 43 |

RAN4 could not agree on the applicable metric (TRP or EIRP) for FR2. It is assumed that RAN4 will discuss this topic again in RAN4#91 [6]. In case RAN4 is not able to agree on an applicable P-Max for FR2 in REL-15, RAN2 needs to clarify the use of P-Max/additionalPmax in 38.331 and Pcompensation in 38.304, i.e. RAN2 needs to discuss the impact on RAN2 specifications:

Observation 1: In case RAN4 does not agree on an applicable P-Max for FR2 in REL-15 then RAN2 needs to discuss the impact on RAN2 specifications.

However RAN4 may be able to agree on an applicable P-Max for REL-15, provided that waivers and a transition period is applied for the associated test cases:

Observation 2: RAN4 may be able to agree on an applicable P-Max for REL-15, provided that waivers and a transition period is applied for the associated test cases.

In RAN4#90bis it was discussed to introduce P-Max in REL-15 as an optional feature, because REL-15 are already out in the field [1].

It is possible that RAN4 agrees on a single P-Max value for the new power classes in FR2 [6]. In such case the existing P-Max IE can be re-used for FR2 without ASN.1 change when an offset is added to the signalled value in 38.101-2:

Observation 3: P-Max IE can be re-used when RAN4 agrees on a single P-Max value for the new power classes in FR2 and an offset (dB) is added to the signalled value in 38.101-2.

This is probably the simplest and cleanest solution to support the new power classes in FR2. The offset would be captured in 38.101-2 because RAN4 anyways has to capture the applicable P-Max for FR2 and RAN4 would also have to agree on the offset value to use [6].

RAN2 has to wait for the outcome of the RAN4 discussions, but independent from the outcome of the RAN4 discussions, there is an impact on the RAN2 specifications:

Proposal 1: RAN2 to wait for RAN4 agreements concerning the applicable P-Max in FR2.

Proposal 2: RAN2 to discuss clarifications for P-Max in FR2.

For discussion sake a CR to 38.331 is provided that assumes that an offset is added to the signalled value for P-Max in FR2 in 38.101-2 [7].

# Summary

RAN2 is kindly asked to discuss signalling the P-max IE applicable in FR2:

Observation 1: In case RAN4 does not agree on an applicable P-Max for FR2 in REL-15 then RAN2 needs to discuss the impact on RAN2 specifications.

Observation 2: RAN4 may be able to agree on an applicable P-Max for REL-15, provided that waivers and a transition period is applied for the associated test cases.

Observation 3: P-Max IE can be re-used when RAN4 agrees on a single P-Max value for the new power classes in FR2 and an offset (dB) is added to the signalled value in 38.101-2.

Proposal 1: RAN2 to wait for RAN4 agreements concerning the applicable P-Max in FR2.

Proposal 2: RAN2 to discuss clarifications for P-Max in FR2.

# References

- R4-1903668, Introducing P-max in FR2, DISC, DOCOMO, RAN4#90bis

- R4-1903669, draft CR for introduction of P-max to FR2, draftCR 38.101-2, DOCOMO, RAN4#90bis

- R4-1903961, Introduction of P-Max for FR2, draftCR 38.101-2, Ericsson, RAN4#90bis

- R2-1817149, Correction to P-Max value range for FR2, Ericsson, CR 38.331, RAN2#104

- R4-1813813, Draft CR for TS 38.101-1: P-Max for 5G NR HPUE, CMCC, CR 38.101-1, RAN4#88bis

- R4-190xxxx, Introduction of P-Max for FR2, Ericsson, CR 38.101-2, RAN4#91

- R2-1906596, Correction for P-Max value range in FR2, Ericsson, CR 38.331, RAN2#106