3GPP TSG-RAN WG2 Meeting #106                                                             	R2-1906644

Reno, USA, May 13th - 17th, 2019                                             	Revision of R2-1903352

Source:	CATT

Title:	Handling of Conditional Handover Failure

Agenda Item:	12.3.3.1

Document for:	Discussion and Decision

Introduction

In the RAN2#104 meeting, the agreement about conditional handover configuration was made as following:

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

From the agreement, network may configure more than one candidate cells for UE to choose one to access. Considering more than one cells prepared to allow the UE access, how to handle the candidate cells by the UE needs to be discussed.

We propose a timer based handover failure procedure to be reused in CHO, and propos the UE to initiate another handover to available suitable candidate cells instead of RRC reestablishment upon handover (CHO or normal handover) failure.

Discussion

- Normal handover

In current handover procedure, the source cell sends the handover command to the UE which includes only one target handover cell. In the handover command, the target cell provides the configuration for the UE, and provides timer T304 to control the handover procedure. Upon the reception of the handover command, the UE is configured according to the handover command and starts timer T304, if the T304 is expired, the UE will consider handover failure and initiates the connection re-establishment procedure.

Observation 1: In normal handover procedure, only one target handover cell is configured for the UE, and the UE will initiate RRC connection reestablishment procedure upon T304 expiry.

- Conditional handover

Different with normal handover, the network may configure more than one candidate cells for the UE as the target handover cells in conditional handover. If the network configures more than one candidate cells for UE, each candidate cell will save the configuration and reserves dedicated resources for the UE. Upon the reception of conditional handover command, the UE will not perform handover immediately. The UE initiates the handover to one candidate cell upon the channel quality of this cell meets the handover execution condition.

Observation 2: More than one candidate cells may be configured for the UE in conditional handover.

- Handling of handover failure

For CHO, when UE successfully sends the CHO complete message to the target cell, the CHO procedure is successful. Similar with the normal handover, a criterion should be specified to define CHO failure. Timer based handover failure procedure used in normal handover can be reused for CHO. A timer like T304 can be used to control handover failure for CHO. When the UE decides to access to the candidate cell upon this cell meets the CHO execution condition, the UE starts the timer. The UE stops the timer upon UE successfully sends the CHO complete message to this target cell. Upon expiry of the timer, the UE considers the handover to this candidate cell is failure.

Proposal 1: Timer based handover failure procedure (like T304) should be reused in CHO. UE starts the timer when UE decides to access to one target candidate cell, and stops the timer when UE successfully sends the CHO complete message. Upon expiry of the timer, UE considers handover to this candidate cell is failure.

If the UE is configured with at least one candidate cells, for the first time that the UE tries to access to one candidate cell, if the handover to this candidate cell is failed, or before the CHO is executed, legacy handover is performed and the legacy handover is failure, which procedure should be triggered needs to be discussed. Considering the source cell and other candidate cells excluding the first target cell save the UE configuration, the re-establishment procedure may not be the optimality policy for conditional handover failure.

- Initiation Reestablishment

If the conventional handover failure handling is used for conditional handover, that is when the UE fails to access the target cell, the UE will initiate RRC reestablishment procedure. An example of the procedure is as the following figure shown:

Figure 1 first handover attempt failure trigger RRC reestablishment

Random access with contention and context fetch procedure will be performed for RRC re-establishment, even for the reestablishment successful procedure, three RRC signaling should be used.

- Initiation access to other candidate cell

Excluding the serving cell and the attempted target cell, there are other candidate cells that have the configuration of the UE. If the UE initiates access to these candidate cells according to handover procedure, the timer of context fetch from source cell will be avoided. An example is shown in the figure:

Figure 2 first handover attempt failure with handover to other candidate cell

As figure 2 shown, only one RRC signaling message is needed to access the target cell according to handover procedure, and the contention free random access procedure can be triggered if the dedicated RA configuration is configured by the target candidate cell. Comparing with reestablishment to other cells, re-initiation of handover to other candidate cells will reduce the latency of RRC connection and data transmission.

Observation 3: It will reduce the latency of RRC connection and data transmission if UE access to other candidate cells upon the failure of handover attempt to one target cell comparing with initiation of RRC reestablishment.

For conditional handover, the configuration of the multiple candidate cells should be taken full advantage. Upon the CHO handover to one candidate cell is failure, or upon the failure of the normal handover, the UE should take priority of the candidate cells than other cells to achieve fast recovery. If channel quality of the available candidate cell is suitable to access, such as the candidate cell meet the cell reselection criterion or the CHO execution condition or other specified condition which can be discussed further, the UE should initiate another handover attempt to the candidate cell met the condition instead of RRC re-establishment procedure. If none of the candidate cell is suitable to access, the UE should initiate RRC reestablishment procedure. The details of the mechanism should be discussed further.

Proposal 2: UE should initiate another handover attempt to available suitable candidate cells instead of RRC reestablishment upon handover (CHO or normal handover) failure, excluding none of the candidate cell is suitable to access.

Proposal 3: The criterion to evaluate whether the candidate cell is suitable to access can be further discussed.

Conclusion

In this contribution, we propose:

Observation 1: In normal handover procedure, only one target handover cell is configured for the UE, and the UE will initiate RRC connection reestablishment procedure upon T304 expiry.

Observation 2: More than one candidate cells may be configured for the UE in conditional handover.

Observation 3: It will reduce the latency of RRC connection and data transmission if UE access to other candidate cells upon the failure of handover attempt to one target cell comparing with initiation of RRC reestablishment.

Proposal 1: Timer based handover failure procedure (like T304) should be reused in CHO. UE starts the timer when UE decides to access to one target candidate cell, and stops the timer when UE successfully sends the CHO complete message. Upon expiry of the timer, UE considers handover to this candidate cell is failure.

Proposal 2: UE should initiate another handover attempt to available suitable candidate cells instead of RRC reestablishment upon handover (CHO or normal handover) failure, excluding none of the candidate cell is suitable to access.

Proposal 3: The criterion to evaluate whether the candidate cell is suitable to access can be further discussed.

Reference

- RAN2-104-Spokane-R16 feMOB_2018-11-14_final