3GPP RAN WG2#106    																					R2-1906665

Reno, USA, 13th – 17th May 2019

Agenda item:		10.4.4

Source:		Intel Corporation

Title:			Update of L2/3 feature lists

Document for:	Discussion and decision

It is to update NR L2/3 feature list accroding to the decisions made at RAN2#105bis and to ask RAN2 to discuss remaining TBD issues. Note if new UE capabilities for SA or late drop are decided, it is indicated in the column of “Note” indicating it is for SA or late drop. Also note E-UTRA side capabilities are not included here.

In the endorsed L2/3 feature lists from RAN2#105bis [R2-1904464, Intel], there are remaining TBD issues for IMS voice over SCG bearer of NE-DC which was introduced for late drop recently. However there was no clear discussion/decision on whether this capability is mandatory/optional or it needs TDD/FDD or FR1/FR2 separation. We think voiceOverEUTRA-5GC capability in NR or ims-VoiceOverMCG-BearerEUTRA-5GC capability in LTE should be baseline to handle TBDs of IMS voice over SCG bearer of NE-DC. So the following proposal is made for mandatory/optional, FDD/TDD and FR1/FR2 separation of IMS voice over SCG bearer of NE-DC

[Proposal1]: It is proposed IMS voice over SCG bearer of NE-DC is optional without the need of TDD/FDD and FR1/FR2 separation.

With the above proposal1, it is also proposed to ask RAN2 to endorse the updated L2/3 feature lists in the table below.

[Proposal2]: RAN2 is asked to endorse the updated L2/3 feature lists in the table below.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if eNB/gNB does not know?) | Consequences if the feature is not supported by the UE | Type (see R2-1712078) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. General (including supported bearer types) | 0-0 | Basic EN-DC procedures | 1) MCG DRB with LTE/NR PDCP 2) SCG DRB with NR PDCP 3) SN addition, modification, and release via RRC connection reconfiguration 4) Joint processing on the combined RRC messages 5) Failure handling (including both MN and SN) |  | No |  |  |  |  |  |  | RAN2 | 1) to 5): Mandatory without capability signaling. |  |
|  | 0-1 | Access stratum release | Access stratum release |  | Yes | Configuration specific to a release is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is {Rel-15, spare7, … , spare1} |  |
|  | 0-2 | SRB | 1) Split SRB with one UL path 2) SRB3 |  | Yes | 1) Configuration of split SRB with one UL path is not possible 2) Configuration of SRB3 is not possible | Type 4 | Yes | No |  | 2) Not applied to NE-DC. | RAN2 |  | 1) Optional with capability signaling 2) Mandatory with capability signaling |
|  | 0-3 | DRB | 1) Maximum number of DRBs 2) Split DRB with one UL path 3) Split DRB with both UL MCG and SCG paths |  | Yes | 1) Configuration of DRB may be failed 2) Configuration of split DRB with one UL path is not possible 3) Configuration of split DRB with both UL MCG and SCG paths is not possible | Type 4 | Yes | No |  | 2) 8 DRBs are supported regardless of bearer types | RAN2 | 1, 2) Mandatory without UE capability signaling | 3) Mandatory with capability signaling |
|  | 0-4 | Direct SN addition in the first RRC connection reconfiguration after RRC connection establishment | Direct SN addition in the first RRC connection reconfiguration after RRC connection establishment |  | Yes | SN addition in the first RRC connection reconfiguration may be failed | Type 4 |  |  |  |  | RAN2 | Mandatory without UE capability signaling |  |
|  | 0-5 | IMS voice | 1) IMS voice over NR 2) Fallback HO to LTE for IMS voice 3) 5GC VoLTE 4) IMS voice over SCG bearer of NE-DC |  | Yes | 1) IMS voice over NR is not supported 2) Fallback HO to LTE for IMS voice is not supported 3) gNB may not decide whether EPS fallback or RAT fallback is needed. 4) IMS voice over SCG bearer of NE-DC is not supported | Type 4 | 1), 3): No 4): No | 1): Yes 3): No 4): No |  | 1) SA only 2) SA only 3) SA only 4) NE-DC only | RAN2 | 3) Optional with capability signaling 4) Optional with capability signaling | 1) Mandatory with capability signaling if UE is IMS voice capable in NR SA. Otherwise optional with capability signaling. 2) No need for a separate capability signaling. |
|  | 0-6 | Codec adaptation and delay budget reporting | 1) Delay budget reporting 2) Bit rate recommendation message 3) Bit rate recommendation query message |  | Yes | RAN-assisted codec adaptation and delay budget reporting is not possible. | Type 4 | No | No |  | 1), 2), and 3): SA only. | RAN2 | 1), 2), and 3): Optional with capability signaling |  |
|  | 0-7 | PCell operation | 1) PCell operation on FR2 |  | Yes | gNB may not know if the UE supports relevant features on FR2. | Type 4 | No | No |  | SA only | RAN2 | Mandatory with capability signaling |  |
|  | 0-8 | Overheating | 1) Overheating assistance information |  | Yes | gNB may not know if UE can support overheating assistance information, so network may not set appropriate configuration under overheating problem. | Type 4 | No | No |  | SA only | RAN2 | Optional with capability signaling |  |
|  | 0-9 | V2X | 1) Support of EUTRA V2X |  | Yes | The support of EUTRA V2X by the UE configured with EN-DC would not be guaranteed. | Type 4 | Yes | No |  | 1) Only applied to EN-DC. | RAN2 | Optional with capability signaling |  |
|  | 0-10 | Architecture options | 1) Support of NE-DC |  | Yes | gNB may not know if UE can support NE-DC, so network may not configure NE-DC related configurations for the UE. | Type 4 | No | No |  | 1) Only applied to NE-DC. Note for EN-DC, it is included in EUTRA side. | RAN2 | Optional with capability signaling |  |
| 1. PDCP | 1-0 | Basic PDCP procedures | 1) (de)Ciphering on DRB/SRB 2) Integrity protection on SRB 3) Timer based SDU discard 4) Re-ordering and in-order delivery 5) Status reporting 6) Duplicate discarding 7) 18bits SN |  | No |  |  |  |  |  |  | RAN2 | 1) to 7): Mandatory without capability signaling |  |
|  | 1-1 | ROHC context | 1) Maximum number of ROHC context sessions 2) Supported ROHC profiles |  | Yes | Configuration of ROHC context sessions may be failed | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is: 1) {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1} 2) {0x0000, 0x0001, 0x0002, 0x0003, 0x0004, 0x0006, 0x0101, 0x0102, 0x0103, 0x0104} |  |
|  | 1-2 | ROHC context continuation operation | ROHC context continuation operation |  | Yes | Configuration of DRB ROHC continuation is not possible. | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 1-3 | Uplink only ROHC Profiles | Uplink only ROHC Profiles |  | Yes | Configuration of uplink only ROHC operation is not possible. | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 1-4 | Out of order delivery | Out of order delivery |  | Yes | Configuration of out of order delivery is not possible | Type 4 | No | No |  |  | RAN2 |  | Optional with capability signaling |
|  | 1-5 | Short SN | Short SN |  | Yes | Configuration of short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 1-6 | PDCP duplication | 1) PDCP duplication for split SRB1/2 2) PDCP duplication for SRB1/2 and/or SRB3 3) PDCP duplication for MCG or SCG DRB 4) PDCP duplication for split DRB |  | Yes | Configuration of PDCP duplication for the associated SRB or DRB is not possible. | Type 4 | No | No |  |  | RAN2 | 1), 2), 3) and 4): Optional with capability signaling |  |
|  | 1-7 | DRB IP data rate | 1) DRB IP data rate in DL 2) DRB IP data rate in UL |  | Yes | MAC PDU may not be processed or generated within a processing time requirement. |  |  |  |  |  |  | Optional capability is signaled by NAS signaling defined in 24.501 |  |
| 2. RLC | 2-0 | Basic RLC procedures | 1) RLC TM 2) RLC AM with 18bits SN* 3) SDU discard Note*: no separate feature is considered for t-PollRetransmit, t-Reassembly and t-StatusProhibit |  | No |  |  |  |  |  |  | RAN2 | 1) to 3): Mandatory without capability signaling. |  |
|  | 2-1 | RLC AM with short SN | RLC AM with short SN |  | Yes | Configuration of RLC AM with short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 2-2 | RLC UM with short SN | RLC UM with short SN |  | Yes | Configuration of RLC UM with short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 2-3 | RLC UM with long SN | RLC UM with long SN |  | Yes | Configuration of RLC UM with long SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 2-4 | NR RLC SN size for SRB | NR RLC SN size for SRB |  | No | Long NR RLC SN size is not possible. |  |  |  |  |  | RAN2 | RAN2 decided only short RLC SN is used for SRB. |  |
| 3. MAC | 3-0 | Basic MAC procedures | 1) RA procedure on PCell or PSCell (in case of EN-DC) 2) UE initiated RA procedure (including for beam recovery purpose) 3) NW initiated RA procedure (i.e. based on PDCCH) 4) Support of ssb-Threshold and association between preamble/PRACH occasion and SSB 5) Preamble grouping 6) UL single TA maintenance 7) HARQ operation for DL and UL 8) LCH prioritization 9) Prioritized bit rate 10) Multiplexing 11) SR with single SR configuration 12) BSR 13) PHR 14) 8bits and 16bits L field |  | No |  |  |  |  |  |  | RAN2 | 1) to 14): Mandatory without capability signaling |  |
|  | 3-1 | LCP restriction | 1) LCP restriction 2) LCP restriction to SCell (s) |  | Yes | Configuration of mapping restrictions for each LCH is not possible | Type 4 | No | No |  |  | RAN2 | 1) Optional with capability signaling 2) Optional with capability signaling |  |
|  | 3-2 | LCH SR delay timer | LCH SR delay timer |  | Yes | Configuration of LCH SR delay timer is not possible | Type 4 | Yes | No |  |  | RAN2 |  | Optional with capability signaling |
|  | 3-3 | DRX | 1) DRX with long DRX cycle 2) DRX with short DRX cycle |  | Yes | Configuration of DRX is not possible | Type 4 | Yes | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 3-4 | Configured grants | Maximum number of configured grant configurations per cell group |  | Yes | Configuration of configured grant may be failed | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-5 | SR | Multiple SR configurations |  | Yes | Configuration of per cell group is not possible | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-6 | Skipping UL transmission | 1) Skipping UL transmission for dynamic UL grant 2) Skipping UL transmission for configured UL grant |  | Yes | Configuration of skipping UL transmission is not possible | Type 4 | 1) Yes 2) No | No |  |  | RAN2 | 2) Conditional mandatory if the UE supports configured grant | 1) Optional with capability signaling. Mandatory with capability signaling from Rel-16 |
| 4. Measurements | 4-1 | Intra-NR measurements and reports | 1) Intra-frequency and inter-frequency measurements and reports 2) Event A-based measurement and measurement report |  | Yes | 1) Configuration for intra-frequency and/or inter-frequency is/are not possible 2) Configuration for event A-based measurement is not possible | Type 4 | Yes | No |  |  | RAN2 | 1) and 2): Mandatory with capability signaling when EN-DC is configured. Mandatory without capability signaling for NR SA. |  |
|  | 4-2 | Inter-NR measurement and reports while in LTE connected | 1) NR measurement and reports while in LTE connected 2) Event B1-based measurement and reports while in LTE connected |  | Yes | 1) Configuration for NR measurement is not possible 2) Configuration for event B1-based measurement is not possible | Type 4 |  |  |  |  | RAN2 | Mandatory without capability signaling |  |
|  | 4-3 | SFTD measurements | 1) SFTD measurements between PCell and PSCell 2) SFTD measurements between PCell and NR Cell |  | Yes | Configuration for SFTD is not possible | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 4-4 | Measurement gaps | Additional measurement gap configurations |  | Yes | Configuration for additional measurement gap is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is: BIT STRING (SIZE (22)) | Optional with capability signaling |
|  | 4-5 | ANR | 1) CGI reporting of EUTRA cell when EN-DC is not configured 2) CGI reporting of NR cell when EN-DC is not configured 3) CGI reporting of NR cell when EN-DC is configured |  | Yes | Configuration for ANR is not possible | Type 4 | No | No |  | 1) and 2): SA only 3): EN-DC only | RAN2 | 1), 2) and 3): Mandatory with capability signaling Note autonomous gap is not supported when ANR (towards NR neighbor cells) configured by NR PCell in NR SA and when ANR (towards NR neighboring cells) configured by NR PSCell in EN-DC. |  |
|  | 4-6 | LTE measurement and reporting while in NR connected | 1) LTE measurement and reporting while NR connected. 2) Event B#N-based measurement and reporting while NR connected |  | Yes | 1) Configuration for LTE measurement is not possible 2) Configuration for event B#N-based measurement is not possible. | Type 4 | No | No |  | SA only | RAN2 | 2) Mandatory with capability signaling if the UE supports LTE | 1) Mandatory with capability signaling if the UE supports LTE. |
| 5. SDAP | 5-1 | QoS | 1) Flow-based QoS 2) Multiple flows to 1 DRB mapping 3) AS reflective QoS |  | Yes | Appropriate configuration for SDAP may not be possible. | Type 4 | No | No |  | SA only | RAN2 | 1), 2): Mandatory without capability signaling | 3) Optional with capability signaling. |
|  | 5-2 | HD format | 1) DL SDAP HD 2) UL SDAP HD 3) SDAP End-marker |  | Yes | Appropriate configuration for SDAP may not be possible. | Type 4 |  |  |  | SA only | RAN2 | 2), 3): Mandatory without capability signaling | 1) Conditional mandatory if either NAS reflective QoS or AS reflective QoS is supported. No capability signaling is needed. |
| 6. Inactive | 6-1 | RRC inactive | RRC inactive |  | Yes | Configuration for RRC inactive is not possible. | Type 4 | No | No |  | SA only | RAN2 | Mandatory with capability signaling |  |
| 7. Mobility | 7-1 | Handover | 1) Intra-frequency HO 2) Inter-frequency HO 3) HO between TDD and FDD 4) HO from NR to LTE 5) HO from NR to eLTE 6) HO between FR1 and FR2 |  | Yes | Associated HO is not supported. | Type 4 | 1), 3) and 6): No 2), 4) and 5): Yes | 1), 3) and 6): No 2), 4) and 5): Yes |  | SA only | RAN2 | 1): Mandatory without capability signaling 2): Mandatory with capability signaling 3): Mandatory with capability signaling if the UE supports both TDD and FDD. 4) and 5): Mandatory with capability signaling if the UE supports the associated RAT. 6) Mandatory with capability signaling if the UE supports both FR1 and FR2. |  |
| 8. Idle/inactive UE procedures | 8-1 | System information acquisition | 1) Msg.1 based on-demand SI provisioning 2) Msg.3 based on-demand SI provisioning |  | No | On-demand SI provisioning is not possible. |  |  |  |  | SA only | RAN2 | 1, 2): Mandatory without capability signaling |  |
| 9. RRC | 9-1 | RRC buffer size | 1) Maximum overall RRC configuration size |  | No |  |  |  |  |  |  | RAN2 | 1) 45 Kbytes |  |
|  | 9-2 | RRC processing time | 1) RRC connection establishment 2) RRC connection resume without SCell addition/release and SCG establishment/modification/release 3) RRC connection reconfiguration without SCell addition/release and SCG establishment/modification/release 4) RRC connection re-establishment. 5) RRC connection reconfiguration with sync procedure 6) RRC connection reconfiguration with SCell addition/release or SCG establishment/modification/release 7) RRC connection resume 8) Initial security activation 9) Counter check 10) UE capability transfer |  | No |  |  |  |  |  |  | RAN2 | 1) to 3): 10ms 4): 10ms 5): 10ms + additional delay (cell search time and synchronization) defined in TS 38.133 6): 16ms 7): 10 or 6ms (See details in 12, TS 38.331) 8) and 9): 5ms 10): 80ms |  |