3GPP TSG-RAN WG2#106	Tdoc R2-1906879

Reno, US, 13-17 May 2019

Agenda Item:	11.12.2

Source:	Ericsson

Title:	Differentiation of DL and UL causes at RLF occurrence

Document for:	Discussion, Decision

# Introduction

In RAN2 #105bis in Xi’an, identification of UL and DL cause of RLF was discussed, and the following was agreed;

Agreements:

1: 	Agree the use case that NR RLF Report can indicate the information to differentiate DL and UL availability after RLF occurrence. Solution is FFS.

2: 	For NR CEF Report is enhanced with further information elements expressing the number of failed connection setup attempts after RLF at least including the number and available location information.

In this contribution, we discuss how downlink and uplink causes for the radio link failure can be identified in different cases.

# Discussion

## Brief introduction on RLF triggering points and reporting mechanism

Like in LTE, the UE can declare RLF in NR when one of following conditions are met:

- Expiry of the radio link monitoring related timer T310;

- Expiry of the measurement reporting associated timer T312 (not receiving the handover command from the network within this timer’s duration despite sending the measurement report when T310 was running);

- Upon reaching the maximum number of RLC retransmissions;

- Upon receiving random access problem indication from the MAC entity;

Upon radio link failure due to one of the above-mentioned causes, the UE logs/stores the RLF report. The content of the RLF report is extensively discussed in [1]. UE indicates availability of the RLF report upon successful re-establishment procedure or once it comes back to the connected mode (in case of failure in re-establishment procedure). The existing mechanism to obtain an RLF report in the LTE system is the UE Information procedure in RRC as specified in section 5.6.5 of RRC specification 36.331. The UE may receive an UEInformationRequest containing a flag called rlf-ReportReq set to ‘true’. Upon that the UE sets the content of the information to be included in the RLF report, to be later transmitted to the network in the UEInformationResponse message, as shown below:

Figure 1 UE Information procedure

## Radio Link Failure in NR

It has been assumed that the RLF procedure and reporting in NR will be built on the LTE solution. The different RLF failure conditions described in section 2.1 can therefore be used to identify the cause for the RLF in NR. In the following, we consider the different failure conditions and how the network can learn whether the RLF cause was due to uplink or downlink problems in the different cases. Note that although T312 is not yet agreed in NR, it is expected to be available in future releases and is therefore considered as well in the below.

### Expiry of the radio link monitoring related timer T310

### The timer T310 is started for a UE connected to a cell when out-of-sync is detected for a specified number of times and is stopped again if the UE retrieves sync for a specified number of times, or when the UE receives a handover command. Upon expiry of T310 the UE declares RLF and reports expiry of T310 as the cause. If the network has sent a handover command the network can understand that the command did not reach the UE, and that the problem causing the RLF occurred due to a bad downlink.

Upon radio link failure with cause T310 expiry, the network can deduce that the radio link failure cause was related to downlink.

### Expiry of the measurement reporting associated timer T312

### The timer T312, if supported, is started for a UE connected where T310 is already running (i.e. UE has detected out-of-sync) and where a measurement report triggering event is fulfilled and the measurement reporting procedure is initiated. The timer is stopped again if the UE retrieves sync for a specified number of times, or when the UE receives a handover command. Upon expiry of T312 the UE declares RLF and reports expiry of T312 as the cause.

### If the network has received a measurement report and sent a handover command the network can understand that the command did not reach the UE, and that the problem causing the RLF occurred due to a bad downlink. (Note that in case of a bad uplink the network would not receive a measurement report from the UE, but RLF would instead be declared due to that the maximum number of retransmissions reached.)

Upon radio link failure with cause T312 expiry, the network can deduce that the radio link failure cause was related to downlink.

### Reaching the maximum number of RLC retransmissions

RLF can also be declared upon reaching the maximum number of RLC retransmissions. The RLF cause is then set to maximum number of RLC retransmissions and the network can understand that the failure was due to bad uplink.

Upon radio link failure with cause RLC maximum number of transmissions, the network can deduce that the radio link failure cause was related to uplink.

### Receiving random access problem indication from the MAC entity

When the handover command has reached the UE, a random-access procedure is started. The random-access procedure consists of the following steps:

- Random Access procedure initialization

- Random Access Resource selection

- Random Access Preamble transmission

- Random Access Response reception

- Contention Resolution

- Completion of the Random-Access procedure

In step 1, parameters are configured and carrier selected. In step 2, the UE listens to and selects an CSI-RS or SSB in order to select a random-access preamble to be used the following steps. In the following step 3, the UE transmits the preamble and in step 4 the random access response is received by the UE. In step 5, contention resolution is performed and in step 6 the random access procedure is completed.

The random-access problem indication triggering a RLF can be indicated from the MAC entity upon reaching the maximum number of preamble transmissions either in step 4 if the random access response is not received, or in step 5 if the contention resolution in not successful. An RLF will then be declared with the cause set to random access problem. However, since the random access procedure relies both on uplink and downlink, it will not be possible for the network to understand whether the failure occurred due to uplink or downlink problems.

Upon radio link failure with cause random access problem, the network will not be able to deduce whether the radio link failure cause was related to uplink or downlink.

Based on the above, it can be concluded that more information related to the reason for the random access problem is needed in order to deduce whether the radio link failure cause was related to uplink or downlink.

The UE should provide information to differentiate whether the radio link failure cause was related to uplink or downlink in case of random access problem. The information to be provided is FFS.

# Conclusion

In this contribution, we have made the following observations:

Observation 1	Upon radio link failure with cause T310 expiry, the network can deduce that the radio link failure cause was related to downlink.

Observation 2	Upon radio link failure with cause T312 expiry, the network can deduce that the radio link failure cause was related to downlink.

Observation 3	Upon radio link failure with cause RLC maximum number of transmissions, the network can deduce that the radio link failure cause was related to uplink.

Observation 4	Upon radio link failure with cause random access problem, the network will not be able to deduce whether the radio link failure cause was related to uplink or downlink.

We propose the following:

Proposal 1	The UE should provide information to differentiate whether the radio link failure cause was related to uplink or downlink in case of random access problem. The information to be provided is FFS.

# References

R2-1906866, MRO overview, Ericsson, 3GPP TSG-RAN WG2 Meeting #106, Reno, USA, 13-17 May 2019.

TS 38.321 NR; Medium Access Control (MAC) protocol specification