3GPP TSG-RAN WG2 Meeting #106                                                            	R2-1906916

Reno, USA, 13th - 17th May 2019                                                    Revision of R2-1903313

Source:	CATT

Title:	Additional Aspects on Early Measurement Configuration

Agenda Item:   11.10.3

Document for:	Discussion and Decision

- Introduction

In RAN2#105bis, we had made many agreements on early measurement reporting and configuration.

In this contribution, we discuss additional aspects  for  early measurement configuration, including use of delta signaling for early measurement configuration in inactive mode and  early measurement configuration for MR-DC involving two nodes for measurement configuration.

- Discussion

  - Measurement Configuration in Inactive Mode

During email discussion [1], most companies agree to reuse most fields of SIB4 for NR specific early measurement configuration. The overhead of these fields mentioned in [1] is listed in table 1. If the max number of inter-frequencies for early measurement configuration in NR is 8 (same as LTE), then the total size of these field for NR early measurement configuration is 8*208 =1664 bits. In LTE, a measurement cell list is configured per frequency while validity area is also configured with per frequency for idle mode measurement. If the similar fields are also introduced in NR, the total size for NR early measurement configuration is beyond 2000 bits.

Table 1 Size of fields for NR specific early measurement configuration

| Field | Size(bits) |
| --- | --- |
| dl-CarrierFreq | 22 |
| FreqBandIndicatorNR | 10 |
| nrofSS-BlocksToAverage | 4 |
| absThreshSS-BlocksConsolidation | 7 |
| Smtc | 14 |
| ssbSubcarrierSpacing | 3 |
| ssb-ToMeasure | 66(Max size) |
| ss-RSSI-Measurement | 82 |
| Total | 208 |

Delta configuration were agreed to be applied to RNA configuration for inactive, as the overhead of RNA configuration is large. The overhead of measurement configuration is also large. Considering that the network can store the dedicated configuration for inactive UEs and early measurement configuration may not change when the UE transits from inactive->connected->inactive, the similar principle of handling RNA configuration can be applied for dedicated early measurement configuration for inactive modes, i.e. delta configuration is applied to early measurement configuration in inactive mode to save signalling overhead.

Proposal 1: Delta configuration is applied to early measurement configuration in inactive mode.

  - Early Measurement Configuration for MR-DC

We had agreed Rel-16 early measurement configuration may contain both NR and LTE configurations, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. However, it is unclear how the network configures early measurement configuration for MR-DC. For MR-DC, RRM is managed by the MN or the SN itself. Even the UE is configured with DC in connected before it is released to idle or inactive state, the MN may have no knowledge of RRM in the SN. For example, for EN-DC the MN is not aware which MOs are configured by the SN. Will the MN send part of early measurement configurations for MR-DC and the SN send the other part of early measurement configurations for MR-DC? Or will the MN send all early measurement configurations for MR-DC? If the MN sends all early measurement configurations for MR-DC, how will the (ng-)RAN configure suitable early measurement configuration for another (ng-)RAN? We propose RAN2 to discuss how the network configures early measurement configuration for MR-DC.

Proposal 2: RAN2 discusses how the network configures early measurement configuration for MR-DC.

- MN sends part of early measurement configurations for MR-DC and SN sends the other part of early measurement configurations for MR-DC; Or

- MN sends all early measurement configurations for MR-DC.

- Conclusion

In this contribution, we discuss some further considerations on early measurement configuration, including measurement configuration in inactive mode, and early measurement configuration for DC. And we propose:

Proposal 1: Delta configuration is applied to early measurement configuration in inactive mode.

Proposal 2: RAN2 discusses how the network configures early measurement configuration for MR-DC.

- MN sends part of early measurement configurations for MR-DC and SN sends the other part of early measurement configurations for MR-DC; Or

- MN sends all early measurement configurations for MR-DC.

- PReference

[1] Summary of email discussion [105#54] [NR/eCA-DC]: measurement configuration, RAN2#105bis