3GPP TSG-RAN WG2 Meeting #106 	R2-1906918

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Bearer type negotiation in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-03-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current bearer establishment at the SN doesn’t support bearer configuration negotiation in case the SN cannot support the requested SN resources. This results in rejection of bearer establishment. Introducing bearer type negotiation allows the SN to suggest appropriate SCG configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | . In section 10.2.2, it is introduced that during SCG Addition, the SN can provide a suggestion for bearer configuration at the SN. The MN provides SN Addition Request message taken into account the suggested bearer configuration by the SN. In section 10.3.2, it is introduced that during SCG modification, the SN can provide a suggestion for bearer configuration at the SN. The MN provides SN Addition Request message taken into account the suggested bearer configuration by the SN. Impact analysis Impacted architecture options: Impacted 5G architecture: NGEN-DC, NE-DC, NR-DC Impacted functionality: SN Modification and SN Addition Inter- operability: There are no inter-operability issues as the UE is not involved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence would be that the SN may reject requested bearer establishment if the SN cannot support the requested resources at the SN , leading to latency of bearer establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------First Change--------------------------------------

10.2.2	MR-DC with 5GC

The Secondary Node (SN) Addition procedure is initiated by the MN and is used to establish a UE context at the SN in order to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the initial SCG serving cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). Figure 10.2.2-1 shows the SN Addition procedure.

Figure 10.2.2-1: SN Addition procedure

1.	The MN decides to request the target SN to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info). In addition, for bearers requiring SCG radio resources, MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. The MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision.

For MN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides Xn-U UL TNL address information. For SN terminated bearers, the MN provides a list of available DRB IDs. The S-NG-RAN node shall store this information and use it when establishing SN terminated bearers. The SN may reject the request.

For SN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the SN decides how to map QoS flows to DRB.

NOTE 1:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 2:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

1a. If the RRM entity in the SN is unable to admit the resource request, e.g. the SN cannot allocate enough resources for the request SCG bearer, a bearer type negotiation procedure is initiated by the SN, SN Modification Required message is sent to the MN which carries the suggested bearer type configuration information, e.g. bearer type and the split ratio.

1b. The MN receives the SN Modification Required message, the MN adjusts the required SN related bearer configuration based on the SN Modification Required message, and sends the SN Addition Request message to SN with the requested SCG configuration information. The MN may reject the suggestion.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources. For bearers requiring SCG radio resources the SN triggers UE Random Access so that synchronisation of the SN radio resource configuration can be performed. The SN decides for the PScell and other SCG Scells and provides the new SCG radio resource configuration to the MN in a SN RRC configuration message contained in the SN Addition Request Acknowledge message. In case of bearer options that require Xn-U resources between the MN and the SN, the SN provides Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

NOTE 3:	In case of MN terminated bearers, transmission of user plane data may take place after step 2.

NOTE 4:	In case of SN terminated bearers, data forwarding and the SN Status Transfer may take place after step 2.

NOTE 5:	For MN terminated NR SCG bearers for which PDCP duplication with CA is configured the MN allocates 2 separate Xn-U bearers.

For SN terminated NR MCG bearers for which PDCP duplication with CA is configured the SN allocates 2 separate Xn-U bearers.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

3.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message, without modifying it.

4.	The UE applies the new configuration and replies to MN with MN RRC reconfiguration complete message, including a SN RRC response message for SN, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the encoded SN RRC response message, if received from the UE.

6.	If configured with bearers requiring SCG radio resources, the UE performs synchronisation towards the PSCell configured by the SN. The order the UE sends the MN RRC reconfiguration complete message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	In case of SN terminated bearers using RLC AM, the MN sends SN Status Transfer.

8.	In case of SN terminated bearers using RLC AM, and dependent on the bearer characteristics of the respective QoS Flows, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

9-12.	For SN terminated bearers, the update of the UP path towards the 5GC is performed via PDU Session Path Update procedure.

--------------Second Change--------------------------------------

10.3.2	MR-DC with 5GC

The SN Modification procedure may be initiated either by the MN or by the SN and be used to modify the current user plane resource configuration (e.g. related to PDU session, QoS flow or DRB) or to modify other properties of the UE context within the same SN. It may also be used to transfer an NR RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used).

The SN modification procedure does not necessarily need to involve signalling towards the UE.

MN initiated SN Modification

Figure 10.3.2-1: SN Modification procedure - MN initiated

The MN uses the procedure to initiate configuration changes of the SCG within the same SN, including addition, modification or release of the user plane resource configuration. The MN uses the procedure to query the current SCG configuration, e.g. when delta configuration is applied in a MN initiated SN change. The MN uses the procedure to provide the S-RLF related information to the SN or to provide additional available DRB IDs to be used for SN terminated bearers. The MN may not use the procedure to initiate the addition, modification or release of SCG Scells. The SN may reject the request, except if it concerns the release of the user plane resource configuration. Figure 10.3.2-1 shows an example signalling flow for a MN initiated SN Modification procedure.

1.	The MN sends the SN Modification Request message, which may contain user plane resource configuration related or other UE context related information, data forwarding address information (if applicable), PDU session level Network Slice info and the requested SCG configuration information, including the UE capabilities coordination result to be used as basis for the reconfiguration by the SN. In case a security key update in the SN is required, a new SN Security Key is included.

1a. If SN Modification Request message including SN bearer configuration addition or modification, and if the RRM entity in the SN is unable to admit the resource request, e.g. the SN cannot allocate enough resources for the request SCG bearer, a bearer type negotiation procedure is initiated by the SN,  SN Modification Required message is sent to the MN which carries the suggested bearer type configuration information, e.g. bearer type and the split ratio.

1b. The MN receives the SN Modification Required message, the MN adjusts the required SN related bearer configuration based on the SN Modification Required message, and sends the SN Addition Request message to SN with the requested SCG configuration information. The MN may reject the suggestion.

2.	The SN responds with the SN Modification Request Acknowledge message, which may contain new SCG radio configuration information within a SN RRC configuration message, and data forwarding address information (if applicable).

NOTE 1:	For MN terminated NR SCG bearers to be setup for which PDCP duplication with CA is configured the MN allocates 2 separate Xn-U bearers

For SN terminated NR MCG bearers to be setup for which PDCP duplication with CA is configured the SN allocates 2 separate Xn-U bearers.

2a.	For SN terminated MCG bearers, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

3/4.	The MN initiates the RRC connection reconfiguration procedure, including SN RRC configuration message. The UE applies the new configuration and replies with MN RRC reconfiguration complete message, including a SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Reconfiguration Complete message.

6.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

7.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status transfer.

8.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-1 depicts the case where a user plane resource configuration related context is transferred from the MN to the SN).

9.	The SN sends the Secondary RAT Data Volume Report message to the MN and includes the data volumes delivered to the UE for the QoS flows to be released.

NOTE 2:	The order the SN sends the Secondary RAT Data Volume Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

10.	If applicable, a PDU Session path update procedure is performed.

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the modification/release of the user plane resource configuration and to trigger PSCell changes (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). The MN cannot reject the release request of PDU session/QoS flows. The SN also uses the procedure to request the MN to provide more DRB IDs to be used for SN terminated bearers or to return DRB IDs used for SN terminated bearers that are not needed any longer. Figure 10.3.2-2 shows an example signalling flow for SN initiated SN Modification procedure.

1.	The SN sends the SN Modification Required message including a SN RRC configuration message, which may contain user plane resource configuration related context, other UE context related information and the new radio resource configuration of SCG. In case of change of security key, the PDCP Change Indication indicates that a SN security key update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required. For bearer type negotiation, suggested bearer type and/or split ration will be included.

The SN can decide whether the change of security key is required.

Editor's note: The use and setting of the PDCP Change Indication for all the MR-DC options is still FFS.

2/3.	The MN initiated SN Modification procedure may be triggered by SN Modification Required message, e.g. when a SN security key change needs to be applied.

NOTE 3:	For SN terminated NR MCG bearers to be setup for which PDCP duplication with CA is configured the SN allocates 2 separate Xn-U bearers.

4.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the MN RRC reconfiguration complete message, including an encoded SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Modification Confirm message containing the encoded SN RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell configured by the SN as described in SN Addition procedure. Otherwise, the UE may perform UL transmission directly after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN sends the MN Status transfer.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-2 depicts the case where a user plane resource configuration related context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Volume Report message to the MN and includes the data volumes delivered to the UE for the QoS flows to be released.

NOTE 4:	The order the SN sends the Secondary RAT Data Volume Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

11.	If applicable, a PDU Session path update procedure is performed.