3GPP TSG-RAN WG2 meeting #106	 R2-1906985

Reno, Nevada, USA, 13th– 17th May, 2019                                                      Revision of R2-1903978

Source: 	Huawei, HiSilicon

Title: 		BAP layer header content design

Agenda Item:	11.1.3

Document for:		Discussion and Decision

- Introduction

As discussed in the study item phase, the adaptation layer is introduced in wireless backhaul links above RLC layer. In RAN2 #105 meeting [1], RAN2 has agreed that “RAN2 confirms that routing and bearer mapping (e.g. mapping of BH RLC channels) are adaptation layer functions”. And in RAN2 #105bis meeting [2], it has been agreed by RAN2 that the name of the “adapt’ is “Backhaul Adaptation Protocol” “BAP”.

In this contribution, we will focus on the details of the adaptation information which should be carried in BAP PDU to enable the two basic functionalities of BAP layer.

- Discussion

Since the BAP layer is responsible for routing and bearer mapping, the BAP entity of an IAB node or donor DU should be provided with enough information to enable the two functionalities.

First, to enable the routing function, the BAP header should contain routing related information, as agreed by RAN2 in last meeting that ““Destination IAB node/IAB donor-DU address” and “Specific path identifier” (carried in the BAP) are considered as candidate for route identifier for routing at an adaptation layer. Additional required information for routing is FFS ”, thus, the routing info can be destination node ID or the specific path identifier. Based on further analysis about the routing in another paper [3], the destination node ID based solution outperforms the path identifier based solution in many perspectives, and we prefer to use destination node ID in the BAP header for routing across wireless BH links.

Then for the bearer mapping function, in last RAN2 meeting, the following agreements are achieved

Confirm that the intention is to support 1-to-1 and 1-to-N bearer mapping, for UE bearers, at least for UP.

For user plane, The UL mapping in the IAB access node to BH RLC channels should be based on the knowledge about UE bearers (identified with GTP TEID)

For control plane (F1-C messages) The UL mapping in the IAB access node to BH RLC channels should be based on F1-C message type. FFS if per UE.

FFS if the mapping should also consider DSCP/Flow labels (e.g. as an intermediate step).

Observation: The UL/DL mapping in intermediate IAB node(s) to egress BH RLC channel will take into account ingress BH RLC channel.

FFS: The UL/DL mapping in intermediate IAB node(s) to egress BH RLC channel could also take into account some ID(s) (from Adaptation Layer).

The above two Bullets are applicable for all types of traffic (e.g. UP, CP, OAM).

For the uplink transmission, it is the access IAB node which maps UL packets to its egress BH RLC channel and adds the BAP header, while for the downlink transmission, the IAB donor DU maps DL packets to an egress BH RLC channel and add the BAP header. Based on the above agreements and observations, for intermediate IAB nodes, the bearer mapping is basically based on the ingress RLC channel, some additional IDs may also be used for enhancement. For example, UE bearer information can be carried in the BAP header as additional ID to do bearer mapping.

Consequently, there may be multiple combinations about the content of Adaptation information in uplink and downlink packets, as enumerated in follows.

Example 1. Downlink packet: IAB node ID; Uplink packet: donor DU ID (optional);

In such case, routing selection in BAP layer is chosen based on the destination node ID, the destination node for downlink is access IAB node, while the destination node for uplink is donor DU. The destination node for uplink transmission can be optional if only one donor DU can forward packets between a given IAB node and CU. Bearer mapping in intermediate IAB node is based on mapping rules between ingress RLC channels to egress RLC channels.

Example 2. Downlink packet: IAB node ID+UE bearer specific ID (e.g. UE ID+UE bearer ID); Uplink packet: donor DU ID (optional) + UE bearer specific ID (e.g. UE ID+UE bearer ID).

In such case, routing selection is same as example 1. Bearer mapping in intermediate IAB node can use the ingress RLC channel information and the UE bearer information to choose suitable egress BH RLC channels. The UE bearer specific ID should be unique in the serving area of IAB donor, and UE ID+UE bearer ID can be used as an example.

Example 2 provides more flexibility for bearer mapping when compared to example 1, since it enables the IAB node to map two UE DRBs to different egress RLC channel even they are mapped to a same ingress RLC channel by the previous hop node.

Proposal 1: RAN2 agrees to include destination node ID (e.g. destination IAB node ID for DL, IAB donor DU ID for UL) in the BAP layer header.

Proposal 2: RAN2 agrees to include UE bearer specific ID in the BAP layer header.

As been analysed in section 9.5.3 of [4], some QoS profiles (e.g. GBR, 5QI/QCI, ARP, etc.) of UE DRB are needed for the access IAB node, the donor DU, and intermediate IAB nodes, to provide E2E QoS guarantee of UE’s traffic transmission across wireless backhaul links in IAB network. One possible solution is carrying necessary QoS parameters in the BAP header of each packet, such solution is so inefficient since it will consuming too much overhead in BAP layer header and should not be considered. Instead, some QoS related identifier (e.g. UE bearer ID) can be carried in BAP layer header to enable each IAB node and the donor DU provide QoS guarantee for UE bearer. RAN2 should study what kind of QoS related ID is suitable to be carried in BAP layer header.

Proposal 3: QoS information (e.g. detailed QoS parameters) is not included in the BAP layer header, which does not exclude the QoS related ID.

In addition, considering that the DL hop by hop flow control for the IAB network may rely on the BAP layer feedback [5], the control PDU of BAP layer to convey the DL transmission status is necessary, then an indicator to identify whether the PDU type is the control PDU or the data PDU of the BAP layer should be contained in the BAP PDU.

Proposal 4: An indicator should be contained in the BAP PDU to identify the PDU type (i.e. control PDU or the data PDU of BAP layer).

- Conclusion

This paper mainly discusses on the adaptation information being carried in BAP header to enable IAB node performing routing and QoS enforcement, the following proposals are given:

Proposal 1: RAN2 agrees to include destination node ID (e.g. destination IAB node ID for DL, IAB donor DU ID for UL) in the BAP layer header.

Proposal 2: RAN2 agrees to include UE bearer specific ID in the BAP layer header.

Proposal 3: QoS information (e.g. detailed QoS parameters) is not included in the BAP layer header, which does not exclude the QoS related ID.

Proposal 4: An indicator should be contained in the BAP PDU to identify the PDU type (i.e. control PDU or the data PDU of BAP layer).

- Reference

- Chairman notes of 3GPP RAN WG2 meeting #105.

- Chairman notes of 3GPP RAN WG2 meeting #105bis.

- R2-1906064, Routing function and configuration. Huawei, HiSilicon.

- 3GPP TR38.874 v16.0.0, Study on integrated access and backhaul.

- R2-1906976, Flow control for IAB network, Huawei, HiSilicon.