3GPP TSG-RAN WG2 Meeting # 106	     R2-1906990

Reno, NV, USA, 13th - 17th  May, 2019

Agenda Item:	11.1.3

Source:	Ericsson

Title:	Backhaul Adaptation Protocol (BAP) Header Content

Document for: Discussion, Decision

# 1	Introduction

At RAN2#105bis, it was agreed to rename the “Adaptation layer” to Backhaul Adaptation Protocol (BAP), which will perform the routing and bearer mapping functions for IAB network. It was further agreed to consider destination IAB node/IAB donor-DU address and path identifier as candidates for “route identifier” that will be carried/included in BAP header for routing purposes. However, some other new identifier(s) may need to be included for other potential functions provided by the BAP. This contribution discusses these aspects in detail.

# 2 	Discussion

Figure 1 shows the UP architecture recommended for the IAB WI, where the full F1-U header (GTP/UDP/IP) terminates at the IAB DU. For this architecture, the GTP TEID and IP header can be used for mapping UE user-plane PDUs to BH RLC channels and identifying the UE-bearer for the PDU at the Access IAB node as explained in [3], the Donor DU and the Access IAB node will perform this mapping based on the configuration done by the CU-CP, since the CU-CP is responsible for the DRB management of all the UEs served by Donor DU as well as downstream IAB nodes. Consequently, the BAP header does not need to carry any specific identifier for the UE and UE-bearer.

Figure 1: User Plane (UP) architecture recommended for Rel-16 WI on IAB.

Regarding static QoS information of UE bearers, the BAP does not need any explicit identifier to carry in its header for appropriate QoS treatment of packets by the network nodes. The CU-CP will implement/enforce the QoS requirements for applications/services used by end-user by appropriately configuring the mapping rules between the ingress and egress BH RLC channels. Thus, the intermediate IAB nodes will simply use these lookup tables to forward packets to egress BH RLC channels suited to packets/data QoS profile. However, some dynamic information, such as the time a packet has already spent in the network, could be useful for enforcing the QoS requirements of the bearers [4]. Optional fields in the BAP can be used to convey such information to intermediate IAB nodes.

When it comes to other potential information/identifier carried in the BAP header, one possible field can be the marker flag used by IAB nodes for marking the packets that experienced queueing delay beyond a threshold value, an approach proposed in [5] for DL flow control in IAB network. Moreover, some control elements might be defined for the BAP layer that can be used for L2 communication between IAB nodes, such as indicating backhaul RLF or recovery to child IAB nodes [6]. For this purpose, the BAP header needs to carry flag(s)/indicator(s) for differentiating control packets from data packets. Lastly, some flags might be included in the BAP header for future compatible extension. Figure 2 shows a sample header for the BAP layer, where the MSB indicates whether it’s a control or data headers and then few reserve bits for flags and future compatibility.

Figure 2: Example of BAP header

The BAP header does not need to carry any specific identifier for the UE, UE-bearer, and static bearer QoS information.

The BAP header would need some flags/fields for flow control, differentiating BAP layer control packets from data packets, and optional fields enabling dynamic QoS enforcement.

The BAP header would need some flags for future compatible extension.

Finally, the BAP header will be counted as overhead incurred in RAN. To keep the overhead low, it will be useful to adopt a minimum set of identifiers/fields needed to provide the essential functions.

The BAP header should contain a flag indicating whether it is a data or control PDU.

The BAP header should contain one or more reserved bits for future extension.

# 3	Conclusion

In this paper we made the following observations:

Observation 1	The BAP header would need some flags/fields for flow control, differentiating BAP layer control packets from data packets, and optional fields enabling dynamic QoS enforcement.

Observation 2	The BAP header would need some flags for future compatible extension.

Leading to the following proposals:

Proposal 1	The BAP header does not need to carry any specific identifier for the UE, UE-bearer, and static bearer QoS information.

Proposal 2	The BAP header should contain a flag indicating whether it is a data or control PDU.

Proposal 3	The BAP header should contain one or more reserved bits for future extension.

# 4		Reference

[1] R2-182882; WID: Integrated Access and Backhaul for NR, Qualcomm, RAN#82, Sorrento, Italy, December 2018.

[2] TR 38.874; Study on Integrated Access and Backhaul.

[3] R2-1901323; QoS Mapping to Backhaul Bearers in IAB Networks, Ericsson, RAN2#105, Athens, Greece, February 2019.

[4] R2-1906996; Short timestamp for user plane latency control, RAN2#106, Reno, USA, May 2019.

[5] R2-1906988; Flow Control in IAB, Ericsson, RAN2#106, Reno, USA, May 2019.

[6] R2-1906991; RLF related notifications in IAB networks, Ericsson, RAN2#106, Reno, USA, May 2019.