3GPP TSG-RAN WG2 Meeting #106	R2-1907026

Reno, Nevada, USA, 13th – 17th May 2019

Title:	DRAFT LS on the ETWS/CMAS in connected mode

Response to:

Release:	Release 16

Work Item:	LTE_eMTC5

Source:	Huawei, HiSilicon [TSG RAN WG2]

To:	TSG-RAN WG1

Cc:

Contact Person:

#### Name:	Yulong Shi

E-mail Address:	shiyulong5@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

To support ETWS/CMAS in connected mode for non-BL UE in CE mode, RAN2 considers monitoring P-RNTI for the notification of ETWS/CMAS and delivering the content of ETWS/CMAS by legacy broadcasted SI.

After receiving the notification, for UEs in Connected mode, the UE may be interrupted due to reception of ETWS/CMAS in other narrowband than its dedicated narrowband, if UE can enter into IDLE mode quietly to receive ETWS/CMAS.

The option without interruption is that a non-BL UE in CE mode receives ETWS/CMAS on one narrowband and receives unicast transmission on its dedicated narrowband simultaneously.

Therefore, RAN2 would like to kindly ask RAN1 to confirm whether a non-BL UE in CE mode can receive SI on the SI narrowband and receive data on its dedicated narrowband simultaneously.

2. Actions:

To RAN1:

RAN2 respectfully asks RAN1 to confirm:

Whether a non-BL UE in CE mode can receive SI on the SI narrowband and receive data on its dedicated narrowband simultaneously.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#107	26 - 30 August 2019   	Prague   	Czech Republic

3GPPRAN2#107bis	14 - 18 October 2019	TBD		China