3GPP TSG RAN WG2 Meeting #106	R2- 1907151

Reno, USA, 13-17 May 2019

Source:	u-blox AG, Mitsubishi Electric

Title:	Text Proposal for addition of SSR Grid Definition message

Agenda Item:	11.8.2.2

Document for:	Discussion and Decision

# Introduction

A new work item on "NR Positioning Support" was agreed at RAN#83 [1]. The objectives include:

Define extensions of LPP protocol to support GNSS SSR (PPP-RTK support) based on the “Compact SSR” definitions specified for QZSS [RAN2]

In this contribution we propose the addition of a new Grid Definition message that enables generalization of CLAS [3] beyond the Japan region in the Running CR [2] for 36.355.

# Discussion

In R2-1970147 [4] we propose a complete set of new messages required to support SSR (PPP-RTK):

- Compact SSR GNSS Orbit Correction,

- Compact SSR GNSS Clock Corrections,

- Compact SSR GNSS Satellite Code Bias,

- Compact SSR Satellite Phase Bias,

- Compact SSR GNSS URA,

- Compact SSR STEC Correction,

- Compact SSR Gridded Correction.

- New Grid Definition Message

The first three are already supported in LPP, Message 4, 6 and 7 have already been added to the Running CR [2]. The text proposal [5] covers message 5 Compact SSR GNSS URA. This text proposal proposes the addition of a flexible Grid Definition message (8) that allows the structure of the Areas and Grids used for atmospheric corrections to be broadcast to the receiver in place of the fixed implicit definition contained in CLAS [3].

# Text Proposal

--- Start of Change ---

#### –	GNSS-GenericAssistData

The IE GNSS-GenericAssistData is used by the location server to provide assistance data for a specific GNSS (e.g., GPS, Galileo, GLONASS, BDS, etc.). The specific GNSS for which the provided assistance data are applicable is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be provided.

-- ASN1START

# GNSS-GenericAssistData ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataElement

# GNSS-GenericAssistDataElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID							OPTIONAL, 	-- Cond GNSS-ID-SBAS

gnss-TimeModels					GNSS-TimeModelList				OPTIONAL,	-- Need ON

gnss-DifferentialCorrections	GNSS-DifferentialCorrections	OPTIONAL,	-- Need ON

gnss-NavigationModel			GNSS-NavigationModel			OPTIONAL,	-- Need ON

gnss-RealTimeIntegrity			GNSS-RealTimeIntegrity			OPTIONAL,	-- Need ON

gnss-DataBitAssistance			GNSS-DataBitAssistance			OPTIONAL,	-- Need ON

gnss-AcquisitionAssistance		GNSS-AcquisitionAssistance		OPTIONAL,	-- Need ON

gnss-Almanac					GNSS-Almanac					OPTIONAL,	-- Need ON

gnss-UTC-Model					GNSS-UTC-Model					OPTIONAL,	-- Need ON

gnss-AuxiliaryInformation		GNSS-AuxiliaryInformation		OPTIONAL,	-- Need ON

...,

[[

bds-DifferentialCorrections-r12

BDS-DifferentialCorrections-r12	OPTIONAL,	-- Cond	GNSS-ID-BDS

bds-GridModel-r12			BDS-GridModelParameter-r12		OPTIONAL	-- Cond	GNSS-ID-BDS

]],

[[

gnss-RTK-Observations-r15	GNSS-RTK-Observations-r15		OPTIONAL,	-- Need ON

glo-RTK-BiasInformation-r15	GLO-RTK-BiasInformation-r15		OPTIONAL,	-- Cond GNSS-ID-GLO

gnss-RTK-MAC-CorrectionDifferences-r15

GNSS-RTK-MAC-CorrectionDifferences-r15

OPTIONAL,	-- Need ON

gnss-RTK-Residuals-r15		GNSS-RTK-Residuals-r15			OPTIONAL,	-- Need ON

gnss-RTK-FKP-Gradients-r15	GNSS-RTK-FKP-Gradients-r15		OPTIONAL,	-- Need ON

gnss-SSR-OrbitCorrections-r15

GNSS-SSR-OrbitCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-ClockCorrections-r15

GNSS-SSR-ClockCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-CodeBias-r15		GNSS-SSR-CodeBias-r15			OPTIONAL	-- Need ON

]],

[[

gnss-SSR-PhaseBias-r16		GNSS-SSR-PhaseBias-r16			OPTIONAL,	-- Need ON

gnss-SSR-STEC-Correction-r16

GNSS-SSR-STEC-Correction-r16	OPTIONAL,	-- Need ON

gnss-SSR-GriddedCorrection-r16

GNSS-SSR-GriddedCorrection-r16	OPTIONAL,	-- Need ON

gnss-SSR-URA-r16			GNSS-SSR-URA-r16				OPTIONAL,	-- Need ON

gnss-SSR-GridDefinition-r16

GNSS-SSR-GridDefinition-r16		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| GNSS-ID-BDS | The field may be present if the GNSS-ID = bds; otherwise it is not present. |
| GNSS-ID-GLO | The field may be present if the GNSS ID = glonass; otherwise it is not present. |

#### –	GNSS-PeriodicAssistData

The IE GNSS-PeriodicAssistData is used by the location server to provide control parameters for a periodic assistance data delivery session (e.g., interval and duration) to the target device.

NOTE:	Omission of a particular assistance data type field in IE GNSS-PeriodicAssistData means that the location server does not provide this assistance data type in a data transaction of a periodic assistance data delivery session, as described in sub-clauses 5.2.1a and 5.2.2a. Inclusion of no assistance data type fields in IE GNSS-PeriodicAssistData means that a periodic assistance data delivery session is terminated.

-- ASN1START

GNSS-PeriodicAssistData-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservations-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

glo-RTK-PeriodicBiasInformation-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-MAC-PeriodicCorrectionDifferences-r15

GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-PeriodicResiduals-r15			GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-FKP-PeriodicGradients-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-SSR-PeriodicOrbitCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicClockCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicCodeBias-r15			GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

...,

[[

gnss-SSR-PeriodicPhaseBias-r16			GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicSTEC-Correction-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicGriddedCorrection-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicURA-r16				GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-SSR-PeriodicGridDefinition-r16		GNSS-PeriodicControlParam-r15	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

--- End of Change ---

--- Start of Change ---

#### –	GNSS-SSR-GridDefinition

The IE GNSS-SSR-GridDefinition is used by the location server to provide the layout and structure of the Areas, Parts and Grids used for the gridded atmospheric corrections.

-- ASN1START

GNSS-GridDefinition-r16 ::= SEQUENCE {

iod-grid-r16                        INTEGER (0..7),

grid-direction-r16                  BIT STRING (SIZE(1)),

grid-resolution-r16                 BIT STRING (SIZE(1)),

number-of-networks-r16              INTEGER (0..31),

gnss-grid-network-list-r16          GNSS-Grid-NetworkList-r16

}

GNSS-Grid-NetworkList-r16 ::= SEQUENCE (SIZE(1..32)) OF GNSS-Grid-NetworkElement-r16

GNSS-Grid-NetworkElement-r16 ::= SEQUENCE {

network-id-r16                      INTEGER (0..31),

reference-latitude-r16              INTEGER (-900..900) or (-9000..9000)

reference-longitude-r16             INTEGER (-1800..1800) or (-18000..18000)

step-availability-r16               BIT STRING (SIZE(1)),

latitude-step-r16                   INTEGER (0..31) or (0..255) OPTIONAL

longitude-step-r16                  INTEGER (0..31) or (0..255) OPTIONAL

number-of-parts-r16                 INTEGER (0..15),

gnss-grid-part-list-r16             GNSS-Grid-PartList-r16

}

GNSS-Grid-PartList-r16 ::= SEQUENCE (SIZE(1..16)) OF GNSS-Grid-PartElement-r16

GNSS-Grid-PartElement-r16 ::= SEQUENCE {

grid-type-r16                       INTEGER (0..3),

gnss-grid-part-element-type1-r16    GNSS-Grid-PartElementType1-r16 OPTIONAL

gnss-grid-part-element-type2-r16    GNSS-Grid-PartElementType2-r16 OPTIONAL

gnss-grid-part-element-type3-r16    GNSS-Grid-PartElementType3-r16 OPTIONAL

}

GNSS-Grid-PartElementType1-r16 ::= SEQUENCE {

latitude-offset-ratio-r16           INTEGER (0..3),

longitude-offset-raito-r16          INTEGER (0..3),

latitude-offset-step-r16            INTEGER (-15..15),

longitude-offset-step-r16           INTEGER (-15..15),

number-of-grids-latitude-r16        INTEGER (0..15),

number-of-grids-longitude-r16       INTEGER (0..15),

}

GNSS-Grid-PartElementType2-r16 ::= SEQUENCE {

latitude-offset-ratio-r16           INTEGER (0..3),

longitude-offset-raito-r16          INTEGER (0..3),

latitude-offset-step-r16            INTEGER (-15..15),

longitude-offset-step-r16           INTEGER (-15..15),

number-of-grids-latitude-r16        INTEGER (0..15),

number-of-grids-longitude-r16       INTEGER (0..15),

grid-definition-mask-r16            BIT STRING(SIZE(Nlat*Nlon))

}

GNSS-Grid-PartElementType3-r16 ::= SEQUENCE {

size-of-offset-r16                  BIT STRING(SIZE(2)),

number-of-grids-offset-r16          INTEGER (0..63),

gnss-grid-offset-list-r16           GNSS-Grid-OffsetList-r16

}

GNSS-Grid-OffsetList-r16 ::= SEQUENCE (SIZE(1..64)) OF GNSS-Grid-OffsetElement-r16

GNSS-Grid-OffsetElement-r16 ::= SEQUENCE {

delta-latitude-r16                  INTEGER (-127..127) or (-511..511) or (-2047..2047)

delta-longitude-r16                 INTEGER (-127..127) or (-511..511) or (-2047..2047)

}

-- ASN1STOP

GNSS-Grid-Definition field descriptions

iod-grid The issue of grid information, which counts up when the set of available grid information is changed.

grid-direction It defines the direction of grid numbering. 0: originated from west-north corner, move eastward then southward, 1: originated from west-sourth corner, move northward then eastward.

grid-resolution It defines the resolution of grid information. 0: normal (0.1deg), 1: fine (0.01deg).

number-of-networks It defines the number of the network (value+1) included in the message.

number-id It defines the network id for a specific network.

reference-latitude The latitude of reference point in each network. If grid-resolution equals to 0, the resolution (size) is 0.1deg (int11), otherwise 0.01deg (int15).

reference-longitude The longitude of reference point in each network. If grid-resolution equals to 0, the resolution (size) is 0.1deg (int12), otherwise 0.01deg (int16).

step-availability It defines the availability of step information (latitude-step, longitude-step) in the message.

latitude-step The interval of grid for latitude direction. Included only if step-availability equals to 1. The resolution is 0.1deg if grid-resolution equals to 0, otherwise 0.01deg.

longitude-step The interval of grid for longitude direction. Included only if step-availability equals to 1. The resolution is 0.1deg if grid-resolution equals to 0, otherwise 0.01deg.

number-of-parts It defines the number of parts included in the network. It it is set to zero, the single grid defined by the coordinate of reference point is defined in the network.

grid-type It define the type of grid. 0: reserved, 1: uniform grids, 2: uniform grid with bitmask, 3: non-uniform grids.

latitude-offset-ratio The coefficient of offset for latitude. latitude-offset-step is not included if latitude-offset-ratio equals to 0. If latitude-offset-ratio set to 1, 2, 3 then r_lat is 1, 0.5, 0.25 respectively.

longitude-offset-ratio The coefficient of offset for longitude. longitude-offset-step is not included if longitude-offset-ratio equals to 0. If longitude-offset-ratio set to 1, 2, 3 then r_lon is 1, 0.5, 0.25 respectively.

latitude-offset-step The offset step for latitude to define the reference point of the part. Included only if latitude-offset-ratio >0. The latitude of reference point of each part is defined as : reference-latitude-part = reference-latitude + r_lat x latitude-offset-step x latitude-step

longitude-offset-step The offset step for longitude to define the reference point of the part. Included only if longitude-offset-ratio >0. The longitude of reference point of each part is defined as : reference-longitude-part = reference-longitude + r_lon x longitude-offset-step x longitude-step

number-of-grids-latitude It defines the number of grids for latitude direction included in the part. The latitude of i-th grid is calculated by : latitude--grid = reference-latitude-part + i x latitude-step

number-of-grids-longitude It defines the number of grids for longitude direction included in the part. The longitude of j-th grid is calculated by : longitude-grid = reference-longitude-part + i x longitude-step

grid-definition-mask It defines the availability of each grid in the rectangular lattice defined by number-of-grids-latitude x number-of-grids-longitude. If n-th grid from the most left bit is set to 1, n-th grid is valid.

size-of-offset It defines the bit-length of offset. 0: 8bit, 1: 10bit, 2: 12bit, 3: reserved

number-of-grids-offset It defines the number of grids for grid-type 3.

delta-latitude It defines the relative latitude of grid for grid-type 3. If grid-resolution equals to 0, the resolution is 0.1deg, otherwise 0.01deg. The range is defined by size-of-offset. The latitude of each grid is calculated by : latitude--grid = reference-latitude-part + delta-latitude

delta-longitude It defines the relative longitude of grid for grid-type 3. If grid-resolution equals to 0, the resolution is 0.1deg, otherwise 0.01deg. The range is defined by size-of-offset. The longitude of each grid is calculated by : longitude-grid = reference-longitude-part + delta-longitude

--- End of Change ---

--- Start of Change ---

#### –	A-GNSS-RequestAssistanceData

The IE A-GNSS-RequestAssistanceData is used by the target device to request GNSS assistance data from a location server.

-- ASN1START

# A-GNSS-RequestAssistanceData ::= SEQUENCE {

gnss-CommonAssistDataReq		GNSS-CommonAssistDataReq		OPTIONAL, -- Cond CommonADReq

gnss-GenericAssistDataReq		GNSS-GenericAssistDataReq		OPTIONAL, -- Cond GenADReq

...,

[[

gnss-PeriodicAssistDataReq-r15

GNSS-PeriodicAssistDataReq-r15	OPTIONAL -- Cond PerADReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| CommonADReq | The field is mandatory present if the target device requests GNSS-CommonAssistData; otherwise it is not present. |
| GenADReq | This field is mandatory present if the target device requests GNSS-GenericAssistData for one or more specific GNSS; otherwise it is not present. |
| PerADReq | This field is mandatory present if the target device requests periodic GNSS assistance data delivery. This field may only be included if any of the fields are included in IE GNSS-GenericAssistDataReq: - GNSS-RTK-ObservationsReq, - GLO-RTK-BiasInformationReq, - GNSS-RTK-MAC-CorrectionDifferencesReq, - GNSS-RTK-ResidualsReq, - GNSS-RTK-FKP-GradientsReq, - GNSS-SSR-OrbitCorrectionsReq, - GNSS-SSR-ClockCorrectionsReq, - GNSS-SSR-CodeBiasReq, - GNSS-SSR-PhaseBiasReq, - GNSS-SSR-STEC-CorrectionReq, or - GNSS-SSR-GriddedCorrectionReq, - GNSS-SSR-URAReq, - GNSS-SSR-GridDefinitionReq |

--- End of Change ---

--- Start of Change ---

#### –	GNSS-GenericAssistDataReq

The IE GNSS-GenericAssistDataReq is used by the target device to request assistance data from a location server for one or more specific GNSS (e.g., GPS, Galileo, GLONASS, BDS, etc.). The specific GNSS for which the assistance data are requested is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be requested.

-- ASN1START

# GNSS-GenericAssistDataReq ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataReqElement

# GNSS-GenericAssistDataReqElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID							OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsReq				GNSS-TimeModelListReq			OPTIONAL, -- Cond TimeModReq

gnss-DifferentialCorrectionsReq	GNSS-DifferentialCorrectionsReq	OPTIONAL, -- Cond DGNSS-Req

gnss-NavigationModelReq			GNSS-NavigationModelReq			OPTIONAL, -- Cond NavModReq

gnss-RealTimeIntegrityReq		GNSS-RealTimeIntegrityReq		OPTIONAL, -- Cond RTIReq

gnss-DataBitAssistanceReq		GNSS-DataBitAssistanceReq		OPTIONAL, -- Cond DataBitsReq

gnss-AcquisitionAssistanceReq	GNSS-AcquisitionAssistanceReq	OPTIONAL, -- Cond AcquAssistReq

gnss-AlmanacReq					GNSS-AlmanacReq					OPTIONAL, -- Cond AlmanacReq

gnss-UTCModelReq				GNSS-UTC-ModelReq				OPTIONAL, -- Cond UTCModReq

gnss-AuxiliaryInformationReq	GNSS-AuxiliaryInformationReq	OPTIONAL, -- Cond AuxInfoReq

...,

[[

bds-DifferentialCorrectionsReq-r12

BDS-DifferentialCorrectionsReq-r12

OPTIONAL,	-- Cond DBDS-Req

bds-GridModelReq-r12		BDS-GridModelReq-r12			OPTIONAL	-- Cond BDS-GridModReq

]],

[[

gnss-RTK-ObservationsReq-r15

GNSS-RTK-ObservationsReq-r15	OPTIONAL,	-- Cond RTK-OSR-Req

glo-RTK-BiasInformationReq-r15

GLO-RTK-BiasInformationReq-r15	OPTIONAL,	-- Cond GLO-CPB-Req

gnss-RTK-MAC-CorrectionDifferencesReq-r15

GNSS-RTK-MAC-CorrectionDifferencesReq-r15

OPTIONAL,	-- Cond MAC-Req

gnss-RTK-ResidualsReq-r15	GNSS-RTK-ResidualsReq-r15		OPTIONAL,	-- Cond Res-Req

gnss-RTK-FKP-GradientsReq-r15

GNSS-RTK-FKP-GradientsReq-r15	OPTIONAL,	-- Cond FKP-Req

gnss-SSR-OrbitCorrectionsReq-r15

GNSS-SSR-OrbitCorrectionsReq-r15

OPTIONAL, 	-- Cond OC-Req

gnss-SSR-ClockCorrectionsReq-r15

GNSS-SSR-ClockCorrectionsReq-r15

OPTIONAL, 	-- Cond CC-Req

gnss-SSR-CodeBiasReq-r15	GNSS-SSR-CodeBiasReq-r15		OPTIONAL 	-- Cond CB-Req

]],

[[

gnss-SSR-PhaseBiasReq-r16	GNSS-SSR-PhaseBiasReq-r16		OPTIONAL,	-- Cond PB-Req

gnss-SSR-STEC-CorrectionReq-r16

GNSS-SSR-STEC-CorrectionReq-r16	OPTIONAL,	-- Cond STEC-Req

gnss-SSR-GriddedCorrectionReq-r16

GNSS-SSR-GriddedCorrectionReq-r16

OPTIONAL,	-- Cond Grid-Req

gnss-SSR-URAReq-r16			GNSS-SSR-URAReq-r16				OPTIONAL,	-- Cond URA-Req

gnss-SSR-GridDefinitionReq-r16

GNSS-SSR-GridDefinitionReq-r16	OPTIONAL,	-- Cond GridDef-Req

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModReq | The field is mandatory present if the target device requests GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Req | The field is mandatory present if the target device requests GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModReq | The field is mandatory present if the target device requests GNSS-NavigationModel; otherwise it is not present. |
| RTIReq | The field is mandatory present if the target device requests GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsReq | The field is mandatory present if the target device requests GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistReq | The field is mandatory present if the target device requests GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacReq | The field is mandatory present if the target device requests GNSS-Almanac; otherwise it is not present. |
| UTCModReq | The field is mandatory present if the target device requests GNSS-UTCModel; otherwise it is not present. |
| AuxInfoReq | The field is mandatory present if the target device requests GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Req | The field is mandatory present if the target device requests BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModReq | The field is mandatory present if the target device requests BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Req | The field is mandatory present if the target device requests GNSS-RTK-Observations; otherwise it is not present. |
| GLO-CPB-Req | The field is mandatory present if the target device requests GLO-RTK-BiasInformation; otherwise it is not present. |
| MAC-Req | The field is mandatory present if the target device requests GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Req | The field is mandatory present if the target device requests GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Req | The field is mandatory present if the target device requests GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Req | The field is mandatory present if the target device requests GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Req | The field is mandatory present if the target device requests GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Req | The field is mandatory present if the target device requests GNSS-SSR-CodeBias; otherwise it is not present. |
| PB-Req | The field is mandatory present if the target device requests GNSS-SSR-PhaseBias; otherwise it is not present. |
| STEC-Req | The field is mandatory present if the target device requests GNSS-SSR-STEC-Correction; otherwise it is not present. |
| Grid-Req | The field is mandatory present if the target device requests GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| URA-Req | The field is mandatory present if the target device requests GNSS-SSR-URA; otherwise it is not present. |
| GridDef-Req | The field is mandatory present if the target device requests GNSS-SSR-GridDefinition; otherwise it is not present. |

#### –	GNSS-PeriodicAssistDataReq

The IE GNSS-PeriodicAssistDataReq is used by the target device to request periodic assistance data delivery from a location server.

-- ASN1START

GNSS-PeriodicAssistDataReq-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservationsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOSR

glo-RTK-PeriodicBiasInformationReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCPB

gnss-RTK-MAC-PeriodicCorrectionDifferencesReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pMAC

gnss-RTK-PeriodicResidualsReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pRes

gnss-RTK-FKP-PeriodicGradientsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pFKP

gnss-SSR-PeriodicOrbitCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOC

gnss-SSR-PeriodicClockCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCC

gnss-SSR-PeriodicCodeBiasReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCB

...,

[[

gnss-SSR-PeriodicPhaseBiasReq-r16		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pPB

gnss-SSR-PeriodicSTEC-CorrectionReq-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pSTEC

gnss-SSR-PeriodicGriddedCorrectionReq-r16

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pGrid

gnss-SSR-PeriodicURAReq-r16				GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Cond pURA

gnss-SSR-PeriodicGridDefinitionReq-r16	GNSS-PeriodicControlParam-r15	OPTIONAL 	-- Cond pGridDef

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| pOSR | The field is mandatory present if the target device requests periodic GNSS-RTK-Observations; otherwise it is not present. |
| pCPB | The field is mandatory present if the target device requests periodic GLO-RTK-BiasInformation; otherwise it is not present. |
| pMAC | The field is mandatory present if the target device requests periodic GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| pRes | The field is mandatory present if the target device requests periodic GNSS-RTK-Residuals; otherwise it is not present. |
| pFKP | The field is mandatory present if the target device requests periodic GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| pOC | The field is mandatory present if the target device requests periodic GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| pCC | The field is mandatory present if the target device requests periodic GNSS-SSR-ClockCorrections; otherwise it is not present. |
| pCB | The field is mandatory present if the target device requests periodic GNSS-SSR-CodeBias; otherwise it is not present. |
| pPB | The field is mandatory present if the target device requests periodic GNSS-SSR-PhaseBias; otherwise it is not present. |
| pSTEC | The field is mandatory present if the target device requests periodic GNSS-SSR-STEC-Correction; otherwise it is not present. |
| pGrid | The field is mandatory present if the target device requests periodic GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| pURA | The field is mandatory present if the target device requests periodic GNSS-SSR-URA; otherwise it is not present. |
| pGridDef | The field is mandatory present if the target device requests periodic GNSS-SSR-GridDefinition; otherwise it is not present. |

--- End of Change ---

--- Start of Change ---

#### –	GNSS-SSR-GridDefinitionReq

The IE GNSS-SSR-GridDefinitionReq is used by the target device to request the GNSS-SSR-GridDefinition assistance from the location server.

-- ASN1START

GNSS-SSR-GridDefinitionReq-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

--- End of Change ---

--- Start of Change ---

#### –	GNSS-GenericAssistanceDataSupport

The IE GNSS-GenericAssistanceDataSupport is used by the target device to provide information on supported GNSS generic assistance data types to the location server for each supported GNSS.

-- ASN1START

# GNSS-GenericAssistanceDataSupport ::=

SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataSupportElement

# GNSS-GenericAssistDataSupportElement ::= SEQUENCE {

gnss-ID								GNSS-ID,

sbas-ID								SBAS-ID						OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsSupport				GNSS-TimeModelListSupport

OPTIONAL, -- Cond TimeModSup

gnss-DifferentialCorrectionsSupport	GNSS-DifferentialCorrectionsSupport

OPTIONAL, -- Cond DGNSS-Sup

gnss-NavigationModelSupport			GNSS-NavigationModelSupport

OPTIONAL, -- Cond NavModSup

gnss-RealTimeIntegritySupport		GNSS-RealTimeIntegritySupport

OPTIONAL, -- Cond RTISup

gnss-DataBitAssistanceSupport		GNSS-DataBitAssistanceSupport

OPTIONAL, -- Cond DataBitsSup

gnss-AcquisitionAssistanceSupport	GNSS-AcquisitionAssistanceSupport

OPTIONAL, -- Cond AcquAssistSup

gnss-AlmanacSupport					GNSS-AlmanacSupport

OPTIONAL, -- Cond AlmanacSup

gnss-UTC-ModelSupport				GNSS-UTC-ModelSupport

OPTIONAL, -- Cond UTCModSup

gnss-AuxiliaryInformationSupport	GNSS-AuxiliaryInformationSupport

OPTIONAL, -- Cond AuxInfoSup

...,

[[

bds-DifferentialCorrectionsSupport-r12

BDS-DifferentialCorrectionsSupport-r12

OPTIONAL, -- Cond DBDS-Sup

bds-GridModelSupport-r12		BDS-GridModelSupport-r12

OPTIONAL	-- Cond BDS-GridModSup

]],

[[

gnss-RTK-ObservationsSupport-r15

GNSS-RTK-ObservationsSupport-r15

OPTIONAL,	-- Cond RTK-OSR-Sup

glo-RTK-BiasInformationSupport-r15

GLO-RTK-BiasInformationSupport-r15

OPTIONAL,	-- Cond GLO-CPB-Sup

gnss-RTK-MAC-CorrectionDifferencesSupport-r15

GNSS-RTK-MAC-CorrectionDifferencesSupport-r15

OPTIONAL,	-- Cond MAC-Sup

gnss-RTK-ResidualsSupport-r15	GNSS-RTK-ResidualsSupport-r15

OPTIONAL,	-- Cond Res-Sup

gnss-RTK-FKP-GradientsSupport-r15

GNSS-RTK-FKP-GradientsSupport-r15

OPTIONAL,	-- Cond FKP-Sup

gnss-SSR-OrbitCorrectionsSupport-r15

GNSS-SSR-OrbitCorrectionsSupport-r15

OPTIONAL, 	-- Cond OC-Sup

gnss-SSR-ClockCorrectionsSupport-r15

GNSS-SSR-ClockCorrectionsSupport-r15

OPTIONAL, 	-- Cond CC-Sup

gnss-SSR-CodeBiasSupport-r15	GNSS-SSR-CodeBiasSupport-r15

OPTIONAL 	-- Cond CB-Sup

]] ,

[[

gnss-SSR-PhaseBiasSupport-r16	GNSS-SSR-PhaseBiasSupport-r16

OPTIONAL,	-- Cond PB-Sup

gnss-SSR-STEC-CorrectionSupport-r16

GNSS-SSR-STEC-CorrectionSupport-r16

OPTIONAL,	-- Cond STEC-Sup

gnss-SSR-GriddedCorrectionSupport-r16

GNSS-SSR-GriddedCorrectionSupport-r16

OPTIONAL,	-- Cond Grid-Sup

gnss-SSR-URASupport-r16			GNSS-SSR-URASupport-r16

OPTIONAL,	-- Cond URA-Sup

gnss-SSR-GridDefinitionSupport-r16

GNSS-SSR-GridDefinitionSupport-r16

OPTIONAL	-- Cond GridDef-Sup

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModSup | The field is mandatory present if the target device supports GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Sup | The field is mandatory present if the target device supports GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModSup | The field is mandatory present if the target device supports GNSS-NavigationModel; otherwise it is not present. |
| RTISup | The field is mandatory present if the target device supports GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsSup | The field is mandatory present if the target device supports GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistSup | The field is mandatory present if the target device supports GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacSup | The field is mandatory present if the target device supports GNSS-Almanac; otherwise it is not present. |
| UTCModSup | The field is mandatory present if the target device supports GNSS-UTC-Model; otherwise it is not present. |
| AuxInfoSup | The field is mandatory present if the target device supports GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Sup | The field is mandatory present if the target device supports BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModSup | The field is mandatory present if the target device supports BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Sup | The field is mandatory present if the target device supports GNSS-RTK-Observations; otherwise it is not present. Note, support for GNSS-RTK-Observations implies support for GNSS-RTK-CommonObservationInfo as well. |
| GLO-CPB-Sup | The field is mandatory present if the target device supports GLO-RTK-BiasInformation; otherwise it is not present. This field may only be present if gnss-ID indicates 'glonass'. |
| MAC-Sup | The field is mandatory present if the target device supports GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Sup | The field is mandatory present if the target device supports GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Sup | The field is mandatory present if the target device supports GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Sup | The field is mandatory present if the target device supports GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Sup | The field is mandatory present if the target device supports GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Sup | The field is mandatory present if the target device supports GNSS-SSR-CodeBias; otherwise it is not present. |
| PB-Sup | The field is mandatory present if the target device supports GNSS-SSR-PhaseBias; otherwise it is not present. |
| STEC-Sup | The field is mandatory present if the target device supports GNSS-SSR-STEC-Correction; otherwise it is not present. |
| Grid-Sup | The field is mandatory present if the target device supports GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| URA-Sup | The field is mandatory present if the target device supports GNSS-SSR-URA; otherwise it is not present. |
| GridDef-Sup | The field is mandatory present if the target device supports GNSS-SSR-GridDefinition; otherwise it is not present. |

--- End of Change ---

--- Start of Change ---

#### –	GNSS-SSR-GridDefinition-Support

-- ASN1START

GNSS-SSR-GridDefinition-Support-r16 ::= 	SEQUENCE {

...

}

-- ASN1STOP

--- End of Change ---

--- Start of Change ---

## 7.2	Mapping of posSibType to assistance data element

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in sub-clause 6.5.2.2. The OTDOA Assistance Data IEs are defined in sub-clause 7.4.2.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-PhaseBias |
|  | posSibType2-21 | GNSS-SSR-STEC-Correction |
|  | posSibType2-22 | GNSS-SSR-GriddedCorrection |
|  | posSibType2-23 | GNSS-SSR-URA |
|  | posSibType2-24 | GNSS-SSR-GridDefinition |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |

--- End of Change ---

# Conclusions

Proposal 1: 	Add the new Grid Definition message to the Running LPP CR for PPP-RTK support (SSR).

# References

# RP-190752, "New WID: NR Positioning Support".

# R2-1903137, "Running LPP CR for PPP-RTK support (SSR)", Qualcomm Incorporated

# IS-QZSS-L6, "Quasi-Zenith Satellite System – Interface Specification – Centimetre Level Augmentation Service", November 5, 2018.

- R2-19007147, “SSR Messages for: Carrier Phase Bias, URA, Atmosphere, Grid Definition”

- R2-19007149, “Text Proposal for addition of SSR URA message”