3GPP TSG-RAN WG2 #106	Tdoc R2-1907161

Reno, Nevada, US, 13-17 May 2019

Agenda Item:	11.6.4.2

Source:	Ericsson, Vodafone

Title:	Paging capacity for NTN

Document for:	Discussion

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The new study item that had update SID [3] has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture. The objectives for layer 2 and above are:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2] - Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1] - Dual connectivity [RAN3 aspects] involving - NTN-based NG-RAN (Transparent GEO or LEO satellites) and terrestrial based NG-RAN access: Xn terminated on the ground - or two NTN-based NG-RAN access (between Regenerative LEO satellites): Xn over ISL - Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3] - Paging: procedure adaptations in case of moving satellite foot prints or cells Note: - This new study item does not address regulatory issues.

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In RAN2#104, the following is agreed for tracking area and paging topic:

Agreements:

- RAN2 to study solutions for two different type of UE categories 1) with GNSS support, 2) without GNSS support

- For GEO, the current tracking area management is assumed as a baseline

- For LEO with moving beams study fixed and moving tracking area solutions

In RAN2#105bis, the following is agreed for paging topic:

Agreements:

=>	Add the parameters to be considered for paging (arrival rate, etc) in the email discussion from rapporteur

=>	Companies are encouraged to bring papers for next meeting on paging capacity and work together before the meeting to converge.

In this paper, we discuss paging capacity for NTN

# Discussion

Paging capacity calculations are slightly more relevant for GEO than for LEO as the assumed NR cell size is larger for GEO. The maximum values for cell diameter assumed in TR 38.821 are 1000km for GEO and 500km for LEO but with 500km and 200km respectively, used as the reference beam footprint diameter. A related discussion is the capacity of an NR cell, with respect to paging, but also with respect to the number of RACH preambles. However, under this AI, we should check the available paging capacity and consider if 500km or 200km NR cell radius is feasible from that perspective.

The following is currently proposed in email discussion to be added to TR:

---

7.3.3.1 Paging Capacity

Following parameters should be considered for calculation of paging capacity

- Paging Frames (PF) per second

- Paging Occasions (PO) per PF

- Maximum number of paging records in paging message

- User density (UEs/km2)

- Satellite beam diameter: in km

- NO_Traffic: fraction of UEs in the cell with network originated traffic

- Arrival session or call rate:  Average#PagePerHourPerUE: average requested paging occasions per hour and per UE

---

We discuss here further the non-multibeam scenario and provide an example paging capacity calculation. A Radio Frame (RF) that includes one or more Paging Occasions (PO) is called a Paging Frame (PF). Every Radio Frame can potentially be configured to be a PF. In a non-multibeam scenario 4 out of 10 subframes per PF can be used for paging, that is there can be at most 4 PO per PF. A paging message can only be sent in a PO. The paging message can at most include 32 paging records in the paging message where each paging record includes the UE identities of the UEs being paged. The theoretical paging capacity as maximum number of UEs paged per second in an NR non-multibeam cell is thus limited by

Paging Capacity=NPF×NPOperPF×NUEperPO.

As each RF can be configured to be a PF, the resulting maximum paging capacity with 100 PF per second is thus 4 × 100 = 400 PO per second. This implies that theoretically, an NR cell can page 32 × 400 = 12 800 UEs per second, or equivalently, more than 46 Million UEs per hour (12 800 × 60 × 60).

Theoretically, a non-multibeam NR cell can page 32*400 = 12800 UEs per second, or equivalently, more than 46 Million UEs per hour (12800*60*60).

The theoretical capacity should be compared to the expected number of users in an NTN cell. The cell area of a hexagonal cell using a radius of 250km, see Figure 1, we get a cell area larger than 163 000km2. If we assume ellipsoid NTN cells with r1 = 250 km and r2 = 200 km, the cell area becomes 157 000 km2. One number proposed during the discussions is 400 users per km2, which would then result in almost 63 Million users per cell.

Figure 1 Area of a hexagonal cell with radius r

Assuming that these UEs need to be paged at least once per hour, approximately 73% of them can be paged per hour because of the previously calculated maximum of 46 Million paging messages per hour. Or equivalently, the paging channel is overloaded by ~137%. Assuming instead that only 20% of all UEs in a cell need to be paged, and only once per 24 hours, then 0.2 × 63 × 106 ∕ 24 ≅ 525 000 users need to be paged per hour, using less than 2% of the capacity of the paging channel.

The number of UEs that is feasible to be paged per hour will most likely be limited by other means than the available paging occasions. Therefore, the paging capacity should also be considered together with the cell’s capacity to support UEs accessing the cell. After being paged, the UE accesses the cell using a random-access procedure which starts by the UE transmitting a random-access preamble on the physical random-access channel (PRACH). An idle mode UE reads from system information, that is broadcasted by the cell, which preambles can be used and which uplink time slot to be used. Further details in

In appendix we provide a TP capturing paging capacity calculations for both GEO and LEO with non-multibeam assumption. Further we provide an example calculation with numbers. It is proposed to adopt the TP to be included in TR.

RAN2 to adopt the TP to be included in TR 38.821

The above calculations are done with non-multibeam assumption. With multibeam assumption where on cell can have more than one SSB beams, the paging occasion definition is more complicated due to beam sweeping. It should be discussed if RAN2 should make paging capacity calculations also for the multibeam assumption.

RAN2 to discuss if paging capacity calculations need to be repeated for GEO or LEO for the multibeam assumption where on cell can have more than one SSB beams

# Conclusion

We made the following observations:

Observation 1	Theoretically, a non-multibeam NR cell can page 32*400 = 12800 UEs per second, or equivalently, more than 46 Million UEs per hour (12800*60*60).

We propose the following:

Proposal 1	RAN2 to adopt the TP to be included in TR 38.821

Proposal 2	RAN2 to discuss if paging capacity calculations need to be repeated for GEO or LEO for the multibeam assumption where on cell can have more than one SSB beams

# References

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla

R3-186269, TR 38.821, Ver 020

RP-182880, Study on solutions for NR to support non-terrestrial networks (NTN)

R2-19071xx, Random access capacity, Ericsson, RAN2#106, Reno, Nevada, May 2019

# Appendix

-_____________________start of TP_____________

## 7.3.1 	Idle mode mobility enhancements

7.3.3.1 Paging Capacity

Following parameters should be considered for calculation of paging capacity

- Paging Frames (PF) per second

- Paging Occasions (PO) per PF

- Maximum number of paging records in paging message

- User density (UEs/km2)

- Satellite beam diameter: in km

- NO_Traffic: fraction of UEs in the cell with network originated traffic

- Arrival session or call rate:  Average#PagePerHourPerUE: average requested paging occasions per hour and per UE

7.3.3.1.1 Paging capacity of non-multibeam cell

In a non-multibeam scenario 4 out of 10 subframes per PF can be used for paging, that is there can be at most 4 PO per PF. A paging message can only be sent in a PO. The paging message can at most include 32 paging records in the paging message where each paging record includes the UE identities of the UEs being paged. The theoretical paging capacity as maximum number of UEs paged per second in an NR non-multibeam cell is thus limited by

Paging Capacity=NPF×NPOperPF×NUEperPO.

As each RF can be configured to be a PF, the resulting maximum paging capacity with 100 PF per second is thus 4 × 100 = 400 PO per second. This implies that theoretically, an NR cell can page 32 × 400 = 12 800 UEs per second, or equivalently, more than 46 Million UEs per hour (12 800 × 60 × 60).

The paging capacity should also be considered together with the cell’s capacity to support UEs accessing the cell. After being paged, the UE accesses the cell using a random-access procedure which starts by the UE transmitting a random-access preamble on the physical random-access channel (PRACH). PRACH capacity is calculated in Section X.

7.3.3.1.1 Example calculation for a typical LEO NTN cell

The cell area of a hexagonal cell using a radius of 250km, see Figure 1, we get a cell area larger than 163 000km2. If we assume ellipsoid NTN cells with r1 = 250 km and r2 = 200 km, the cell area becomes 157 000 km2. If further 400 users per km2 is assumed, it would result in almost 63 Million users per cell.

Figure 1 Area of a hexagonal cell with radius r

Assuming that these UEs need to be paged at least once per hour, approximately 73% of them can be paged per hour because of the previously calculated maximum of 46 Million paging messages per hour. Or equivalently, the paging channel is overloaded by ~137%. Assuming instead that only 20% of all UEs in a cell need to be paged, and only once per 24 hours, then 0.2 × 63 × 106 ∕ 24 ≅ 525 000 users need to be paged per hour, using less than 2% of the capacity of the paging channel.

___________________________end of TP________________