3GPP TSG-RAN WG2#106	R2-1907250

Reno, Nevada, USA, 13th – 17th May 2019	Resubmission of R2-1904476

Agenda Item:	11.10.3

Source:	Ericsson

Title:	Broadcast of early measurement configuration in NR

Document for:	Discussion, Decision

# Introduction

The Work Item Description for DC and CA enhancements in [1] was approved at the RAN#81 meeting. One of the objectives concerns how to reduce the delay until the setup of DC and/or CA through early measurements:

Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode

  - The impacts on UE power consumption should be minimized

  - The LTE Rel-15 euCA work should be utilized, when applicable

Early measurement reporting was discussed at the RAN2#105 meeting, with the following agreements:

Agreements:

For IDLE/INACTIVE

- Rel-16 early measurement configuration may contain both NR and LTE configuration, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. FFS on details.  IDLE mode and INACTIVE mode details will be discussed separately

- NR early measurement configuration should include NR specific measurement parameters configurations.

- Available beam and cell level measurement results can be included in early measurement reporting if configured.

At the RAN2#105bis meeting the following agreements were reached for early measurement configuration:

Agreements

1:	NR early measurements can be configured in both NR RRCRelease message and NR system information.

FFS: Whether there are differences in the configuration that can be provided by RRCRelease and SI.

2:	Introduce some indication about the cell's early measurement support in NR system information.

3: 	To control the duration of UE performing both IDLE and INACTIVE measurements, a single validity timer (similar to measIdleDuration in LTE euCA) is mandatory indicated only in NR RRCRelease message, i.e. not included in NR SIB.

4:	For both IDLE and INACTIVE early measurements, the following IEs can be optionally configured per NR frequency in both NR RRCRelease message and NR SIB:

-	A list of frequencies and optionally cells (similar to measCellList in LTE euCA) the UE is required to perform early measurements.

-	A cell quality threshold (similar to qualityThreshold in LTE euCA) the UE is required to report the measurement results only for the cells which met the configured thresholds.

FFS: A validity Area (similar to validityArea in LTE euCA) to indicate the list of cells within which UE is required to perform early measurements. If the UE reselects to a cell outside this list, the early measurements are no longer required (same as timer expiry).

o	If it is absent, the UE will not have area limitation of early measurements.

For SSB based measurements:

5:	For both IDLE and INACTIVE early measurements, SSB frequencies to be measured can be located out of sync raster

6: 	For both IDLE and INACTIVE early measurements, RSRP and RSRQ can be configured as cell and beam measurement quantity.

7: 	For both IDLE and INACTIVE early measurements, the configuration parameters provided per SSB frequency follow the same principles as those provided in SIB2/4 for the purposes of Idle/Inactive mobility. (Details differences can be discussed at stage 3 level)

8: 	As LTE euCA, cell / beam SINR is not introduced as measurement quantity in NR early measurement configuration in Rel-16.

For SSB based beam level measurement configurations:

9	The UE is required to report the beam with the highest measurement quantity

FFS: Whether additional beams can be reported.

10: For both IDLE and INACTIVE early measurements, the UE can be configured with one of the 3 beam reporting types

1)	No beam reporting;

2)	Only beam identifier

3)	Both beam identifier and quantity

FFS: Whether to support CSI-RS based NR early measurements

11: LTE UE in IDLE mode, IDLE with suspended, and INACTIVE can be configured with NR early measurements to support fast setup of (NG)EN-DC (i.e. euCA is extended to support NR measurements). Details are FFS

The early measurement configuration shall support both NR and LTE configuration. Using the LTE Rel-15 euCA work as baseline, the dedicated provision should then be included in the RRCRelease message. Where to include the broadcast configuration in NR is however not clear, especially considering that it may contain information related to both NR and E-UTRA carriers/cells. In this contribution we discuss how to provide broadcast configuration of early measurements in NR.

# Discussion

## Idle mode measurement configuration in LTE Rel-15

In LTE Rel-15 euCA the configuration of E-UTRA carriers/cells where idle mode measurements are to performed is provided either through dedicated signaling (in the RRCConnectionRelease message) or broadcast (in SIB5):

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond UP-EDT

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

SystemInformationBlockType5 ::=		SEQUENCE {

interFreqCarrierFreqList			InterFreqCarrierFreqList,

...,

lateNonCriticalExtension				OCTET STRING	(CONTAINING SystemInformationBlockType5-v8h0-IEs)				OPTIONAL,

[[	interFreqCarrierFreqList-v1250	InterFreqCarrierFreqList-v1250		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-r12	InterFreqCarrierFreqListExt-r12	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1280	InterFreqCarrierFreqListExt-v1280	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1310		InterFreqCarrierFreqList-v1310		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1310	InterFreqCarrierFreqListExt-v1310	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1350		InterFreqCarrierFreqList-v1350	OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1350	InterFreqCarrierFreqListExt-v1350	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1360	InterFreqCarrierFreqListExt-v1360	OPTIONAL	-- Need OR

]],

[[	scptm-FreqOffset-r14				INTEGER (1..8)					OPTIONAL	-- Need OP

]],

[[	interFreqCarrierFreqList-v1530		InterFreqCarrierFreqList-v1530		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1530	InterFreqCarrierFreqListExt-v1530	OPTIONAL,	-- Need OR

measIdleConfigSIB-r15				MeasIdleConfigSIB-r15			OPTIONAL	-- Need OR

]]

}

MeasIdleConfigSIB-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15,

...

}

MeasIdleConfigDedicated-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15				OPTIONAL,	-- Need OR

measIdleDuration-r15		ENUMERATED {sec10, sec30, sec60, sec120,

sec180, sec240, sec300, spare},

...

}

EUTRA-CarrierList-r15 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierEUTRA-r15

MeasIdleCarrierEUTRA-r15::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

allowedMeasBandwidth-r15			AllowedMeasBandwidth,

validityArea-r15					CellList-r15					OPTIONAL,	-- Need OR

measCellList-r15					CellList-r15					OPTIONAL,	-- Need OR

reportQuantities					ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r15				SEQUENCE {

idleRSRP-Threshold-r15				RSRP-Range					OPTIONAL,	-- Need OR

idleRSRQ-Threshold-r15				RSRQ-Range-r13				OPTIONAL	-- Need OR

}																	OPTIONAL, 	-- Need OP

...

}

CellList-r15 ::=		SEQUENCE (SIZE (1.. maxCellMeasIdle-r15)) OF PhysCellIdRange

In case of broadcast of the early measurement configuration, it is thus included in SIB5, which contains system information for inter-frequency cell reselection. The broadcast early measurement configuration (in SIB5) is used by the UE when it is configured with an idle mode measurement configuration in the RRCConnectionRelease message, which does not contain the measIdleCarrierListEUTRA-r15.

In LTE Rel-15 the early measurement configuration (i.e. the idle mode measurements) was only defined for LTE CA, and thus only included measurement configuration for additional E-UTRA carriers (and possibly cells). The broadcast information is thus placed in SIB5, which contains the inter-frequency cell reselection information.

- In LTE Rel-15 the early measurement configuration only includes E-UTRA carriers/cells. The broadcast information is thus placed in SIB5, which contains inter-frequency cell reselection information.

## Broadcast early measurement configuration in NR

The early measurement configuration in NR should support both measurements on NR/cells/beams and E-UTRA carriers/cells, i.e. both an “NR early measurement configuration” and an “E-UTRA early measurement configuration”. For provision of the early measurement configuration through broadcast, i.e. system information, the question is then in what SIB or SIBs to include the configuration.

The following SIBs exist in NR:

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access

-	SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection

-	SIB3 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection

-	SIB4 contains information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection

-	SIB5 contains information about E-UTRA frequencies and E-UTRA neighbouring cells relevant for cell re-selection

-	SIB6 contains an ETWS primary notification;

-	SIB7 contains an ETWS secondary notification;

-	SIB8 contains a CMAS warning notification;

-	SIB9 contains information related to GPS time and Coordinated Universal Time (UTC).

SIB1 contains information that is needed for initial access, and it always needs to be broadcast (as long as it is possible to camp in the cell). The size of it may however be an issue in some cases and we should therefore avoid adding information there, which can be included elsewhere.

SIB3 contains information related to NR intra-frequency cell reselection, and thus does not seem suitable for the early measurement configuration since they concern inter-frequency carriers or (E-UTRA) inter-RAT carriers.

SIB6-8 are for Public Warning System, which are only transmitted in rare occasions.

SIB9 contains time information, which is not necessarily broadcast in a network.

The possible options for inclusion of the early measurement configurations therefore seem to be:

- NR early measurement configuration in SIB4 & E-UTRA early measurement configuration in SIB5, i.e. include the early measurement configuration together with the corresponding cell reselection related system information.

- Both NR early measurement configuration & E-UTRA early measurement configuration in SIB2.

- Both NR early measurement configuration & E-UTRA early measurement configuration in a new (separate) SIB.

To split the early measurement configuration into 2 different SIBs, as in option 1, would be well aligned with the existing information in the corresponding SIBs. UEs supporting early measurement configuration in NR will also typically need to acquire both these SIBs for cell reselection information anyway. It may however increase the complexity slightly since the UE will need to acquire both these SIBs in order to get the whole early measurement configuration. In case there is (broadcast) early measurement information that is not related to the carriers/cells to measure on, it then also becomes unclear where to include this.

It therefore seems simpler to keep the early measurement configuration in a single SIB. SIB2 already contains information that is common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection. It is always broadcast in a cell and acquired by a UE. It therefore seems suitable for inclusion of the early measurement configuration. The alternative to introduce a new separate SIB just to provide the early measurement configuration seems unnecessary. Such a SIB would anyway typically be included in the same SI message as other SIBs. We therefore propose to include the early measurement configuration in SIB2 in NR.

The broadcast early measurement configuration in NR should be included in SIB2.

# Conclusion

In the previous sections we have made the following observations:

Observation 1	In LTE Rel-15 the early measurement configuration only includes E-UTRA carriers/cells. The broadcast information is thus placed in SIB5, which contains inter-frequency cell reselection information.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The broadcast early measurement configuration in NR should be included in SIB2.

# References

RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core), RAN#81