3GPP TSG-RAN WG2 Meeting #106	R2-1907277

Reno, USA, 13th  – 17th of May 2019

Agenda item:	12.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhanced signalling for single active protocol stack solution

WID/SID:	LTE_feMob-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

It has been agreed in RAN2 #105 to progress on non-split bearer solution for reducing the service interruption time during the handover in LTE [1]. Moreover, in RAN2#105-bis a decision has been made that any solution that is specified will be modelled as a single PDCP entity at the UE side [2].

In this paper, we discuss an enhanced signalling for the single active protocol stack solutions which are described as Option 0/1/2 in [3].

# 2	Discussion

Two main categories for non-split bearer solution with single active protocol stack have been discussed in [3].

- In the first category (Option 0/1), the UE stops the transmission/reception in the source cell after reception of a valid RAR in the target cell.

- In the second category (Option 2), the UE stops the transmission/reception in the source cell after transmission of handover complete (i.e. RRC Connection Reconfiguration Complete) in the target cell.

In both categories, it would be beneficial to have the source cell aware about the time instant that the UE detaches such that data forwarding or SN Status Transfer is performed in more predictable and controlled manner as in conventional handover and unlike MBB solution of Rel-14.

Observation 1: It would be beneficial to have the source cell aware about the time instant that the UE detaches such that data forwarding or SN Status Transfer is triggered in more predictable and controlled manner as in conventional handover and unlike MBB solution of Rel. 14.

One approach is that the target cell informs the source cell when the UE performs access to the target cell. That is for solutions in category 1, the target cell informs the source cell once the RAR is sent to the UE whereas for solution in category 2 the indication is sent once RRC Connection Reconfiguration Complete is received by the target cell. Herein, the indication may be received late after the UE has detached from the source cell due to delay over Xn interface. This will (in practice) contribute in an implementation-dependant (i.e. somewhat unpredictable) way to the mobility interruption time.

Observation 2: The indication sent by the target cell to the source cell upon UE’s successful random access may be received late after UE has already detached from the source cell. This will (in practice) contribute in an implementation-dependant (i.e. somewhat unpredictable) way to the mobility interruption time.

To overcome this issue, the following solution is proposed:

- After sending/or receiving the RACH response (RAR), the target cell/or UE informs the source cell about the completion of the random access.

- Once the random access indication is received from the target cell/or UE, the source cell sends to the UE a message to resume the handover, i.e., by sending RRC Connection Reconfiguration Complete to the target cell.

- After sending the handover resume message, the source cell stops the transmission and reception with the UE.

Similarly, the UE detaches from the source cell once it receives the handover resume message.

For clarity, the signalling of the solution is depicted in Fig. 1.

Figure 1: Signalling procedure for non-split bearer with single active protocol stack.

As shown in Fig. 1, the source cell and UE will stop the radio link after the transmission/reception of the handover resume message in Step 10. Herein, both the source cell and UE are simultaneously aware about the time instant that the radio link with the source cell is stopped. As a result, data forwarding and SN Status Transfer can be initiated by the source cell in the same way as conventional handover without uncertainties of MBB-like Rel-14 solutions.

The handover resume message does not have to carry a lot of information, since all necessary details are already available at the UE, thanks to the handover command of step 7 in Fig. 1. It can even be considered to implement it as a MAC control element to accelerate the detection at the UE side.

Proposal 1: RAN2 is asked to consider the handover resume message for non-split bearer with single active protocol for having more predictable and controlled manner for data forwarding or SN Status Transfer message. Details of such message can be further discussed.

# 3	Conclusion

In this paper, we have analysed the detach of the UE from the source cell and the time instant for triggering data forwarding in single active protocol stack solutions. The observations and proposals are summarized in the following:

Observation 1: It would be beneficial to have the source cell aware about the time instant that the UE detaches such that data forwarding or SN Status Transfer is triggered in more predictable and controlled manner as in conventional handover and unlike MBB solution of Rel. 14.

Observation 2: The indication sent by the target cell to the source cell upon UE’s successful random access may be received late after UE has already detached from the source cell. This will (in practice) contribute in an implementation-dependant (i.e. somewhat unpredictable) way to the mobility interruption time.

Proposal 1: RAN2 is asked to consider the handover resume message for non-split bearer with single active protocol for having more predictable and controlled manner for data forwarding or SN Status Transfer message. Details of such message can be further discussed.

# References

- R2-1903001 RAN2#105 Meeting Report.

- R2-1905188, Report from session on Rel-16 LTE mobility enhancements WI, 3GPP TSG-RAN WG2 Meeting #105bis, Xian China, 8th – 12th April 2019.

- R2-1904255 Report of [105#57][LTE/feMOB] UE and network side impacts of single/dual protocol stacks, ZTE, 3GPP TSG RAN WG2 Meeting #105-bis, Xian, China, 8th-12th April 2019.